Regulation (EU) No 528/2012 concerning the making available on the market and use of biocidal products

RISK ASSESSMENT OF A BIOCIDAL PRODUCT FOR NATIONAL AUTHORISATION APPLICATIONS

Jerbor

Product type 8

Boric acid and Disodium tetraborate pentahydrate

Case Number in R4BP: BC-FW057615-11

Evaluating Competent Authority: Sweden

Date: [07/03/2023]

Table of Contents

1.	CONCLUSION	4

2.	ASSESSMENT REPORT	6

2.1.	Summary of the product assessment	6

2.1.1.	Administrative information	6

2.1.1.1 Identifier of the product	6

2.1.1.2 Authorisation holder	6

2.1.1.3 Manufacturer of the product	6

2.1.1.4 Manufacturer(s) of the active substance(s)	6

2.1.2 Product composition and formulation	7

2.1.2.1 Identity of the active substance	7

2.1.2.2. Candidate(s) for substitution	7

2.1.2.3 Qualitative and quantitative information on the composition of the biocidal product	8

2.1.2.4 Information on technical equivalence	8

2.1.2.5 Information on the substance(s) of concern	8

2.1.2.6 Type of formulation	8

2.1.3 Hazard and precautionary statements	9

2.1.4 Authorised use and directions for use	10

2.1.4.1 Use description	10

2.1.4.2 Use-specific instructions for use	11

2.1.4.3 Use-specific risk mitigation measures	11

2.1.4.4 Where specific to the use, the particulars of likely direct or indirect effects, first aid instructions and emergency measures to protect the environment	11

2.1.4.5 Where specific to the use, the instructions for safe disposal of the product and its packaging	11

2.1.4.6 Where specific to the use, the conditions of storage and shelf-life of the product under normal conditions of storage	11

2.1.5 General directions for use	11

2.1.5.1 Instructions for use	11

2.1.5.2 Risk mitigation measures	14

2.1.5.3 Particulars of likely direct or indirect effects, first aid instructions and emergency measures to protect the environment	15

2.1.5.4 Instructions for safe disposal of the product and its packaging	16

2.1.5.5 Conditions of storage and shelf-life of the product under normal conditions of storage	16

2.1.6 Other information	16

2.1.7 Packaging of the biocidal product	16

2.1.8 Documentation	17

2.1.8.1 Data submitted in relation to product application	17

2.1.8.2 Access to documentation	17

2.2.	Assessment of the biocidal product	18

2.2.1 Intended use(s) as applied for by the applicant	18

2.2.2. Physical, chemical and technical properties	19

2.2.3 Physical hazards and respective characteristics	27

2.2.4 Methods for detection and identification	29

2.2.5 Efficacy against target organisms	35

2.2.5.1 The proposed label claims for the product	35

2.2.5.2 Function and field of use	35

2.2.5.3 Organisms to be controlled and products, organisms or objects to be protected	38

2.2.5.4 Effects on target organisms, including unacceptable suffering	40

2.2.5.5 Mode of action, including time delay	40

2.2.5.6 Efficacy data	42

2.2.5.7 Occurrence of resistance and resistance management	44

2.2.5.8 Known limitations	45

2.2.5.9 Relevant information if the product is intended to be authorised for use with other biocidal product(s)	45

2.2.6 Risk assessment for human health	45

2.2.6.1 Assessment of effects on Human Health	45

2.2.6.2 Exposure assessment	53

2.2.6.3 Risk characterisation for human health	67

2.2.7 Risk assessment for animal health	75

2.2.8 Risk assessment for the environment	76

2.2.8.1 Effects assessment on the environment	78

2.2.8.2 Exposure assessment	87

2.2.8.3 Risk characterisation	102

2.2.9 Measures to protect man, animals and the environment	107

2.2.10 Assessment of a combination of biocidal products	107

2.2.11 Comparative assessment	108

No comparative assessment has been made as it has not yet been assessed for whether the active substance meets the criteria for exception under Article 5(2) and is a candidate for substitution in accordance with Article 10 of BPR.	108

3.	AnnexeS	109

3.1 List of studies for the biocidal product	109

3.2  Output tables from exposure assessment tools	114

3.2.1 Input files for the human health risk assessment	114

3.2.2 Input data for environmental risk assessment model scenario 2B	115

3.2.3 Output of model scenario 2B	117

3.3	Confidential annex	118

3.4 Other	119

3.4.1.1 Overall use pattern	120

3.4.1.2 Method of application including description of system used	121

3.4.1.3 Application rate and the final concentration of the biocidal product and active substance in the system in which the preparation is to be used	125

3.4.1.4 Number and timing of applications, and information relating to geographical variations	127

3.4.1.5 Analytical methods for monitoring	128

- CONCLUSION

Jerbor is a biocidal product that contains the active substances boric acid (53.7%) and disodium tetraborate pentahydrate (46.3%). The intended use of the product is in-situ treatment of foundation wood against decay by wood rooting fungi (PT8).

The application slurry (Eurobor) is prepared from Jerbor (9.6% w/w) dissolved in hot water (70.4% w/w) and sodium bentonite (20% w/w). Use is restricted to professionals, trained according to Manual Euroborbehandling (manual of Eurobor technique) (Bandh, 2022).

APCP

The shelf life for Jerbor is 10 weeks (production only on demand). No significant changes in physico-chemical and technical properties were observed after storage at ambient temperature for 10 weeks. HDPE-packaging compatibility has been demonstrated in the stability study. Longer ambient temperature storage stability tests (2 years) are ongoing and will be completed in January 2024.

‘Do not store above 25 °C.’ and ‘Store in original container in a dry, cool, and well-ventilated place.’ should be included on the label since acceptable accelerated storage stability study is missing and because it is known that storage at elevated temperatures and under humid condition may result in caking and formation of lumps.

Jerbor is not classified for any physical hazards.

Human health

Jerbor is classified Eye Irrit. 2, H319 and Repr. 1B, H360fd. Risk assessment has been performed for the different steps (production, mixing, transfer, application, cleaning and maintenance/repair) involved in the Jerbor treatment. The tool ECETOC TRA has been used for the exposure calculations and risk assessment.

A qualitative local risk assessment has also been performed due to the eye irritating properties of Jerbor and the sodium bentonite added to the slurry that could be irritating to eyes, skin and the respiratory tract.

It can be concluded that Jerbor can be used in the different scenarios without any unacceptable local or systemic risks for the professionals, provided that the prescribed PPE is worn and that the work is performed in accordance with the Manual Euroborbehandling (manual of Eurobor technique) (Bandh, 2022).

Efficacy

The laboratory efficacy studies for curative treatment against fungal degradation of Scots pine wood miniblock according to a modified EN 113 test showed that brown and white rot fungal decay stopped at a retention of 0.7 kg BAE/m3 wood. Data on boric acid show that the toxic threshold limit in non-termite areas can be set to 1 kg BAE/m3 wood. Wood and sampling in the follow-up controls verify that targeted retention levels are achieved after five years.

Available scientifical peer-reviewed data on the efficacy of the two active substances in Jerbor against the target organisms show that they are efficacious against brown and white rotting fungi and that the toxic threshold for the most tolerant fungus was 2.2 kg BAE/m³. Less data is available on the efficacy of boron species against soft rot and non-toxic threshold was shown for this type of organisms, although field data on retention levels of BAE and microscopic inspections years after the treatment indicate that the active substances in Jerbor can be expected to be efficacious also against soft rot. Field data on retention levels of boron found in timber foundation piles 5-18 years after the remedial treatments as well as the results on diffusion of boron into an infected 100-year-old pile strongly indicate that the active substances in Jerbor actually reach the site of decay and at high enough concentrations. In summary, according to the data submitted it can be expected that Jerbor will be efficacious against the selected target organisms.

Environment

The risk assessment for the active substances has been performed using modelling techniques for two foundation types – foundation wall of concrete and foundation of stone. The modelling results were confirmed by environmental sampling/monitoring studies with the conclusion that the risks are acceptable for all the environmental compartments if the directions for use according to chapter 2.1.5 are followed and the following risk mitigation measures are considered:

- Treatment zone must be surrounded by dense clay of marine origin with low water turnover of at least 2 m deep

- Installation of groundwater tubes for environmental control must always be done before the treatment.

- All low-lying openings of drain shafts must be sealed before the treatment

Based on the above it was therefore concluded that the use of Jerbor for in-situ treatment of foundation wood for use by professionals within PT8 is considered to be efficacious and safe with regards to human and animal health and the environment.

- ASSESSMENT REPORT

Summary of the product assessment

    - Administrative information

2.1.1.1 Identifier of the product

| Identifier | Country (if relevant) |
| --- | --- |
| Jerbor | Sweden |

2.1.1.2 Authorisation holder

| Name and address of the authorisation holder | Name | Jerbol System AB |
| --- | --- | --- |
|  | Address | c/o WSP Sverige AB 121 88 Stockholm-Globen, Sweden |
| Authorisation number | 5689 |  |
| Date of the authorisation | 2023-03-07 |  |
| Expiry date of the authorisation | 2028-03-05 |  |

2.1.1.3 Manufacturer of the product

| Name of manufacturer | Jerbol System AB |
| --- | --- |
| Address of manufacturer | c/o WSP Sverige AB Box 13033 402 51 Göteborg, Sweden |
| Location of manufacturing sites | Spadegatan 6 424 65 Angered, Sweden |

2.1.1.4 Manufacturer(s) of the active substance(s)

| Active substance | Boric acid and disodium tetraborate pentahydrate |
| --- | --- |
| Name of manufacturer | Rio Tinto Iron & Titanium GmbH |
| Address of manufacturer | Mergenthalerallee 77 65760 Eschborn, Germany |
| Location of manufacturing sites | 14486 Borax Road Boron, CA 93516 USA |

| Active substance | Boric acid and disodium tetraborate pentahydrate |
| --- | --- |
| Name of manufacturer | Etimine S.A. |
| Address of manufacturer | Immeuble 67 204, Z.I. Scheleck 2 L-3225, Bettembourg, Luxembourg |
| Location of manufacturing sites | Eti maden isletmeleri g.m. Emet kolemanit Islt. Müdürlügü 43700 Emet/Kütahya Turkey |

2.1.2 Product composition and formulation

Does the product have the same identity and composition as the product evaluated in connection with the approval for listing of the active substance(s) on the Union list of approved active substances under Regulation No. 528/2012?

Yes

No 	 The active substances are evaluated under Directive 98/8/EC and the product have the same identity and composition as the product evaluated in connection with the approval for listing of the active substances

2.1.2.1 Identity of the active substance

| Main constituent(s) |  |
| --- | --- |
| ISO name | Boric acid |
| IUPAC or EC name | Boric acid |
| EC number | 233-139-2 |
| CAS number | 10043-35-3 |
| Index number in Annex VI of CLP | 005-007-00-2 |
| Minimum purity / content | 99.0% w/w (990 g/kg) |
| Structural formula |  |

| Main constituent(s) |  |
| --- | --- |
| ISO name | Disodium tetraborate pentahydrate |
| IUPAC or EC name | Disodium tetraborate pentahydrate |
| EC number | 215-540-4 |
| CAS number | 12179-04-3 |
| Index number in Annex VI of CLP | 005-011-02-9 |
| Minimum purity / content | 99.0% w/w (990 g/kg) |
| Structural formula |  |

2.1.2.2. Candidate(s) for substitution

The active substances are classified in accordance with Regulation (EC) No 1272/2008 as toxic for reproduction category 1B (H360FD) and therefore meets the exclusion criteria listed in Article 5(1) of Regulation 528/2012. As the draft CAR was submitted prior to September 2013 it has not yet been assessed for whether it meets the criteria for exception under Article 5(2) and is a candidate for substitution in accordance with Article 10 of BPR.

2.1.2.3 Qualitative and quantitative information on the composition of the biocidal product

| Common name | IUPAC name | Function | CAS number | EC number | Content (% w/w) |
| --- | --- | --- | --- | --- | --- |
| Product, Jerbor composition |  |  |  |  |  |  |
| Boric acid | Boric acid | Active substance | 10043-35-3 | 233-139-2 | 53.7 |
| Disodium tetraborate pentahydrate | Disodium tetraborate pentahydrate | Active substance | 12179-04-3 | 215-540-4 | 46.3 |

2.1.2.4 Information on technical equivalence

The manufacturing sources for the active substances boric acid and disodium tetraborate pentahydrate were assessed at active substance approval and are listed in the Union list of approved substances under Regulation (EU) No. 528/2012.

2.1.2.5 Information on the substance(s) of concern

No substances of concern have been identified for Jerbor.

2.1.2.6 Type of formulation

Water soluble granules (SG)

2.1.3 Hazard and precautionary statements

Classification and labelling of the product of the according to the Regulation (EC) 1272/2008

| Classification |  |
| --- | --- |
| Hazard category | Eye Irrit.2 Repr.1B |
| Hazard statement | H319 Causes serious eye irritation. H360FD May damage fertility. May damage the unborn child. |
| Labelling |  |
| Signal words | Danger |
| Hazard statements | H319 Causes serious eye irritation. H360FD May damage fertility. May damage the unborn child. |
| Precautionary statements | P201 Obtain special instructions before use. P202 Do not handle until all safety precautions have been read and understood. P280 Wear protective gloves/protective clothing /eye protection/face protection*. P264 Wash hands or other body parts that have come into contact with the product thoroughly after handling. P305+P351+P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do so. Continue rinsing. P337+P313 If eye irritation persists: Get medical advice/attention. P308+P313 IF exposed or concerned: Get medical advice/attention. P405 Store locked up. P273 Avoid release to the environment. P391 Collect spillage. P501 Dispose of container/contents in accordance with national legislation. |
| Note | *Manufacturer/supplier to specify the appropriate type of equipment. |

2.1.4 Authorised use and directions for use

The use and application of Jerbor is strictly limited to upper part of wooden foundations, piles and grillages, in ground with dense clay soils (of marine origin), with low water turnover according to Manual Euroborbehandling (manual of Eurobor technique) (Bandh, 2022).

2.1.4.1 Use description

Table 1. Use # 1 – Preservation of wooden foundations (curative treatment)

| Product Type | PT 8 (wood preservatives) |
| --- | --- |
| Where relevant, an exact description of the authorised use |  |
| Target organism (including development stage) | Wood-rotting fungi in solid softwood (C.10, B10) including: G.10, Basidiomycetes, brown rot fungi G.11, Basidiomycetes, white rot fungi G.12, Soft rot fungi |
| Field of use | E.40 Wood preservative, use class 4a, remedial application of timbers in service in ground contact (curative). |
| Application method | F.20, Injection (Eurobor slurry is injected to a depot outside the wood). F.60, Diffusion (diffusion of active substances from injected depot into the wet wood) |
| Application rate(s) and frequency | Concentration of AS in solution/slurry before application is 9.0% BAE w/w (about 10.6% BAE w/v). Estimated/applied dose of Jerbor equal to 5-12 kg BAE/m3 Target dose of Jerbor equal to 2 kg BAE/m3 Target retention of active substances in wood after 5 years, above 1 kg BAE/m3 One application for each foundation |
| Category(ies) of users | A.30 Professional, Use is restricted to professional users, approved by Jerbol Systems AB, according to Manual Euroborbehandling (manual of Eurobor technique). |
| Pack sizes and packaging material | HDPE barrel: 30 L |

2.1.4.2 Use-specific instructions for use

See General directions for use.

2.1.4.3 Use-specific risk mitigation measures

See General directions for use.

2.1.4.4 Where specific to the use, the particulars of likely direct or indirect effects, first aid instructions and emergency measures to protect the environment

See General directions for use.

2.1.4.5 Where specific to the use, the instructions for safe disposal of the product and its packaging

See General directions for use.

2.1.4.6 Where specific to the use, the conditions of storage and shelf-life of the product under normal conditions of storage

See General directions for use.

2.1.5 General directions for use

2.1.5.1 Instructions for use

The instructions for use of Jerbor are available in Manual Euroborbehandling (2022). The use is restricted only to professional staff trained according to the manual. Overall use activities The following activities are mandatory: 1. Foundation investigation 2. Preparations before treatment 3. Treatment with measurements 4. Follow-up of result Required results from the overall use activities 1. Before a recommendation of a treatment with Jerbor, a foundation investigation have to result in following. laboratory results, showing fungi degradation in grillage or piles and that wood is still worth preserving documented foundation design from inspection of pits wood and groundwater levels control of sewage pipes beneath the building documented ground conditions, showing that the foundation is surrounded by dense clay with low water turnover in the decay/treatment zone measurements showing uniform settling pace within the building knowledge of the hydrogeological conditions in the surroundings 2. Before a treatment with Jerbor, preparations have to result in following: necessary control documents to ensure, that intended effect of the treatment is achieved that the work can be performed safely, both for professionals and for third parties that work can be done without environmental impact outside the treatment zone all professionals and third parties are properly informed injection holes are marked out with each holes identity on site according to drilling drawing any leaking sewage pipes beneath the building are repaired and any wastewater contaminated groundwater is pumped out installed ground water tubes outside the foundation for environmental control sampled and analysed ground water, showing background levels of boron substances before treatment if necessary, sealing measures are performed 3. Treatment with measurements, have to result in following: appropriate health and safety on site required work documents available on site responsible management present at site protocol of drilled injection holes indoor set up of mixing station with preparations to handle any spill protocol of injected amount Eurobor slurry with measurements of water level, electric conductivity and pH value before and during injection. The protocol should include injection scheme, noted spread and comments. protocol and lab results of environmental controls sealed and restored injection holes and cleaned premises final report of the work including protocols, lab results, updated drilling drawing and instructions to the client 4. Follow-up of results must generate: settling pace within the building, monitored with subsidence levelling active substance in wood (after 5 years), a routine sampling of wood and analysing of boric acid equivalent (BAE) is always made after five years before the commercial warranty expires. In some cases, wood sampling is also done as a control in the beginning of the warranty time. In most cases the property owner/customer (on recommendation) wants continued monitoring of settling pace, groundwater levels and sampling etc. after end of warranty. It is often 7 to 10 years from a first foundation investigation of a building to the end of the warranty time. Usually, a Eurobor treatment is made once. In some cases, additional treatments might be needed. The protocol in these cases is, that extended wood sampling and analyses of borates are performed, and then a limited area is chosen for additional treatment. Instructions to prepare Eurobor slurry The Eurobor slurry is prepared from Jerbor (9.6 % w/w), hot water (70.4 % w/w) and sodium bentonite (20 % w/w). 151 L of hot water (50-70°C) is filled in mixing tank and circulation pump is started. 20.6 kg of Jerbor is portioned in mixing tank under carefully stirring. 18 L of the Jerbor water solution is filled (through a connected hose with a ball valve) in to the mixing vessel and 4.8 kg of sodium bentonite is portioned during mixing with the plaster and mortar mixer. Mixing is done until all bentonite is in the mixing vessel and then mixing will stop for a minute and the mixing starts again until the slurry is smooth and the viscosity is correct. The 20 L of slurry is poured by tilting the mixing vessel into the plaster and mortar pump and is ready for injection. Instructions for safe handling Only professionals trained according to Manual Euroborbehandling (manual of Eurobor technique) are allowed to handle the product. The product must be stored in a locked space at working site Only personnel have access to the working site and the product Wear protective gloves/protective clothing/eye protection/face protection If protective clothing is contaminated, work must be stopped and a change to clean clothing must take place. Use respiratory protection when handling dry product Keep away from food, drink, drinking water and animal feed Limit handling that generates dust and provide for good ventilation Pregnant or lactating women should not work with the product Running tap water should always be available at working site Eyewash should always be available at mixing place Immediately remove spill of the product according to guidelines Immediately remove bentonite spill from floor, slippery when wet Keep order and tidy in the workplace If there is a risk of spillage, floor wells should be covered and sewer pipes below the floor should be intact.

2.1.5.2 Risk mitigation measures

The following risk mitigating measures should be applied during the different steps when handling the product: Production: The packages with the active substances should be opened under a ventilated hood (LEV). Use of respiratory protective equipment (RPE) with particle filter P3 is mandatory (filter type (code letter, colour) to be specified by the authorisation holder within the product information). Wear a protective coverall [type 6, EN 13034]. Wear protective chemical resistant gloves during production (glove material to be specified by the authorisation holder within the product information). The use of eye protection during production of Jerbor is mandatory. Mixing/loading: Use of respiratory protective equipment (RPE) with particle filter P3 is mandatory (filter type (code letter, colour) to be specified by the authorisation holder within the product information). Wear a protective coverall [type 6, EN 13034]. Wear protective chemical resistant gloves during mixing/loading of the product (glove material to be specified by the authorisation holder within the product information). The use of eye protection during mixing/loading is mandatory. Application: Application must be done via a closed system. Wear a protective coverall [type 6, EN 13034]. Wear protective chemical resistant gloves during application of the product (glove material to be specified by the authorisation holder within the product information). Cleaning: Wear a protective coverall [type 6, EN 13034]. Wear protective chemical resistant gloves during cleaning of the equipment (glove material to be specified by the authorisation holder within the product information). The use of eye protection during cleaning of the equipment is mandatory. Maintenance/repair: Wear a protective coverall [type 6, EN 13034]. Wear protective chemical resistant gloves during cleaning of the equipment (glove material to be specified by the authorisation holder within the product information). The use of eye protection during cleaning of the equipment is mandatory. Environment: Treatment zone must be surrounded by dense clay of marine origin with low water turnover of at least 2 m deep Installation of groundwater tubes for environmental control must always be done before the treatment. All low-lying openings of drain shafts must be sealed before the treatment

2.1.5.3 Particulars of likely direct or indirect effects, first aid instructions and emergency measures to protect the environment

Particulars of likely direct or indirect effects Inhalation; may cause respiratory irritation Skin contact; may cause skin irritation Eye contact; irritating to eyes Ingestion; may cause nausea, vomiting, diarrhoea First aid instructions IF EXPOSED OR CONCERNED: Get medical advice/attention. IF INHALED: If symptoms occur call a POISON CENTRE or a doctor. IF ON SKIN: Wash skin with water. If symptoms occur call a POISON CENTRE or a doctor. IF IN EYES: Rinse with water. Remove contact lenses, if present and easy to do. Continue rinsing for 5 minutes. Call a POISON CENTRE or a doctor. IF SWALLOWED: Rinse mouth. Give something to drink, if exposed person is able to swallow. Do NOT induce vomiting. Call a POISON CENTRE or a doctor. Emergency measures to protect the environment In case of dry product spills, sweep/shovel up/clean with an industrial vacuum cleaner large spills and place in a labelled sealable container. Rinse area thoroughly with water. In case of slurry spills, pick up with absorbent material (e.g. sand, diatomaceous earth or a proprietary absorbent material). Keep in suitable closed containers for disposal. Dispose of absorbed material and the cleaning water in accordance with regulations as hazardous waste. In case of major accidental emissions, immediately inform the local authorities and limit the damage as much as possible. Decontamination equipment and activated carbon filter must always be available at site. Recycle spilled chemicals if possible in the next batch. Otherwise, dispose of as hazardous waste.

2.1.5.4 Instructions for safe disposal of the product and its packaging

Dosage and use of the product are performed according to Manual Euroborbehandling (manual of Eurobor technique) to minimize residues. Residues and spills are collected to be reused if possible, otherwise disposed of as hazardous waste. Empty packaging is considered as hazardous waste and should be disposed of in accordance with local regulations. Cleaned packaging can be recycled or reused. Reuse of empty packaging is only allowed for refilling of the Jerbor product. Rinse the packaging three times with water and let it dry. If possible, use the rinsing water in the production of the next batch. Otherwise, the cleaning water shall be disposed of as hazardous waste. Product residues and contaminated material are hazardous waste and should be disposed of in accordance with local/regional/national/international regulations. Waste and residues EWC Code: 06 13 01* - Inorganic plant protection products, wood- preserving agents and other biocides Uncleaned packaging EWC Code: 15 01 10* - Packaging containing residues of or contaminated by dangerous substance

2.1.5.5 Conditions of storage and shelf-life of the product under normal conditions of storage

Shelf life: 10 weeks. Do not store above 25°C. Store in original container in a dry, cool, and well-ventilated place. Keep container tightly closed.

2.1.6 Other information

Procedures for cleaning application equipment Clean the application equipment in water. Collect the water in a barrel. If possible, use this water in the production of the next batch. Wear protective clothing, safety goggles and protective gloves in nitrile rubber during the cleaning.

2.1.7 Packaging of the biocidal product

| Type of packaging | Size/volume of the packaging | Material of the packaging | Type and material of closure(s) | Intended user (e.g. professional, non-professional) | Compatibility of the product with the proposed packaging materials (Yes/No) |
| --- | --- | --- | --- | --- | --- |
| UN-approved barrels | 30 L | High density polyethylene (HDPE) | The polyethylene lid is secured with a metal clamping ring | Professional | Yes |

2.1.8 Documentation

2.1.8.1 Data submitted in relation to product application

See Annex 3.1 of this PAR.

2.1.8.2 Access to documentation

European Borates Association A.I.S.B.L. is the data owner of the dossiers for boric acid and disodium tetraborate pentahydrate and has provided Jerbol System AB with a letter of access to both active substance dossiers.

Assessment of the biocidal product

2.2.1 Intended use(s) as applied for by the applicant

Table 2. Intended use # 1 – Preservation of wooden foundations according to the Eurobor technique

| Product Type | PT 8, (wood preservatives) |
| --- | --- |
| Where relevant, an exact description of the authorised use | D.50, Curative treatment / wood in service Professional remedial treatment of investigated wooden foundations, situated in dense clay. In situ treatment of wooden piles and grillages after proven fungi degradation according to the manual of Eurobor technique. |
| Target organism (including development stage) | Wood-rotting fungi in solid softwood (C.10, B10) including; G.10, Basidiomycetes, brown rot fungi G.11, Basidiomycetes, white rot fungi G.12, Soft rot fungi |
| Field of use | E.40 Wood preservative, use class 4a according to CEN 335-1 standard, remedial application of timbers in service in ground contact (curative). (Assessment Report Boric acid, 2009, and Assessment Report Disodium tetraborate, 2009) |
| Application method | F.20, Injection (Eurobor slurry is injected to a depot outside the wood). Jerbor is a granular concentrate and must be diluted in hot (50-70°C) water and mixed with sodium bentonite before application. Maximum permitted concentration of Jerbor is 9,6% w/w and the minimum allowable proportion of sodium bentonite in the Eurobor slurry is 20% w/w. F.60, Diffusion (diffusion of active substances from injected depot into the wet wood) |
| Application rate(s) and frequency | Concentration of AS in solution/slurry before application is 9,0% BAE w/w (about 10,6% BAE w/v). Estimated/applied dose of Jerbor equal to 5-12 kg BAE/m3 Target dose of Jerbor equal to, 2 kg BAE/m3 Target retention of active substances in wood after 5 years, above 1 kg BAE/m3 Application is made once (1) for a foundation |
| Category(ies) of users | A.30 Professional, Use is restricted to trained professional users, approved by Jerbol Systems AB, according to the manual of Eurobor technique. |
| Pack sizes and packaging material | UN-approved polyethylene (PE) barrel with two carry handles, 30l, with lid locked with a clamping ring. 20,6 kg net weight |

2.2.2. Physical, chemical and technical properties

Table 2.3.2-1 Physical, chemical, and technical properties

| Numbering according to Annex III of BPR | Property | Guideline and Method | Tested product/batch (AS% w/w) | Results | Reference |
| --- | --- | --- | --- | --- | --- |
| 3.1 3.1.1 3.1.2 | Appearance, physical state and colour at 20 °C and 101.3 kPa | n/a | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | White solid | Theorin, M. (2022) Report no. 140185 |
| 3.1.3. | Odour at 20 °C and 101.3 kPa | n/a | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | Odourless | Theorin, M. (2022) Report no. 140185 |
| 3.2. | Acidity, alkalinity and pH value | SS-EN ISO 10523:2012 SS-EN ISO 9963-2 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | pH 12 % w/w: 7.3 ± 0.1 (at 25°C) 1 % w/w: 8.5 ± 0.1 (at 25°C) Alkalinity 12 % w/w: 23.8 ± 2.2 g HCO3/l (at 25°C) 1 % w/w: 1.94 ± 0.09 g HCO3/l (at 25°C) | Theorin, M. (2022) Report no. 140185 |
| 3.3. | Relative density / bulk density | CIPAC MT 186 OECD Test Guideline 109 (rev. 2012) Pycnometer test SS-ISO 758 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | Pour density: 0.949 kg/dm-3 ± 0.005 kg/dm-3 Tap density: 1.01 kg/dm-3 ± 0.003 kg/dm-3 Aqueous soln. 12 % w/w 1.05 g/mL ± 0,0002 g/mL (at 21.5°C) | Theorin, M. (2022) Report no. 140185 |
| 3.4.1.1. | Storage stability test – accelerated storage | n/a | - | Test waived. The phrase “Do not store above 25 ºC.” should be indicated on the label. | - |
| 3.4.1.2. | Storage stability test – long-term storage at ambient temperature | GIFAP (Croplife International) monograph no. 17, 3:rd ed. (2021) SS EN ISO 17294-1 SS EN ISO 17294-1 US EPA 200.8 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) Batch AJP 1-5 | Storage: 10 weeks at 25 ºC (22.6% Rh) Packaging: 1 L HDPE container Appearance Initial: white solid, no lumps or caking After storage: white solid, no lumps or caking Active substance content Boron (total nominal content: 16.1 % w/w based on a min. purity of 99% for boric acid and disodium tetraborate pentahydrate) Initial: 14.8 % w/w After storage: 15.5 % w/w (+4.7 %) Sodium (total nominal content: 7.2 % w/w based on a min. purity of 99% for disodium tetraborate pentahydrate) Initial: 6.6 % w/w After storage: 7.5 % w/w (+13.9 %) pH (SS-EN ISO 10523) (1 % aq. soln., at 25°C) Initial: 8.5 After storage: 8.5 Alkalinity (SS-EN ISO 9963-2) (1 % aq. soln., at 25 °C) Initial: 1.9 g HCO3-/L After storage: 1.9 g HCO3-/L Degree of dissolution and dilution stability (CIPAC MT 179.1) Conc.: 30 g in 250 mL water Initial: 0.031 % (5 min), 0.027 % (24 h) After storage: 0.031 % (5 min), 0.025 % (24 h) Dustiness (CIPAC MT 171.1, gravimetric) Initial: 9.9 mg (nearly dust-free) After storage: 6.0 mg (nearly dust-free) Attrition resistance (CIPAC MT 178) Initial: 94.4% After storage: 94.8% Stability of packaging Initial: Intact container and without leakages. After storage: Container remains intact. No seepage, ballooning or panelling observed. Weight change <0.005%. | Theorin, M. (2022) Report no. 140185 |
| 3.4.1.3. | Storage stability test – low temperature stability test for liquids | n/a | - | Not required for solid formulations. | - |
| 3.4.2.1. | Effects on content of the active substance and technical characteristics of the biocidal product – light | n/a | - | Boric acid and disodium tetraborate do not have absorption maximum or minimum in neutral, basic or acidic medium in the range 190-750 nm. Jerbor is therefore considered to be stable to light. | CAR Boric acid (2009) CAR Disodium tetraborate (2009) |
| 3.4.2.2. | Effects on content of the active substance and technical characteristics of the biocidal product – temperature and humidity | n/a | - | Boric acid and disodium tetraborate show tendency for caking with temperature cycling and exposure to humid environment. The phrase “Store in original container in a dry, cool, and well-ventilated place.” should be indicated on the label. | - |
| 3.4.2.3. | Effects on content of the active substance and technical characteristics of the biocidal product - reactivity towards container material |  | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) Batch AJP 1-5 | After storage for 10 weeks at ambient temperature, the HDPE-container remained intact and no seepage, ballooning or panelling was observed. The packaging material is compatible with Jerbor. | Theorin, M. (2022) Report no. 140185 |
| 3.5.1. | Wettability | - | - | Not required for SG formulations. | - |
| 3.5.2. | Suspensibility, spontaneity, and dispersion stability | - | - | Not required for SG formulations. | - |
| 3.5.3. | Wet sieve analysis | CIPAC MT 185 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) Batch AJP 1-5 | Test concentration: 10 g in 100 mL water Residue: 0.016 % | Theorin, M. (2022) Report no. 140185 |
| 3.5.4. | Emulsifiability, re-emulsifiability and emulsion stability | - | - | Not required for SG formulations. | - |
| 3.5.5. | Disintegration time | - | - | Not required for SG formulations. | - |
| 3.5.6. | Particle size distribution, content of dust/fines, attrition, friability | CIPAC MT 186 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) Batch AJP 1-5 | Attrition resistance: 94.4 % Particle size distribution (SS-EN 933-1:2012) Sieve (mm); Retained on sieve (%). <0.053; 0.9. 0.053; 1.6. 0.075; 5.0. 0.125; 20.8. 0.5; 46.7. 1; 24.0. Sum; 99.9. | Theorin, M. (2022) Report no. 140185 |
| 3.5.7. | Persistent foaming | CIPAC MT 47.2 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | Concentration: 2.06 g in 15 mL water No foaming observed. | Theorin, M. (2022) Report no. 140185 |
| 3.5.8. | Flowability/pourability/dustability | - | - | Only required for granular formulations applied through application equipment that would subject the granules to pressure and/or heat. | - |
| 3.5.9. | Burning rate — smoke generators | - | - | Not required for SG formulations. | - |
| 3.5.10. | Burning completeness — smoke generators | - | - | Not required for SG formulations. | - |
| 3.5.11. | Composition of smoke — smoke generators | - | - | Not required for SG formulations. | - |
| 3.5.12. | Spraying pattern — aerosols / spray | - | - | Not required for SG formulations. | - |
| 3.6.1. | Physical compatibility | n/a | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) | Jerbor is applied as a slurry (Eurobor) that is prepared on-site by dissolution of Jerbor (9.6 % w/w) in hot water (50-70°C) and mixing with sodium bentonite (20 % w/w). Sodium bentonite is used as a thickening agent in the slurry, to increase efficacy and to reduce risk for leakage. Eurobor is applied by pumping the slurry in a closed system into the wooden foundation of buildings. No incompatibilities have been reported or documented during 30 years of use when the product is used according to instructions on the label and in Manual Euroborbehandling. | Theorin, M. (2022) Report no. 140185 |
| 3.6.2. | Chemical compatibility | - | - | See 3.6.1 | - |
| 3.7. | Degree of dissolution and dilution stability | CIPAC method MT 179.1 | Jerbor (boric acid 53.7 %, disodium tetraborate pentahydrate 46.3 %) Batch AJP1-5 | Concentration: 30 g in 250 mL Residue on 75 µm sieve After 5 min: 0.031% ± 0.005% After 24 h: 0.027% ± 0.006% | Theorin, M. (2022) Report no. 140185 |
| 3.8. | Surface tension | - | - | Not required for SG formulations. | - |
| 3.9. | Viscosity | - | - | Not required for SG formulations. | - |
| 3.10 | Other | In house method | Jerbor (boric acid 47 %, disodium tetraborate decahydrate 53 %) | Swelling tests with Eurobor. Test was also done at different pH and salinity (NaCl) and different amount of bentonite. Eurobor: Jerbor deca (11% w/w) dissolved in hot water and sodium bentonite (10 or 20 % w/w) Swelling is observed when the concentration of boron substances differs between Eurobor and the water surrounding the Eurobor slurry. The swelling increases with the release of boron subtances from the Eurobor slurry. Swelling also increases with increased amount of bentonite. Swelling is affected by pH and salinity (NaCl) in the water surrounding the Eurobor slurry. Higher concentration of NaCl decreases swelling. The largest swelling is observed at neutral pH. Viscosity: 35000 mPas (at 23 °C) Method: Brookfield digital rotational viscometer DV-I with RV-spindle no. 4 at 5 rpm Test item: Jerbor deca (11 % w/w) diluted in water (50-70 °C) and sodium bentonite (20 % w/w) | Bandh, K. and Theorin, M. (2011a) Theorin, M. (2011d) Report no. 2011:6 |

Conclusion on physical, chemical, and technical properties

Conclusion on physical, chemical, and technical properties

Jerbor is water soluble granules (SG) formulation type. The applicant has stated that the formulation type should be changed from granules (GR) to water soluble granules. RefMS has accepted this based on how Jerbor is used to prepare the application slurry (Eurobor), its solubility in hot water and the wet sieve test result. In the wet sieve test only 0.016 % residue is obtained. And based on more than 15 years of field experience, the applicant states that complete dissolution is achieved when Jerbor is mixed with water (50-70 °C). The dissolution of Jerbor in hot water is the first step in the preparation of the Eurobor slurry as described in the instructions for use. The claimed shelf life for Jerbor is 10 weeks (production only on demand). No significant changes in appearance, pH, alkalinity, dilution stability, dustiness, attrition resistance, and active substance content were observed after storage at ambient temperature for 10 weeks. HDPE-packaging compatibility has been demonstrated in the stability study where no degradation, ballooning, panelling or seepage were observed. All studies have been performed in accordance with the current requirements and the results are deemed to be acceptable. Longer ambient temperature storage stability test (T=2 years) is ongoing and will be completed in January 2024. Implications for labelling: “Do not store above 25°C.” and “Store in original container in a dry, cool, and well-ventilated place.” should be included on the label.

2.2.3 Physical hazards and respective characteristics

Table 2.2.2-1 Physical hazards and respective characteristics

| Numbering according to Annex III of BPR | Property | Guideline and Method | Tested product / batch (AS% (w/w) | Results |
| --- | --- | --- | --- | --- |
| 4.1. | Explosives | n/a | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Jerbor does not have explosive properties since it is only composed of boric acid and disodium tetraborate pentahydrate which do not contain groups with explosive properties. |
| 4.2. | Flammable gases | - | - | Jerbor is not a gas. |
| 4.3. | Flammable aerosols | - | - | Jerbor is not an aerosol. |
| 4.4. | Oxidising gases | - | - | Jerbor is not a gas. |
| 4.5. | Gases under pressure | - | - | Jerbor is not a gas. |
| 4.6. | Flammable liquids | - | - | Jerbor is not a liquid. |
| 4.7. | Flammable solids | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Jerbor does not have flammable properties. In the CAR for boric acid, it was concluded that the active substance is a non-flammable solid based on the result of a flammability test. In the CAR for sodium tetraborate it was concluded that the active substance is a non-flammable solid since it is known for its flame retardant properties. (Reference: CAR (Boric acid, 2004) and CAR (Sodium tetraborate, 2005)) |
| 4.8. | Self-reactive substances and mixtures | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Jerbor does not have self-reactive properties since it is only composed of boric acid and disodium tetraborate pentahydrate which do not contain groups associated with self-reactive properties. |
| 4.9. | Pyrophoric liquids | - | - | Jerbor is not a liquid. |
| 4.10. | Pyrophoric solids | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Pyrophoric solids are substances/mixtures that react with oxygen in air and ignite spontaneously without the aid of a supplied spark, flame or heat. Experience in handling and use of Jerbor shows that it does not ignite spontaneously upon contact with air at normal temperatures, therefore, Jerbor should not be classified as a pyrophoric solid. |
| 4.11. | Self-heating substances and mixtures | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Self-heating is a process where the gradual reaction of a substance/mixture with oxygen (in the air) generates heat which may lead to self-ignition and combustion. Boron acid and disodium tetraborate pentahydrate are substances in which the element B is already in the highest possible oxidation state (+3), and thus are not prone to react further with oxygen. Jerbor does not have self-heating properties. |
| 4.12. | Substances and mixtures which in contact with water emit flammable gases | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Experience in handling and use of Jerbor shows that it does not emit flammable gases upon contact with water. |
| 4.13. | Oxidising liquids | - | - | Jerbor is not a liquid. |
| 4.14. | Oxidising solids | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Jerbor does not have oxidising properties. The testing procedures for oxidising solids are based on the capability of an oxidising solid to enhance the combustion of a combustible material. In the CAR for boric acid and sodium tetraborate it is stated that the active substances are known for their flame retardant properties and are not expected to have oxidising properties. (Reference: CAR (Boric acid, 2004) and CAR (Sodium tetraborate, 2005)) |
| 4.15. | Organic peroxides | - | - | Jerbor does not contain any organic peroxides. |
| 4.16. | Corrosive to metals | - | - | Study not feasible to conduct since Jerbor is not a liquid. |
| 4.17.1. | Auto-ignition temperatures of products (liquids and gases) | - | - | Jerbor is not a gas or liquid. |
| 4.17.2. | Relative self-ignition temperature for solids | - | Jerbor (boric acid 53.7%, disodium tetraborate pentahydrate 46.3%) | Jerbor does not have pyrophoric and self-heating properties and the determination of the relative self-ignition temperature is therefore not relevant. |
| 4.17.3. | Dust explosion hazard | - | - | Jerbor does not have dust explosive properties since it is only composed of boric acid and disodium tetraborate pentahydrate which are not prone to oxidation. The oxidation state of boron in both compounds are +3 and boron will therefore not be further oxidised. |

Conclusion on physical hazards and respective characteristics

Conclusion on physical hazards and respective characteristics

Jerbor is not classified for any physical hazards.

2.2.4 Methods for detection and identification

There are no direct methods to determine the active substances as such in the formulation, instead, a multi-element determination of total boron (B) and sodium (Na) using ICP-SFMS technique is used. The reason for this is explained in the CAR for both active substances. In aqueous solution, disodium tetraborate is converted into boric acid/borate while boric acid exists as undissociated boric acid and polynuclear boron complexes. The formation of the various boron species in an aqueous solution is dependent on the concentration of boron and the pH of the solution rather than the boron substances that are added to the solution.

Table 2.2.4-1 Analytical methods for the analysis of the product as such including the active substance, impurities, and residues

| Analytical methods for the analysis of the product as such including the active substance, impurities, and residues |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Principle of the method [ICP-SFMS]: Analysis performed according to ALS Scandinavia Method B-01 which is a modified version of the method EN ISO 17294-1. For the determination of total B and Na, the sample is prepared by dissolution and dilution with water followed by addition of nitric acid and internal standards (total dilution factor of 10000). |  |  |  |  |  |  |  |  |  |  |
| Analyte | Linearity | Specificity | Fortification range, level and number of measurements for each level |  | Recovery rate (%) |  | Precision (%) |  | Limit of Quantification LOQ – only for impurit(y/ies) | Reference |
|  |  |  | Level | Number of measurements | Mean | RSD | Conc. tested | No. of replicates |  |  |
| Boric acid and disodium tetraborate pentahydrate (as total boron) | r2 (10B) = 0.9958 slope 1.0973 Intercept -3339 r2 (11B) = 0.9993 slope 1.0018 Intercept 1140 | Conc. difference between 10B and 11B n = 6 mean 0.2 % Matrix-effect minimized by: - matrix matching of calibration and test sample solutions. - use of certified reference material as internal standard. | 100 % Jerbor and a sample fortified to 120 % | n = 3 at each level | Level 100%: 101.5 (10B) 101.6 (11B) Level 120%: 98.6 (10B) 97.8 (11B) | 2.6 | 700 µg B/l Horrat value (Hr) 1.47 Justification for Hr > 1: - difficulties in sampling because Jerbor consists of a mixture of 2 granular components. - difficulties with analysing highly concentrated samples (even after 10000-fold dilution). | n = 6 | N/A | Genberg, U. and Engström, E. (2022) Report no. VB01 |
| Boric acid and disodium tetraborate pentahydrate (as total sodium) | r2 = 0.9986 slope 0.9982 Intercept 756 | High resolution mass spectra for calibration blank (water/nitric acid/IS) and test sample (Jerbor) showed no spectral interferences. | 100 % Jerbor and a sample fortified to 120 % | n = 3 at each level | Level 100%: 101.5 Level 120%: 93.2 | 3.5 | 300 µg Na/l Horrat value 1.76 Justification for Hr > 1: - difficulties in sampling because Jerbor consists of a mixture of 2 granular components. - difficulties with analysing highly concentrated samples (even after 10000-fold dilution). | n = 6 | N/A | Genberg, U. and Engström, E. (2022) Report no. VB01 |

Table 2.2.4-2 Analytical methods for soil

| Analytical methods for soil |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte (active substance) Analytical method | Linearity | Specificity | Fortification range, level and number of measurements at each level | Recovery rate (%) |  |  | Precision (%) |  | Limit of quantification (LOQ) or other limits | Reference |
|  |  |  |  | Range | Mean | RSD | Concentration tested | Number of replicates |  |  |
| Boron ICP-AES | 400 – 106 ppm | No interferants at 249.678 nm | 100 mg/kg 400 mg/kg | 93 - 100 | 96 | 2.5 | 20.4 mg/kg | - | - | US EPA Method 200.7 |

Table 2.2.4-3 Analytical methods for air

| Analytical methods for air |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte (active substance) Analytical method | Linearity | Specificity | Fortification range, level, and number of measurements at each level | Recovery rate (%) |  |  | Precision (%) |  | Limit of quantification (LOQ) or other limits | Reference |
|  |  |  |  | Range | Mean | RSD | Concentration tested | Number of replicates |  |  |
| Method for detection in air is not required due to lack of volatility and the intended use. |  |  |  |  |  |  |  |  |  |  |

Table 2.2.4.4 Analytical methods for water

| Analytical methods for water |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte (active substance) Analytical method | Linearity | Specificity | Fortification range, level, and number of measurements at each level | Recovery rate (%) |  |  | Precision (%) |  | Limit of quantification (LOQ) or other limits | Reference |
|  |  |  |  | Range | Mean | RSD | Concentration tested | Number of replicates |  |  |
| Boron ICP-AES | 400 – 106 ppm | Minor Fe and Cr interferants at 182.52 and 249.773 nm. No interferants at 249.678nm | 0.1 mg/L 0.4 mg/L | 97-111 | - | - | 0.023 mg/L 0.034 mg/L 0.063 mg/L | - | 0.005 mg/l | US EPA Method 200.7 |

Table 2.2.4-5 Analytical methods for animal and human body fluids and tissues

| Analytical methods for animal and human body fluids and tissues |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte (active substance) Analytical method | Linearity | Specificity | Fortification range, level, and number of measurements at each level | Recovery rate (%) |  |  | Precision (%) |  | Limit of quantification (LOQ) or other limits | Reference |
|  |  |  |  | Range | Mean | RSD | Concentration tested | Number of replicates |  |  |
| Analytical methods for animal and human body fluids and tissues are not required. Studies show that injected Eurobor (slurry of Jerbor) remains within the treatment area and is not to a large extent transported/dispersed from this area (Vickman and Danielsson (2011) and Bandh (2020)). Furthermore, Jerbor is only for professional users with appropriate PPE which limits the exposure. In the CAR, it is stated that the same method as for soil can be used. |  |  |  |  |  |  |  |  |  |  |

Table 2.2.4-6 Analytical methods for monitoring of active substances and residues in food and feeding stuff

| Analytical methods for monitoring of active substances and residues in food and feeding stuff |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Analyte (active substance) Analytical method | Linearity | Specificity | Fortification range, level, and number of measurements at each level | Recovery rate (%) |  |  | Precision (%) |  | Limit of quantification (LOQ) or other limits | Reference |
|  |  |  |  | Range | Mean | RSD | Concentration tested | Number of replicates |  |  |
| The product is not intended to be used on surface in contact with food/feed of plant and animal origin; therefore, analytical method for the determination of active substance in food/feed of plant and animal origin is not required. |  |  |  |  |  |  |  |  |  |  |

Conclusion on methods for detection and identification

Conclusion on methods for detection and identification

An analytical method (Genberg, M. (2022)) for the determination of total boron and sodium in the biocidal product is available. Specificity, linearity, accuracy and precision were checked and found acceptable. For the precision, the Horrat values (Hr) were >1 for both boron and sodium. The is considered acceptable with the justification that there are difficulties in sampling because Jerbor consists of a mixture of 2 granular components and difficulties with analysing highly concentrated samples (even after 10000-fold dilution). Methods for the determination of total boron in boric acid and disodium tetraborate pentahydrate in soil and water were provided and deemed acceptable at EU level. The applicant has also submitted methods for the analysis of total boron in soil and water. Method for detection in air is not required due to lack of volatility and the intended use. In the CAR, it is stated that the same method as for soil can be used for analysis in animal and human body fluids and tissues. The product is not intended to be used on surface in contact with food/feed of plant and animal origin; therefore, analytical method for the determination of active substance in food/feed of plant and animal origin is not required. No other data is required.

2.2.5 Efficacy against target organisms

2.2.5.1 The proposed label claims for the product

The proposed label claims for Jerbor are:

Fungicide for curative control of confirmed fungal decay in wooden piles and grillages in investigated foundations located in dense clay soils (Product Type 8).

The use of Jerbor can be regarded to belong to use class 4a (situation in which the wood or wood-based product is in contact with the ground), even though the use deviates in several aspects from the “traditional” use in this use class (UC).

The use of Jerbor according to the Manual Euroborbehandling (Bandh 2022) is to control decay by wood-rotting fungi in foundation wood for > 5 years.

2.2.5.2 Function and field of use

Jerbor is used in-situ for wood protection (PT8) and with function as a fungicide to stop foundation wood rot damages by degradation fungi (brown rot G.10, white rot G.11 and soft rot G.12). The wood to be protected is explicitly foundation wood, i.e. piles and grillages (solid wood C.10, softwood B10) below buildings situated in areas with dense soft clay soils (UC 4a). The location of areas with dense soft clay soils are in direct contact or vicinity to rivers running out into marine estuaries. These soils have a thickness of clay in the magnitude of 2-100 meters and they have poor geotechnical quality for bearing loads from buildings.

The buildings subject to the treatment are often of high value in historic, environmental, social and economic aspects. The most common type of building subject to treatment is heavy brick wall or concrete buildings with deep basement foundations lowered in dense clay of marine origin.

Techniques to enhance the clay soils load bearing capacity have been used for centuries and in older buildings the foundation often consists of a drywall on horizontal layer of wood (grillage) which in turn rests on wooden piles. Buildings of a later date (1900-1970) often have a foundation of concrete wall and footings resting on wooden piles with the top of a pile casted in the bottom of the footing. The foundation wood of concern is always installed in a foundation trench into the dense clay soil (often 2 - 4 m below the ground surface) with the purpose of having constant groundwater cover of the wood. Originally the other part of the buildings and the surrounding land use did not disturb the foundation trench and the groundwater level was kept above the wood. However, from the 1950s to the present day, interferences on the building’s foundation trench have been made by installation of new sewage tubes, cables, draining etc. Consequently, the groundwater level has been decreasing below the upper part of the foundation wood. The foundation wood will eventually be exposed to air and oxygen in levels that are sufficient for decay fungi to colonize the wood.

The efficacy of Jerbor can be regarded to depend on the fulfilment of three prerequisites:

That the active substances boric acid and disodium tetraborate decahydrate are effective against the target organisms.

That the active substances actually reach the sites of the decaying fungus in the wood.

That the dose then obtained is adequate in order to reach the toxic threshold levels.

The total success of a treatment with Jerbor is also dependant on several other, more technical requirements being fulfilled such as: that a successful injection technique is being used, that a correct assessment of the strength of the wood and degree of decay can be made, that the geological prerequisites are fulfilled and that these can be properly estimated, etc.

The function of Jerbor against fungal decay:

Application is made, with sodium bentonite mixed with Jerbor water solution into a slurry, with low pressure swelling properties and thus filling out voids in the foundation and the decay zone.

Circulation of groundwater will decrease when the voids are filled and minimizing availability and mobility for oxygen, nutrients, trace elements and other components that the biological degradation of foundation wood is dependent on

After application the active substances (BAE) are located in a depot outside the foundation wood and enclosed within the foundation trench and the treatment zone

The depot of the active substances will be present and available for diffusion into wood for a long time since leaching of the active substances from the depot is strictly limited by the surrounding geological barrier of dense clay soils and the mixture with sodium bentonite that is limiting transport in and out of the treatment zone

Transport of the active substances from the depot and into the foundation wood will be made by diffusion of active substances due to a concentration gradient. The high moisture content of the wood (MC) enables a quick and extensive diffusion of the active substances.

Fungicidal effect (i. e. concentration >2 kg BAE/m3 wood) will occur gradually deeper in the sapwood when concentration increases over time, as the depot releases borates. Over longer time (years) the fungicidal effect (i.e. concentration >1 kg BAE/m3 wood) will stop the fungal degradation, by a concentration at this level over the whole fungal decay zone and beyond.

The level of pH is stable at 7.5 in the slurry with a high alkalinity i. e. buffer capacity which has a secondary effect on decreasing fungal activity, which is pH dependent in the range below pH = 6.5 in the decay zone inside the wood.

The active substances form a complex mixture of boric acid-polyborate-borate depending on the concentration of each boron species, pH and temperature (see Figure 2.2.5.2-1) even though the chemistry is not fully understood (Kochkodan et al 2015). All boron species are converted into boric acid/borate upon dissolution in water. In acid solution at pH<5, boron is mainly present as B(OH)3 and in alkaline solution at pH>12.5, boron is mainly present as B(OH)4-. At in-between values (pH 5-12) polynuclear anions are found as well as B(OH)3 and B(OH)4-.

Figure 2.2.5.2-1 Distribution of boric acid, borate and polyborate species as a function of pH in 0.4 M boric acid solution in water (from Kochkodan et al 2015)

Boric acid forms complex with different polyols, such as monosaccharides, which are known degradation products from wood decay by fungi. Complex forming between boric acid and chemical species of polyol type can be seen in 2.2.5.2-2.

Figure 2.2.5.2-2. Chemical structure of mono- and bisborate complexes with polyol species (from Kochkodan et al 2015)

Boric acid can also form complex with other organic substances such as salicylic acid, salicylic alcohol, malic acid and their derivatives as well as humic substances, see Figure 2.2.5.2-3 (Kockkodan et al 2015). Complex bonding with organic acids is a mechanism which will result in increased pH level.

Figure 2.2.5.2-3. Proposed mechanism of borate complex bonding with salicylic acid forming mono (a) and bischelat complex (b) (from Kochkodan et al 2015)

When complex bonding occurs with enzymes in microorganisms or plants, stabilisation and/or inhibition effects occur causing, for example, fungicidal effect.

Endpoint summary on function and field of use

The active substances in Jerbor have their function by the behaviour of boric acid and the borate ion. The specific application technique and formulation enables boric acid to reach the treatment zone where it diffuses into the wood decay zone. The degradation activity is stopped by complexing between boric acid and polyol chemical species in the foundation wood and in the degrading organisms’ metabolic enzymes. The applied Jerbor slurry acts as a depot within the treatment zone but outside the foundation wood with the effect that voids are filled out, groundwater circulation is more limited, possibility for intrusion of air decreases, pH level is kept above the optimum for fungal decay and the retention level of boric acid is above the toxic threshold for fungal decay of wood of 1 kg BAE (boric acid equivalent)/m3 wood.

2.2.5.3 Organisms to be controlled and products, organisms or objects to be protected

The aim for using Jerbor is to control decay (wood-rotting) fungi (brown rot G.10, white rot G.11 and soft rot G.12) in the upper part of wooden foundations (building supporting constructions). The type of wood in timber foundation is solid, C.10 and soft wood, B.10, of species such as Scots pine (Pinus Sylvestris) and Norway spruce (Picea abies). Fungal decay of wood is often defined as white-, brown- and soft rot damage types depending on how the wood is degraded.

Wood-rotting fungi are, according to the assessment of the active substances (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009), the major concern of wood decay in non-termite areas. Bacterial degradation and soft rot decay are also a concern for waterlogged wood but the degradation rate is slow in comparison to fungal degradation (brown and white rot) due to anoxic metabolism (Klaassen 2008). The use of Jerbor according to the Eurobor technique is to control decay by wood-rotting fungi in foundation wood for > 5 years.

The main purpose with Jerbor is not to control specific species of wood-rotting fungi but to control the damage types that are caused by the fungi. However, the species of wood-rotting fungi identified in foundations include fungi such as the brown rot fungi; Coniophora puteana, Gloeophyllum abietinum (Flemming and Koch, 1992; Jerbo, 2004; Bech-Andersen 2005; Schmidt 2007), Antrodia sinuosa, Fibrioporia vaillantii and white rot fungi; Physisporinus vitreus, Trametes versicolor (Blanchette 1984, Daniel and Bergman 1997), Heterobasidium annosum (Blanchette 1984, Daniel and Bergman 1997). According to the applicant, soft rot has also been found in wooden foundations, although to a much lesser extent, but no species have been determined. Since no specific type of soft rot decay organism is identified there is a natural lack of data regarding standard protocol tests, such as EN 113.

In samples from foundation wood a typical observation pattern can be seen in Figure 2.2.5.3-1 with moisture content of the wood (MC) and microscopic observations of decay pattern and the occurrence of degrading microorganisms. The decay zone and front can be noted to a depth of 40-50 mm from the outer limit of sapwood.

Figure 2.2.5.3-1. Graph with average MC in wood duplicate samples in position from outer sapwood to heartwood. Microscopic observations of decay pattern and degrading organisms such as bacteria and fungi (from Bandh, Fimmerstad and Theorin 2011)

Endpoint summary on organisms to be controlled and products, organisms or objects to be protected

Jerbor is used to control degradation of foundation wood by organisms that cause the damage types of brown rot, white rot and soft rot. Use is only allowed when the foundation has been investigated with fungal degradation verified. By protection of buildings’ foundation wood from degradation by fungal decay advantages such as prolonged technical lifespan of the foundation, decreased need for refurbishment and underpinning actions due to settling damages, lower environmental impact by avoiding unnecessary use of additional construction material required to replace the foundation wood can be achieved.

2.2.5.4 Effects on target organisms, including unacceptable suffering

Effects on target organisms from Jerbor are claimed to be the same as for the active substances i.e. boric acid and the borate ion. See evaluation of active substances for details (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009).

Boric acid and its corresponding borate ion are the active substance that has effect against the decay fungi. Once boric acid/borate ion diffuse into the decay zone of the wood, i.e. where the decay fungi are actively degrading the foundation wood, the mode of action is a result of:

Hydrogen bonding between boric acid/borate ion with hydroxyl groups in the wood and thus blocking the fungal extracellular enzymes.

Diffusion into the fungi hyphae and blocking intracellular enzymes with hydrogen bonding.

Hydrogen bonding with mono- and polysaccharides in the wood cellular structure and inside the fungal hyphae cell structure.

Blocking of the enzymatic activities will hinder the metabolism of the decay fungi and thus inhibit the production of organic acids by the fungal metabolism.

pH levels in the wood pore water and inside the fungal hyphae will rise and the fungal enzymes will lose their effect.

Simultaneous effects of hydrogen bonding between borate ion and mono- and polysaccharides and fungal enzymes and the rise of pH level will stop the fungal decay of the foundation wood.

Target concentration for a treatment with Jerbor according to the Eurobor technique is to maintain a concentration above 1 kg BAE/m3 for at least 5 to 10 years in treatable wood.  Treatable wood is roughly the cross-section of sapwood located in the ½ - 1 m of the upper part of wooden foundation. A successful treatment according to the Eurobor technique gives a protection against decay by wood-rotting fungi well beyond >5 years (Bandh and Theorin, 2011b).

Endpoint summary on effects on target organisms, including unacceptable suffering

The target organisms are microorganisms that causes degradation of foundation wood with damage types such as brown-, white- and/or soft rot. Ongoing decay is stopped when target retention levels of about 1 kg BAE/m3 wood is reached. The metabolism of the decay fungi is stopped by blocking both the nutrients (wood mono- and polysaccharides) and the fungal enzymatic activity (complexing with the active site of the enzymes) and increase the level of pH (complexing with organic acids).

2.2.5.5 Mode of action, including time delay

On target organisms i. e. the type of damages caused by decay fungi as a group, the mode of action of Jerbor is attributed to both the effect of the active substances (boric acid and borate ion) and the effect of the application technique.

Injection is made through holes down into the foundation trench where the foundation wood of concern is located (treatment zone), see Figure 2.2.5.5-1 and Figure 2.2.5.5-2. Entry of the borates into the foundation wood (with water content often well above 70%) is by diffusion from the applied slurry. The borates reach the site of decay fungi in the wood more easily compared to wood without ongoing decay (Schwarze et al. 2006; Schwarze, 2007; Lehringer 2011; Bandh 2010c).

| Figure 2.2.5.5-1. Photo of the treatment zone with wooden piles in concrete and dense clay soil below (from Bandh and Theorin 2011a) | Figure 2.2.5.5-2. The principle of Eurobor injection (from Bandh and Theorin 2011a) |
| --- | --- |

Bentonite swelling will reduce the already low groundwater turnover and any leakage from the treatment zone. Upon swelling borates are released, which leads to reduced cohesive forces between bentonite particles in the swelling front. This will give increased swelling and with time sealing of the void. The purpose is to reach toxic retention levels against fungal decay in the wood and seal the foundation from air and water circulation. The fungi demand for oxygen will thereby be minimized as a consequence of the treatment. The decay will be prevented by both biocide effect and decrease of oxygen levels close to the wood.

Degradation of wood by decay fungi increases permeability of water and diffusive substances into the wood and further inside the porewater of the wood. Diffusion of borates into foundation wood from the Jerbor-bentonite-slurry has been studied in laboratory and the results show that toxic retention is reached within 14 days after the application (Bandh et al 2011). Diffusion of borates into the wood will increase the pH level in the wood pore water. This will reduce the growth and degradation ability of the fungi which favour pH below 6.5. The increase of pH in wood from the buffering capacity of borates will both enhance fixation and limit growth conditions for the fungi.

Endpoint summary on mode of action including time delay

Boric acid and borate ion are the substances responsible for the effect on the target organisms. Borates form complex with polyols including mono- and polysaccharides, organic acids and enzymes in the treatment zone blocking the fungal metabolism and thus inhibit the production of organic acids which will increase the pH resulting in decreased fungal decay. In addition, the application technique of Jerbor will result in filling of voids, limiting groundwater and air circulation and increase the pH level due to buffer capacity of the slurry.

2.2.5.6 Efficacy data

The CARs covering the different boron species report that the toxic threshold level for most brown- and white rot wood-decaying fungi was established to be 2.0 kg BAE/m³ wood. Although, the most persistent fungi assessed in the CARs (Gloeophyllum trabeum) will require 2.2 kg BAE/m³.

Almost all efficacy studies presented in the CARs for the different boron species were performed on brown and white rot fungi. This is consistent with studies found in the open literature where there are numerous studies on the efficacy of brown and white rot fungi (see e.g. review by Freeman et al., 2009) while very few studies are available on the efficacy of boron against soft rot fungi. In the CARs there is one study in which relatively high toxic threshold levels for boron against a soft rot fungus was determined. However, since leaching during the experiments of this study was not prevented the threshold levels determined were not considered reliable by the RefMS. If leaching is not prevented during the efficacy testing the resulting threshold levels become much higher compared to studies where leaching is prevented or minimised (Freitag and Morrell, 2005). However, in a study by Henningsson et al. (1988) the growth-inhibiting concentration of boric acid was found to be the same for soft rot fungus as for brown rot fungus. In the same study, results were also presented on the growth of wood-destroying fungi after remedial treatment of poles with different boron containing techniques and products. The results indicated that several species of soft rot fungus can be controlled but threshold levels were not convincingly expressed even though it was indicated that the levels are in line with the target dose suggested by the applicant (2 kg BAE/m³).

2.2.5.6.1 Dose

According to the applicant, the target dose for treatable wood is 2 kg BAE/m³ but since the treatment is performed in situ it is relevant to discuss also the estimated dose required and the applied dose, respectively.

The foundation inspection is the basis for the estimation of the dose for application. When estimating the dose for application there are several factors to consider: first is to estimate the volume of treatable wood. This volume is roughly the sapwood located in the 0.5-1 m of the upper part of the wooden foundation. The applied dose has to be at least twice the target dose of 2 kg BAE/m³ since the external depot will strive towards the equilibrium with the foundation wood. However, 4 kg BAE/m³ is just enough for an ideal state without any losses and full equilibrium between the depot and the wood. Considering adsorption to concrete and clay surfaces, some dilution in water in the treatment zone and uneven distribution, the dose needed is usually estimated to be 6–9 kg BAE/m³. The volume of voids in the treatment zone also needs to be considered. It is important to get an accurate horizontal spread between the injection holes and to get a sealing effect of the bentonite swelling. This estimation of voids requires experience and is used as a final adjustment factor for the dose needed. If volume of voids is high the adjustment of results toward an increase of the estimated dose needed, and in some cases when the volume of void is low the adjustment can be a minor reduction.

Dose needed = target dose for treatable wood (2 kg BAE/m³) × 2 (for equilibrium between depot and wood) × 1.5 to 2.25 (for adsorption and dilution etc.) → 6–9 kg BAE/m³ treatable wood.

The finally estimated dose = needed dose adjusted for volume of voids = 5 - 12 kg BAE/m³.

The applied dose is finally determined during the injection. Normally the applied dose is consistent with the estimated dose for a complete treatment but will differ somewhat within different parts of the foundation.

2.2.5.6.2 Diffusion of boron species into the wood

It has been suggested that use class 4a would not be suitable for wood treated with substances like boric acid and disodium tetraborates due to their proneness to leaching from the treated wood when in contact with moisture. This limitation of boron preservatives is directly linked to their mobility within wood which is an asset when it comes to diffusion of boron from the Eurobor into the foundation wood. When the wood has developed decay caused by fungi it has also been shown that its permeability increases and the uptake of liquid substances is significantly improved (Lehringer, 2011).

The levels of boron in timber foundation piles have been determined 5-18 years after in situ remedial treatments with Jerbor in areas with wet clay ground conditions (Bandh and Theorin, 2011b). For example, pile samples from foundation in Stockholm (Sweden) showed levels of >5 kg BAE/m³ 6 years after the treatment; pile samples from a foundation in Turku (Finland) showed levels between 1.2 and 2.6 kg BAE/m³ 5 years after the treatment and pile samples from foundation in Gothenburg (Sweden) showed levels from 0.6 to 5.3 kg BAE/m³ (mean 1.74 kg BAE/m³) 12 years after the treatment. The study did not go into details on which wood depths correlated to which levels. In another study, the levels in pile samples from another foundation in Gothenburg were found to be between 0.9 and 4.0 kg BAE/m³ (mean 2.2 kg BAE/m³) 5 years after the treatment (Bandh, 2010a). Although not quite clear, higher levels were indicated to have been found closest to the wood surface.

In order to study the diffusion of boron species from a clay-water mixture into the wood of a 100-year-old foundation pile a laboratory study was performed (Theorin, M., Fimmerstad, K. & Bandh K-M., 2011). The wood samples were determined to be infested by fungus and the outer sapwood showed decay by fungal activity. The samples (wedges cut out of intact sections of the pile, 210-240 mm in diameter and 60-80 mm thickness) were sealed on all sides except the tangential section and placed in incubation vessels at 8 °C and a relative humidity of 85% and Eurobor slurry was placed on each sample. Samples for analysis of boron were taken after 15, 30 and 45 days. The results showed that the highest levels of boron were found in the outer sapwood and decreasing at increasing depth towards the heartwood. After 15 days diffusion, levels of at least 2 kg BAE/m³ were found at depths up to approximately 35 mm (from the wood surface). After 30 and 45 days, levels of at least 2 kg BAE/m³ were found at a depth of approximately 40 mm. It should be mentioned though that the BAE levels in the study were reported on a weight basis and has been converted here to kg/m³-basis by using a wood density of 500 kg/m³ as suggested in the CAR for boric acid. It was further indicated in the study that the decay fronts towards the heartwood were approximately at a depth of 40 mm.

2.2.5.6.3 Laboratory efficacy studies

A modified EN 113 test with miniblocks of Norway spruce has been performed on Jerbor water solution to achieve retention levels of 0.2 – 1.2 kg BAE/m3. The results showed that the brown rot fungi C. puteana was suitable for use in this test with a strong degradation of the control samples. The white rot fungus P. vitreus showed a weak degradation of the control sample therefore the test was performed for a longer time.

| Experimental data on the efficacy of the biocidal product against target organism(s) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Function | Field of use envisaged | Test substance | Test organism(s) | Test method | Test system / concentrations applied / exposure time | Test results: effects | Reference |
| Biostatic and fungicidal function – Determination of the fungicidal effectiveness against degradation by wood-destroying basidiomycetes | Curative treatment against brown rot fungi on wood use class 4 | Jerbor | Coniophora puteana | Modified EN 113, miniblock | Scots pine, sapwood, vacuum impregnation, retention of 0.2 – 1.2 kg BAE/m3 wood, 8 weeks incubation time | <3 % mass loss at retention of 0.7 kg BAE/m3 | RISE 2020 |
| Biostatic and fungicidal function - Determination of the fungicidal effectiveness against degradation by wood-destroying basidiomycetes | Curative treatment against white rot fungi on wood use class 4 | Jerbor | Physisporinus vitreus | Modified EN 113, miniblock | Scots pine, sapwood, vacuum impregnation, retention of 0.2 – 1.2 kg BAE/m3 wood, 8 weeks incubation time | Slow degradation rate, < 0.5 % mass loss at retention of 0.7 kg BAE/m3 | RISE 2020 |

Endpoint summary on efficacy data

The laboratory efficacy studies for curative treatment against fungal degradation of Scots pine wood miniblock according to a modified EN 113 test showed that brown and white rot fungal decay stopped at a retention of 0.7 kg BAE/m3 wood. Data on boric acid show that the toxic threshold limit in non-termite areas can be set to 1 kg BAE/m3 wood. Use condition for Jerbor is restricted to harsh conditions for the decay fungi, therefore the in-situ threshold limit most likely is lower than 1 kg BAE/m3 wood. Since no suitable laboratory standard is available for testing of toxic threshold value against degradation the most reliable method for controlling efficacy is the follow-up sampling of treated wood 5 years after the application. Comprehensive data from Eurobor treatments (with a target dose of 2 kg BAE/m3) show that the retention level after five years is > 1.2 kg BAE/m3 wood. Jerbor is considered efficient and justification for it has been shown.

2.2.5.7 Occurrence of resistance and resistance management

No resistance against the active substances has been reported or claimed in the assessment for borates as wood preservative. Borates have been used as preservative for almost 100 years with no known development of resistance. Jerbor has been used in wooden foundations since the 1980s and there are no signs of any development of resistance.

2.2.5.8 Known limitations

Known limitations are regulated in the mandatory conditions for use of Jerbor. This includes an investigation of each building foundation to determine that safe and efficient use of Jerbor can be guaranteed as described in the Eurobor manual.

2.2.5.9 Relevant information if the product is intended to be authorised for use with other biocidal product(s)

Not relevant, no other biocidal products or active substances are intended to be used with the product.

Conclusion on the efficacy of the product

The laboratory efficacy studies for curative treatment against fungal degradation of Scots pine wood miniblock according to a modified EN 113 test showed that brown and white rot fungal decay stopped at a retention of 0.7 kg BAE/m3 wood. Data on boric acid show that the toxic threshold limit in non-termite areas can be set to 1 kg BAE/m3 wood. Wood and sampling in the follow-up controls verify that targeted retention levels are achieved after five years. Available scientifical peer-reviewed data on the efficacy of the two active substances in Jerbor against the target organisms show that they are efficacious against brown and white rotting fungi and that the toxic threshold for the most tolerant fungus was 2.2 kg BAE/m³. Less data is available on the efficacy of boron species against soft rot and non-toxic threshold was shown for this type of organisms, although field data on retention levels of BAE and microscopic inspections years after the treatment indicate that the active substances in Jerbor can be expected to be efficacious also against soft rot. Field data on retention levels of boron found in timber foundation piles 5-18 years after the remedial treatments as well as the results on diffusion of boron into an infected 100-year-old pile strongly indicate that the active substances in Jerbor actually reach the site of decay and at high enough concentrations. In summary, according to the data submitted it can be expected that Jerbor will be efficacious against the selected target organisms.

2.2.6 Risk assessment for human health

2.2.6.1 Assessment of effects on Human Health

2.2.6.1.1 Introduction

Since Jerbor contains only the active substances of disodium tetraborate pentahydrate (CAS No. 12179-04-3) and boric acid (CAS 10043–35-3) and no other co-formulants, the data from the Assessment Reports for Boric acid and Disodium tetraborate (2009) are relevant for the product. Therefore, no new testing has been performed for the risk assessment of human health.

2.2.6.1.2 Skin corrosion and irritation

| Conclusion used in Risk Assessment – Skin corrosion and irritation |  |
| --- | --- |
| Value/conclusion | Jerbor is not irritant to the skin. |
| Justification for the value/conclusion | Neither Boric acid nor Disodium tetraborate pentahydrate revealed skin corrosion or skin irritation potential (Assessment Reports (CAR) for Boric acid and Disodium tetraborate, 2009) and are not classified as skin irritants. |
| Classification of the product according to CLP and DSD | Classification for skin corrosion/irritation is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.1 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

2.2.6.1.3 Eye irritation

| Conclusion used in Risk Assessment – Eye irritation |  |
| --- | --- |
| Value/conclusion | Jerbor is irritant to the eyes. |
| Justification for the value/conclusion | Boric acid is not classified as an eye irritant (Assessment Report (CAR) for Boric acid, 2009). Disodium tetraborate pentahydrate revealed eye irritant effects in two eye irritation studies and workers exposed occupationally to borax dust at concentrations of about 4 mg/m3 reported eye irritation (Assessment Report (CAR) for Disodium tetraborates, 2009). Therefore, it was concluded in the CAR that disodium tetraborate pentahydrate should be regarded as an eye irritant. The product Jerbor contains >10% of disodium tetraborate pentahydrate and should therefore be regarded as an eye irritant. |
| Classification of the product according to CLP and DSD | Eye Irrit. 2 H319 according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.2 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

2.2.6.1.4 Respiratory tract irritation

| Conclusion used in the Risk Assessment – Respiratory tract irritation |  |
| --- | --- |
| Value/conclusion | Jerbor is not classified as irritating to the respiratory tract. However, there is a national Swedish occupational exposure limit of 2 mg/m3 of borax (in dust; Swedish Work Environmental Authority; AFS 2018:1). |
| Justification for the conclusion | None of the substances are classified as a specific organ toxicant (STOT) in category 1 or 2 as regards to respiratory tract irritation. Therefore, no classification for respiratory tract irritation is warranted. However, according to studies on workers cited in the CARs (Assessment Reports for Boric acid and Disodium tetraborate, 2009), the workers occupationally exposed to borax dust at concentrations of about 4 mg/m3 reported effects on the respiratory tract (such as dry mouth, nose or throat, sore throat, acute and chronic respiratory irritation and productive cough). |
| Classification of the product according to CLP and DSD | Classification for respiratory tract irritation is not required according to Regulation (EC) No 1272/2008 |

2.2.6.1.5 Skin sensitization

| Conclusion used in Risk Assessment – Skin sensitization |  |
| --- | --- |
| Value/conclusion | Jerbor is not a skin sensitizer. |
| Justification for the value/conclusion | No signs of skin sensitization were observed in Buehler method skin sensitization tests with boric acid or disodium tetraborates and no evidence of skin sensitization in humans exposed occupationally to borates has been reported (Assessment Reports (CAR) for Boric acid and Disodium tetraborate, 2009) and therefore, the substances do not need to be classified as skin sensitizers. |
| Classification of the product according to CLP and DSD | Classification for skin sensitization is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.3 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

2.2.6.1.6 Respiratory sensitization (ADS)

| Conclusion used in Risk Assessment – Respiratory sensitisation |  |
| --- | --- |
| Value/conclusion | Jerbor is not a respiratory sensitizer. |
| Justification for the value/conclusion | Boric acid and disodium tetraborate pentahydrate have not been tested for respiratory sensitization. However, since none of the substances show skin sensitizing potential they are not predicted to be respiratory sensitizers. Therefore, it can be concluded that Jerbor most likely is not a respiratory sensitizer. |
| Classification of the product according to CLP and DSD | Classification for respiratory sensitization is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.4 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

2.2.6.1.7 Acute toxicity

Acute toxicity by oral route

| Value used in the Risk Assessment – Acute oral toxicity |  |
| --- | --- |
| Value | Jebor is not acutely toxic via the oral route. |
| Justification for the selected value | Boric acid and other borates are of low acute oral toxicity: LD50 oral rat >2000 mg/kg bw (Assessment Reports (CAR) for Boric acid and Disodium tetraborate, 2009). |
| Classification of the product according to CLP and DSD | Classification for acute oral toxicity is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.5.1 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

Acute toxicity by inhalation

| Value used in the Risk Assessment – Acute inhalation toxicity |  |
| --- | --- |
| Value | Jebor is not acutely toxic via inhalation. |
| Justification for the selected value | Boric acid and other borates are of low acute toxicity by inhalation: LC50 inhalation rat >2 mg/l (highest attainable concentration) (Assessment Reports (CAR) for Boric acid and Disodium tetraborate, 2009). |
| Classification of the product according to CLP and DSD | Classification for acute toxicity by inhalation is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.5.2 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

Acute toxicity by dermal route

| Value used in the Risk Assessment – Acute dermal toxicity |  |
| --- | --- |
| Value | Jebor is not acutely toxic via the dermal route. |
| Justification for the selected value | Boric acid and other borates are of low acute toxicity by the dermal route: LD50 dermal rat >2000 mg/kg bw (Assessment Reports (CAR) for Boric acid and Disodium tetraborate, 2009). |
| Classification of the product according to CLP and DSD | Classification for acute dermal toxicity is not required according to Regulation (EC) No 1272/2008 |

| Data waiving |  |
| --- | --- |
| Information requirement | BPR Annex III 8.5.3 |
| Justification | Testing of the product Jerbor has not been performed as there are valid data available on each of the components in the mixture sufficient to allow classification according to Regulation (EC) No 1272/2008. |

2.2.6.1.8 Information on dermal absorption

Jerbor (dry)

| Value(s) used in the Risk Assessment – Dermal absorption |  |  |
| --- | --- | --- |
| Substance | Boric acid | Disodium tetraborate pentahydrate |
| Value(s) | 10% | 10% |
| Justification for the selected value(s) | Since no data from studies on dermal absorption of dry boric acid is available, the default value of 10% for water-based/dispersed or solid concentrated formulations, from EFSA Guidance on dermal absorption (2017), has been used in the risk assessment. | Since no data from studies on dermal absorption of dry disodium tetraborate is available, the default value of 10% for water-based/dispersed or solid concentrated formulations, from EFSA Guidance on dermal absorption (2017), has been used in the risk assessment. |

Jerbor slurry (wet)

| Value(s) used in the Risk Assessment – Dermal absorption |  |  |
| --- | --- | --- |
| Substance | Boric acid | Disodium tetraborate pentahydrate |
| Value(s) | 5% | 5% |
| Justification for the selected value(s) | Using the default value (10%) from EFSA Guidance on dermal absorption (2017) for dermal absorption of the slurry would lead to an unacceptable risk (RR 1.22) for the combined exposure during mixing, application and daily cleaning. Therefore, a refined assessment of dermal absorption has been made. According to the CARs for the active substances (Assessment Reports, Boric acid and Disodium tetraborate, 2009) the value of dermal absorption for the active substances is 0.5 %. This value was based on an in vivo study with aqueous solutions of boric acid, disodium tetraborate decahydrate and disodium octaborate tetrahydrate. The slurry of Jerbor is also an aqueous solution of the active substances with no added co-formulants. The slurry however contains bentonite but due to its inert properties bentonite is not assumed to affect the dermal absorption of the slurry. Considering this, the data used for dermal absorption of the active substances can also be used in support of the product as a slurry. The concentration of active substance is higher in the slurry (9.0 % BAE) compared to the tested aqueous solution (5.0 % BAE). In general, the dermal absorption is inversely related to the concentration of the active substance and to derive a dermal absorption value of dilutions with higher concentrations than the tested solution the dermal absorption from the tested solution should apply (EFSA 2017) and it would therefore be worst-case to use for the higher concentrated solution. There are uncertainties with the underlying data and the study results have been questioned due to low recovery (49-64 %) of the administered dose (but in support of toxicokinetic studies the CAR however concluded the that the dermal absorption of the active substances is predicted to be low). For products with boric acid and also containing potential penetration enhancers a dermal absorption of 20 % has previously been suggested by DE using expert judgement. One of the arguments for this value was based on the assumption that only approximately 10 % of the applied dose in the above-mentioned dermal absorption study was available for absorption. This expert judgement has been accepted and used for several PT8 products with penetration enhancers. Using the same assumption, that only 10 % of the applied dose is available for dermal absorption, the applicant suggested to use a value of 5 % for the slurry which is achieved when correcting for variability and loss of material in the study. The result from the study, 0.5 %, was corrected for the loss of material resulting in 5 % (0.5 % x 10). Based on the above argumentation, the value of 5 % dermal absorption, is used in the risk assessment for the slurry. | Using the default value (10%) from EFSA Guidance on dermal absorption (2017) for dermal absorption of the slurry would lead to an unacceptable risk (RR 1.22) for the combined exposure during mixing, application and daily cleaning. Therefore, a refined assessment of dermal absorption has been made. According to the CARs for the active substances (Assessment Reports, Boric acid and Disodium tetraborate, 2009) the value of dermal absorption for the active substances is 0.5 %. This value was based on an in vivo study with aqueous solutions of boric acid, disodium tetraborate decahydrate and disodium octaborate tetrahydrate. The slurry of Jerbor is also an aqueous solution of the active substances with no added co-formulants. The slurry however contains bentonite but due to its inert properties bentonite is not assumed to affect the dermal absorption of the slurry. Considering this, the data used for dermal absorption of the active substances can also be used in support of the product as a slurry. The concentration of active substance is higher in the slurry (9.0 % BAE) compared to the tested aqueous solution (5.0 % BAE). In general, the dermal absorption is inversely related to the concentration of the active substance and to derive a dermal absorption value of dilutions with higher concentrations than the tested solution the dermal absorption from the tested solution should apply (EFSA 2017) and it would therefore be worst-case to use for the higher concentrated solution. There are uncertainties with the underlying data and the study results have been questioned due to low recovery (49-64 %) of the administered dose (but in support of toxicokinetic studies the CAR however concluded the that the dermal absorption of the active substances is predicted to be low). For products with boric acid and also containing potential penetration enhancers a dermal absorption of 20 % has previously been suggested by DE using expert judgement. One of the arguments for this value was based on the assumption that only approximately 10 % of the applied dose in the above-mentioned dermal absorption study was available for absorption. This expert judgement has been accepted and used for several PT8 products with penetration enhancers. Using the same assumption, that only 10 % of the applied dose is available for dermal absorption, the applicant suggested to use a value of 5 % for the slurry which is achieved when correcting for variability and loss of material in the study. The result from the study, 0.5 %, was corrected for the loss of material resulting in 5 % (0.5 % x 10). Based on the above argumentation, the value of 5 % dermal absorption, is used in the risk assessment for the slurry. |

2.2.6.1.9 Available toxicological data relating to non-active substance(s) (i.e. substance(s) of concern)

Not relevant since the product contains no substances of concern.

2.2.6.1.10 Available toxicological data relating to a mixture

The final treatment slurry contains of 9.6% Jerbor, 70.4% water and 20% sodium bentonite. Sodium bentonite has no harmonised classification and is not expected to have any adverse health effect, except that it, as a powder, may cause serious eye irritation, skin irritation and respiratory irritation (notified classification; C&L inventory, ECHA).

2.2.6.1.11 Other information

Boric acid is classified for reproductive toxicity (Repr. 1B, H360FD: May damage fertility. May damage the unborn child) and disodium tetraborate pentahydrate is notifed for the same classification. In the CLH report (Proposal for Harmonised Classification and Labelling) submitted by SE (Nov 2018) a harmonised classification as Repr. 1B, H360FD was proposed also for disodium tetraborate pentahydrate. Furthermore, it was proposed that the Generic Concentration Limit (GCL) of 0.3% (for substances classified Repr. 1A and 1B) should be applied to all borates.  At the RAC (Risk Assessment Committee, ECHA) meeting 16.09.2019, the opinion was adopted. Thus, Jerbor should be classified as Repr. 1B, H360FD.

The active substances boric acid and disodium tetraborate pentahydrate are currently not considered to have endocrine disrupting (ED) properties according to Regulation (EU) 528/2012.  The substances have not been assessed according to the new ED criteria (Commission Delegated Regulation (EU) 2017/2100). However, the current guidance for application of these criteria (CA March18 Doc.7.3b-final, paragraph 19) specifies that the evaluating body should not evaluate endocrine disrupting properties nor request additional data on ED properties of active substances in the context of product authorisation procedures.

2.2.6.2 Exposure assessment

Identification of main paths of human exposure towards active substance(s) and substances of concern from its use in biocidal product

| Summary table: relevant paths of human exposure |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Exposure path | Primary (direct) exposure |  |  | Secondary (indirect) exposure |  |  |  |
|  | Industrial use | Professional use | Non-professional use | Industrial use | Professional use | General public | Via food |
| Inhalation | N.A. | Yes | N.A. | N.A. | N.A. | N.A. | N.A. |
| Dermal | N.A. | Yes | N.A. | N.A. | N.A. | N.A. | N.A. |
| Oral | N.A. | No | N.A. | N.A. | N.A. | N.A. | N.A. |

2.2.6.2.1. Description of Jerbor and the intended use

The biocidal product Jerbor is a water-soluble product used against rot in the ground reinforcement wood (poles, etc.). Jerbor is only used by professionals in accordance with the instructions for use (2.1.4.2). For more specific requirements and details, see Manual Euroborbehandling (manual of Eurobor technique) (Bandh, 2022).

The product contains 46.3 wt-% of disodium tetraborate pentahydrate and 53.7 wt-% of boric acid as active ingredients. Both substances are relevant for the risk assessment of the biocidal product. The toxic effects of these substances are equal due to the content of boron.

The physical state of the product is granular. Use of the product is restricted to a prepared solution with dilution of Jerbor (9.6 wt-%) in hot water (50 - 70°C) (70.4 wt-%) mixed with the thickener sodium bentonite (20 wt-%).

The slurry of the biocidal product Jerbor contains only the above-mentioned ingredients. The sodium bentonite used in the slurry, at a concentration of 20% (w/w) (CAS No. 1302-78-9), is not considered a dangerous substance. It is not expected to have any serious adverse health effects, except that it, as a powder, may cause serious eye irritation, skin irritation and respiratory irritation (notified classification; C&L inventory, ECHA).

| Summary table: scenarios |  |  |  |
| --- | --- | --- | --- |
| Scenario number | Scenario (e.g. mixing/ loading) | Primary or secondary exposure Description of scenario | Exposed group (e.g. professionals, non-professionals, bystanders) |
| 1a. | Production | Manufacturing- Inhalation - Primary exposure | Professionals |
| 1b. | Production | Manufacturing - Dermal - Primary exposure | Professionals |
| 1c. | Production | Handling/packaging - Inhalation - Primary exposure | Professionals |
| 1d. | Production | Handling/packaging - Dermal - Primary exposure | Professionals |
| 2.1 a. | Loading mixing tank | Loading Jerbor into mixing tank- Inhalation - Primary exposure | Professionals |
| 2.1 b. | Loading mixing tank | Loading Jerbor into mixing tank– Dermal – Primary exposure | Professionals |
| 2.2 a. | Mixing tank | Mixing Jerbor & water - Inhalation - Primary exposure | Professionals |
| 2.2 b. | Mixing tank | Mixing Jerbor & water - Dermal - Primary exposure | Professionals |
| 3a. | Transfer to mixing barrel | Transfer from mixing tank to mixing barrel - Inhalation- Primary exposure | Professionals |
| 3b. | Transfer to mixing barrel | Transfer from mixing tank to mixing barrel - Dermal- Primary exposure | Professionals |
| 4a. | Mixing barrel | Mixing Jerbor water solution & bentonite - Inhalation - Primary exposure | Professionals |
| 4b. | Mixing barrel | Mixing Jerbor water solution & bentonite - Dermal - Primary exposure | Professionals |
| 5a | Transfer to injection pump | Transfer mixing barrel to injection pump - Inhalation- Primary exposure | Professionals |
| 5b | Transfer to injection pump | Transfer mixing barrel to injection pump - Dermal- Primary exposure | Professionals |
| 6a. | Application | Application - Inhalation - Primary exposure | Professionals |
| 6b. | Application | Application - Dermal - Primary exposure | Professionals |
| 7a. | Daily cleaning | Daily cleaning - Inhalation - Primary exposure | Professionals |
| 7b. | Daily cleaning | Daily cleaning - Dermal - Primary exposure | Professionals |
| 8a. | Final cleaning | Cleaning after completed project- Inhalation - Primary exposure | Professionals |
| 8b. | Final cleaning | Cleaning after completed project- Dermal - Primary exposure | Professionals |
| 9a. | Maintenance/repair | Maintenance and repair – Inhalation – Primary exposure | Professionals |
| 9b. | Maintenance/repair | Maintenance and repair – Dermal – Primary exposure | Professionals |

There are many steps involved in the production and use of the product Jerbor which are relevant for the exposure assessment. A schematic picture can be found below (Fig. 2.2.6.2-1) which illustrates how the mixing of Jerbor takes place at the site of treatment.

Figure 2.2.6.2.2-1 Illustrate scenario 2-7 of the Jerbor preparation and treatment. From right to left; Blandningstank (med lock) =Mixing tank (with a lid) for Jerbor and water (250 litres stainless steel, conical bottom, with circulation pump), Högblandare med 50 liters blandningskärl  = Mounted plaster and mortar mixer with 50 liter vessel (mounted in a holder with tilt function),  Fjärrstyrd injekteringspump = Remote controlled plaster and mortar pump, Tryckgivare = Manometer. The picture is taken from the Manual Euroborbehandling (Bandh 2022).

2.2.6.2.2 Method used for Exposure Assessment

The ECETOC Targeted Risk Assessment (TRA) tool

Since the use of Jerbor is different in many ways compared to a traditional wood preservative and no suitable exposure scenarios are available in relevant guidelines for biocides, the applicant has used ECETOC (European Center or Ecotoxicology and Toxicology of Chemicals) TRA tool. ECETOC TRA is a recommended Tier 1 model for exposure estimation for occupational exposure within the REACH legislation and is based on pre-defined PROCs (process categories) describing tasks, application techniques or process types.

Although the TRA tool is considered to be conservative in lower tier assessments it should be used wisely when applied to CMRs. CMRs and other highly hazardous substances are often handled under specific conditions to prevent and control exposure. These conditions cannot be readily translated into available input values (Guidance on information requirements and chemical safety assessment Chapter R.12 Use description and Chapter R.14: Occupational exposure estimation). For instance for personal protective equipment, it is only possible to specify protection factor (PF) for gloves and respiratory mask.

Even though there are disadvantages when using ECETOC TRA tool for CMRs, SE CA agrees that this tool could be used for calculating the exposure of professionals during the different steps when handling the product and the Eurobor slurry, since the professionals using the biocidal product are trained according to Manual Euroborbehandling and should wear respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protection goggles.

2.2.6.2.3 Industrial exposure

This section is not relevant as the biocidal product is for professional use only.

2.2.6.2.4 Professional exposure

The production, mixing, application, cleaning and maintenance/repair (scenario 1-9) are always carried out by approved, educated and professional personnel. According to Manual Euroborbehandling (manual of Eurobor technique) only personnel working with Jerbor shall be present during treatment, i.e. exposure scenarios for bystanders do not need to be taken into account. In case of bystanders (other workers) the exposure scenarios for professionals will cover also bystanders.

The PROC’s used in the TRA tool for the different scenarios are described in Table below.

| The PROC’s used in the TRA tool for all the relevant scenarios |  |  |  |
| --- | --- | --- | --- |
| PROC3 | Use in closed batch process (synthesis or formulation) | Batch manufacture of a chemical or formulation where the pre-dominant handling is in a contained manner, e.g. through enclosed transfers, but where some opportunity for contact with chemicals occurs, e.g. through sampling | 2.2 a-b 6 a-b |
| PROC5 | Mixing or blending in batch processes for formulation of preparations and articles (multi stage and/or significant contact) | Manufacture or formulation of chemical products or articles using technologies related to mixing and blending of solid or liquid materials, and where the process is in stages and provides the opportunity for significant contact at any stage | 1 a-d 4 a-b |
| PROC8a | Transfer of substance or preparation (charging/discharging) from/to vessels/large containers at non-dedicated facilities | Sampling, loading, filling, transfer, dumping, bagging in non- dedicated facilities. Exposure related to dust, vapour, aerosols or spillage, and cleaning of equipment to be expected | 2.1 a-b 3 a-b 5 a-b 7 a-b 8 a-b (9 a-b) |

Scenarios 1a-d: Production - Proc 5

The active ingredients are delivered in 25 kg plastic bags to a storing and handling facility. Jerbor is only produced on demand according to the estimated need for a treatment. When producing Jerbor, 46.3 % disodium tetraborate pentahydrate and 53.7 % boric acid are mixed in a 30 dm3 container. To minimise dusting only granular disodium tetraborate and boric acid are used in the production of Jerbor. The packages are opened in a ventilated hood behind a transparent door, so that the content can go directly into the transport containers. Each batch (20.6 kg) is packed into UN approved transport containers with a tightly sealed lid (30 litres) and labelled according to CLP and transported to the construction site by truck.

Only dry granules are handled. The facility has general ventilation and LEV (Local Exhaust Ventilation) at the site of opening of the bags and at the site of filling of the containers. The person performing this work should wear respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protection googles.

The production step will be done indoors, 20 days/year during 2 h, by one person.

Scenarios 2.1 a-b: Loading mixing tank – Proc 8a

The equipment for Jerbor injection will be installed indoors in the building where the injection (treatment) takes place. The dry Jerbor granules is manually transferred to the mixing tank by gently tipping, to avoid splashing, the transport container with Jerbor into the hot water. When the transport container is completely emptied, the lid of the mixing tank is put on place. The work will be done by one person.

The building has normally general ventilation. The person performing this work should wear respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protection goggles.

The total number of working days per year for scenario 2.1 is about 100 days. This working step will be done 5 times per day during 2 min per time which gives total 10 minutes per person and day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 2.2 a-b: Mixing tank – Proc 3

In the mixing tank (denoted blandningstank med lock in Figure 2.2.6.2.2-1) Jerbor in granular form and water will be mixed automatically in a closed system. The lid will be opened, after 8 minutes of mixing, to inspect that Jerbor is fully solved. Prolonged mixing will occasionally occur if the product is not solved and a new inspection will then take place. The work will be done by one person. The mixture will contain 12.0 % Jerbor and 88.0 % warm (50 – 70°C) water.

The building has normally general ventilation. The person doing this work wears respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protection goggles.

The total number of working days per year for scenario 2.2 is about 100 working days. This working step will be done 5 times per day for 8 min per time which gives total

40 minutes per person and day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 3a-b: Transfer from mixing tank to mixing barrel – Proc 8a

The transfer of the Jerbor/water-solution from the mixing tank to the mixing barrel is performed by a transfer hose connected with a coupling to the mixing tank and the other end hanging over the edge of the mixing barrel so that the Jerbor solution will flow straight into the barrel (transfer from “blandningstank” to “högblandare med 50 liters blandningskärl” in 2.2.6.2-1).

The work will be done by one person. Only dissolved borates are handled in this step. The building has normally general ventilation. The person performing this work wears respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protective googles.

The total number of working days per year for scenario 3 is about 100 working days. This working step will be done 25 times per day, 1 min at a time, which gives total

25 minutes per day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 4a-b: Mixing barrel – Proc 5

In the mixing barrel, 18 litres of the Jerbor water solution will be mixed with 4.8 kg of sodium bentonite with a mounted plaster and mortar mixer in an opened system (“högblandare med 50 liters blandningskärl” in Figure 2.2.6.2.2-1). The bentonite is manually poured carefully in small portions, to avoid splashing, into the Jerbor slurry. The bentonite is added to make the mixture viscous (and for it to swell after injection). The mixture will now contain 9.6 % Jerbor and 70.4 % water and 20 % bentonite (all w/w) and is ready for application.

The work will be done by one person. Only dissolved borates are handled in this step. The building has normally general ventilation. The person doing this work wears respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protection googles.

The total number of working days per year for scenario 4 is about 100 working days. This working step will be done 25 times per day, 2 min at a time, which gives total

50 minutes per day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 5a-b – Transfer from mixing barrel to injection pump – Proc 8a

The transfer from the mixing barrel to the injection pump (transfer from “högblandare med 50 liters blandningskärl” to “Fjärrstyrd injekteringspump” in Figure 2.2.6.2.2-1) is done by pouring 20 litres of slurry by gently tilting the mixing vessel into the remote-controlled plaster and mortar pump.

Only dissolved borates are handled in this step. The building has normally general ventilation. The person doing this work wears respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber and protective googles.

The total number of working days per year for scenario 5 is about 100 working days. This working step will be done 25 times per day, 15 sec at a time, which gives total

6 minutes per person and day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 6 a-b – Application – Proc 3

Application of the product can now be performed (transfer from “Fjärrstyrd injekteringspump” to “Euroborslurry injekteras” in Figure 2.2.6.2.2-1). The transfer from the plaster and mortar pump to the injection place is always done with low pressure (usually below 1 bar and never above 3 bar) and low flow (approx. 2 litres per minute). The transfer will be done in a closed system. Communication between the injection site and installation site for the pump takes place continuously via communication radio.

Only dissolved borates are handled in this step. The building has normally general ventilation. The persons doing this work wear protective clothing and protective gloves of nitrile rubber.

The total number of working days per year for scenario 6 is about 100 working days. This application step is done during 6h/working day.  Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 7 a-b – Daily cleaning – Proc 8a

The working day ends with cleaning of the equipment. First, the mixing barrel is rinsed out with hot water. The wash water is then transferred to the plaster and mortar pump, and from there it will be discarded to the injection site. The plaster and mortar pump are then also rinsed out with hot water and the wash water is disposed in the same way. Finally, the hoses and the injection packer are flushed out with hot water, and this wash water is also disposed to the treatment zone of the foundation.

This step is performed by one person (one of the persons doing the injection work in scenario 6) and takes about 10 minutes/day.

Only dissolved borates are handled in this step. The person doing this work wears protective clothing, protective gloves of nitrile rubber and protection googles.

The total number of working days per year for scenario 7 is about 100 working days. This working step will be done for 10 minutes and day. Working steps 2.1 to 7 are rotated between the professionals working at the site of treatment during the day.

Scenarios 8 a-b – Final cleaning after completed project -Proc 8a

More careful cleaning is performed after the treatment with Jerbor are completed. The equipment and hoses are transferred to a storehouse and cleaned thoroughly with water. This step is performed by one person and takes about 2h each time and is done approximately 10 times a year. The person doing this work wears protective clothing, protective gloves of nitrile rubber and protective googles.

The total number of working days per year for Scenario 8 is 10 working days. The final cleaning takes about 2h per day.

Scenarios 9 a-b – Maintenance/repair -Proc 8a

This scenario could be considered to be covered by scenarios 7 and 8 as the main part of the maintenance/repair is made during the daily/final cleaning.

The person doing this work wears protective clothing, protective gloves of nitrile rubber and protection googles.

Critical endpoints and assessment factors used for calculation of AOEL

Overall NOAEL

The major targets for toxicity of borates are the testes and the blood. In the repeated dose studies with mouse, rat and dog, consistently effects on the testes and on blood parameters were found. In a 90-day study in mouse, the animals appeared to be more sensitive to the effects on the haematopoietic system than on the testes. In the rat effects on both the testes and on the blood were observed at dose levels of 334 mg boric acid/kg bw/day. The NOAEL in this study was 100 mg/kg bw/day (17.5 mg boron/kg bw/day). Based on the NOAEL, from the rat study, for embryotoxic/teratogenic effects of boric acid of 55 mg/kg bw/day (9.6 mg boron/kg bw/day) the overall NOAEL is 9.6 mg boron/kg bw/day.  Similar results were obtained from studies with disodium tetraborate at equimolar doses of boron (Assessment Reports (CARs) for Boric acid and Disodium tetraborate 2009).

Total assessment factor

The available information on kinetics and dynamics does not allow refinement of the standard assessment factors (10 for interspecies variation and 10 for intraspecies variation). There are indications that the sensitivity for the effects on the testes does not differ markedly between subchronic and chronic exposure. Therefore, the application of an additional assessment factor for extrapolation from subchronic to chronic exposure is not required (Assessment Reports (CARs) for Boric acid and Disodium tetraborate 2009).

AOEL (acceptable operator exposure level)

Using the standard assessment factor of 100 an oral AOEL of 0.096 mg B/kg bw/day can be derived based on the NOAEL for embryotoxic/teratogenic effects of boric acid of 9.6 mg B/kg bw/day.

AOEL = 0.1 mg B/kg bw/day (Assessment Reports (CARs) for Boric acid and Disodium tetraborate 2009).

Primary exposure

Assumption used in the estimation of exposure in TRA:

- Adult bodyweight 60 kg

- Given that this product is solely used by professionals PPE has been included

- Dermal PPE (gloves); 90% protection

- Respiratory PPE; 90% protection

- Inhalation rate 1.25 m3/h, inhalation uptake 100%; 10 m3 during a working day (8 h)

- Dermal absorption for borates from Jerbor granulates (dry): 10%.

- Dermal absorption for borates from Jerbor slurry (wet): 5%.

- AOEL = 0.1 mg B/kg bw/day which gives:

- TRA input Threshold value long-term dermal, systemic for Jerbor slurry= AOEL (mg B/kg bw/day)/dermal absorption (5%) = 0.1 mg/kg/day/0.05 = 2 mg B/kg bw/day

- TRA input Threshold value long-term dermal, systemic for Jerbor granulate= AOEL (mg B/kg bw/day)/dermal absorption (10%) = 0.1 mg/kg/day/0.1 = 1 mg B/kg bw/day

- TRA input Threshold value long-term inhalation, systemic = AOEL (mg B/kg bw/day) x body weight (kg)/inhaled volume air (m3/day) = 0.1 mg B/kg bw/day x 60 kg/10 m3/day = 0.6 mg B/m3

The toxicokinetics and toxicological effects of boric acid and disodium tetraborate pentahydrate are likely to be similar on a boron equivalents basis, therefore, the active substances are converted to equivalents of boron, see below.

| Conversion factors for boron species into equivalent doses of boric acid equivalents or boron |  |  |  |
| --- | --- | --- | --- |
|  | Molecular formula | Molecular weight (g/mol) | Conversion factor for equivalent dose of boron (B), multiply by |
| Boric acid | H3BO3 | 61.83 | 0.175 |
| Disodium tetraborate pentahydrate | Na2B4O75H2O | 291.296 | 0.148 |

| The used input parameters in ECETOC TRA for exposure times and boron equivalent concentrations per exposure scenario 1 – 8* |  |  |
| --- | --- | --- |
| Exposure scenario | Exposure time | Boron equivalents |
| Scenario 1 Production | 1-4 hours | 46.3 % borax x 0.148 + 53.7 % boric acid x 0.175 = 6.85 + 9.39 = 16.24% boron equivalents |
| Scenario 2.1 Loading mixing tank | Less than 15 min | Same as Scenario 1: 16.24% boron equivalents |
| Scenario 2.2 Mixing tank | 15-60 min | 5.56% borax x 0.148 + 6.45% boric acid x 0.175 = 0.82 + 1.13 = 1.95% boron equivalents |
| Scenario 3 Transfer to mixing barrel | 15-60 min | Same as Scenario 2b: 1.95% boron equivalents |
| Scenario 4 Mixing barrel | 15-60 min | 4.44 % borax x 0.148 + 5.16 % boric acid x 0.175 = 0.66 + 0.90 = 1.56% boron equivalents |
| Scenario 5 Transfer to injection pump | Less than 15 min | Same as scenario 4: 1.56% boron equivalents |
| Scenario 6 Application | >4 hours | Same as scenario 4: 1.56% boron equivalents |
| Scenario 7 Daily cleaning | Less than 15 min | Same as scenario 4: 1.56% boron equivalents |
| Scenario 8 Final cleaning | 1-4 hours | Same as scenario 4: 1.56% boron equivalents |

*No calculations for scenario 9 (maintenance/repair) have been performed as the exposure is covered by scenario 7 and 8.

Calculations for Scenario [1-9]

For the calculations of the exposures see Appendix 3.3. Ecetoc TRA calculations professionals.

| Exposure scenario | Tier/PPE | Estimatedlong-term inhalation uptake (mg/m3) | Estimated long-term inhalation uptake (mg/kg/ day)* | Estimated long-term dermal uptake(mg/kg/day) ** | Estimated total systemic long-term uptake (Dermal & Inhalation Exposure) (mg/kg/day) |
| --- | --- | --- | --- | --- | --- |
| 1 a-d Production | LEV efficiency [%]: 80, Respiratory mask with particle filter P3, protective clothing and protective gloves of nitrile rubber, Protective goggles | 0.0072 | 0.0006 (4h) | 0.0494 | 0.05 |
| 2.1 a-b Transfer to mixing tank | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.003 | 0.00002 (15 min) | 0.0082 | 0.0082 |
| 2.2 a-b Mixing tank | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.00018 | 0.000004 (1 h) | 0.0007 | 0.0007 |
| 3 a-b Transfer to mixing barrel | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.00018 | 0.000004 (1 h) | 0.0137 | 0.0137 |
| 4 a-b Mixing barrel | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.00018 | 0.000004 (1 h) | 0.0137 | 0.0137 |
| 5 a-b Transfer to injection pump | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.00009 | 0.0000005 (15 min) | 0.0137 | 0.0137 |
| 6 a-b Application | Protective clothing & protective gloves of nitrile rubber. | 0.009 | 0.0011 (6 h) | 0.000685 | 0.001785 |
| 7 a-b Daily cleaning | Protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0009 | 0.000005 (15 min) | 0.0137 | 0.0137 |
| 8 a-b Final cleaning | Protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0054 | 0.00045 (4h) | 0.0137 | 0.0142 |
| 9 a-b Maintenance/repair*** | Protective clothing, protective gloves of nitrile rubber, Protective goggles | - | - | - | - |

*These values have been calculated using the data on long-term inhalation exposure from ECETOC TRA and a body weight of 60 kg and an inhalation rate 1.25 m3/h. The time indicated in parentheses is the working time/day used in the calculation.

** The estimated long-term dermal exposure uptake has been calculated by using the data from ECETOC TRA and 10% dermal uptake of borates in Jerbor granulates and 5% dermal uptake of borates in Jerbor slurry, respectively.

*** No calculations for scenario 9 (maintenance/repair) have been performed as the exposure is covered by scenario 7 and 8.

Further information and considerations on scenario [1-9]

Since Jerbor is classified Eye Irritant category 2 (H319), local risk characterisation has been performed (see 2.2.6.3.2 Risk for professional users, Local effects).

2.2.6.2.5 Combined scenarios

The production takes place in a different location and not at the same day as the mixing and applying of the slurry, therefore the production step should not be combined with the other scenarios.

In the location where mixing of the slurry and the application takes place one professional could be mixing, applying and do the daily cleaning within the same day as the team working in the treatment location is rotating between the different tasks.

The final cleaning step takes place in a location different from the treatment area. The equipment is first cleaned as for daily cleaning and then taken apart and transported to a location where it is thoroughly cleaned and then stored away. This step does not take place within the same day as the mixing, application and daily cleaning and therefore the final cleaning is not combined with other scenarios.

Maintenance/repair is mainly performed during the daily and final cleaning therefore these scenarios are covering for scenario 9.

| Scenarios combined | Estimated inhalation uptake (mg/kg/day) | Estimated dermal uptake (mg/kg/day) | Estimated total uptake (mg/kg/day) |
| --- | --- | --- | --- |
| Scenarios [1] | 0.0006 | 0.0494 | 0.050 |
| Scenarios [2.1-7] | 0.0011 | 0.0630 | 0.064 |
| Scenarios [8] | 0.00045 | 0.0137 | 0.014 |

2.2.6.2.6 Non-professional exposure

Not relevant. Professional use only.

2.2.6.2.7 Exposure of the general public

Not relevant since only professionals are allowed in the working areas.

2.2.6.2.8 Dietary exposure

Not relevant since the biocidal product should not be used in a way that results in dietary exposure.

Estimating livestock exposure to active substances used in biocidal product

Not relevant since the treatment with Jerbor will not result in exposure to livestock animals.

Estimating transfer of biocidal active substances into foods as a result of professional use

Not relevant since the treatment with Jerbor will not result in transfer of biocidal active substances into foods.

Estimating transfer of biocidal active substances into foods as a result of non-professional use

Not relevant since the treatment with Jerbor will not result in transfer of biocidal active substances into foods.

2.2.6.2.9 Exposure associated with production, formulation and disposal of the biocidal product

See table “Calculations for scenario [1-9]” on page 61.

2.2.6.2.10 Aggregated exposure

This section is not relevant since exposure to boric acid and disodium tetraborate from other sources than the biocidal product is expected to be negligible.

Summary of exposure assessment

| Scenarios and values to be used in risk assessment |  |  |  |
| --- | --- | --- | --- |
| Scenario number | Exposed group | Tier/PPE | Estimated total uptake (mg/kg/day) |
| 1 a-d Production | Professionals | LEV efficiency [%]: 80, Respiratory mask with particle filter P3, protective clothing and protective gloves of nitrile rubber, Protective goggles | 0.05 |
| 2.1 a-b Transfer to mixing tank | Professionals | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0082 |
| 2.2 a-b Mixing tank | Professionals | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0007 |
| 3 a-b Transfer to mixing barrel | Professionals | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0137 |
| 4 a-b Mixing barrel | Professionals | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0137 |
| 5 a-b Transfer to injection pump | Professionals | Respiratory mask with particle filter P3, protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0137 |
| 6 a-b Application | Professionals | Protective clothing & protective gloves of nitrile rubber | 0.001785 |
| 7 a-b Daily cleaning | Professionals | Protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0137 |
| 8 a-b Final cleaning | Professionals | Protective clothing, protective gloves of nitrile rubber, Protective goggles | 0.0142 |
| 9 a-b Maintenance/repair | Professionals | Protective clothing, protective gloves of nitrile rubber, Protective goggles | No calculations for scenario 9 (maintenance/repair) have been performed as the exposure is covered by scenario 7 and 8. |

2.2.6.3 Risk characterisation for human health

Reference values to be used in Risk Characterisation

Boric acid and Disodium tetraborate

| Reference | Study | NOAEL (LOAEL) | AF1 | Correction for oral absorption | Value |
| --- | --- | --- | --- | --- | --- |
| AOELshort-term AOELmedium-term AOEL long-term | Embryotoxicity/ teratogenicity study (rat) | 9.6 mg B/kg bw/day | 1001 | Not relevant | 0.1 mg B/kg bw/day |
| ARfD |  |  |  |  | Not relevant |
| ADI |  |  |  |  | Not relevant |

1 10 for interspecies- and 10 for intraspecies variation

For information on critical endpoints and assessment factors see section 2.2.6.2.4; Critical endpoints and assessment factors used for calculation of AOEL.

2.2.6.3.1 Risk for industrial users

This section is not relevant as the biocidal product is for professional use only.

2.2.6.3.2 Risk for professional users

Systemic effects

| Task/ Scenario | Systemic NOAEL mg/kg bw/d | AOEL mg/kg bw/d | Estimated uptake mg/kg bw/d | Estimated uptake/ AEL (%) | Acceptable (yes/no) |
| --- | --- | --- | --- | --- | --- |
| 1 a-d Production | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.05 | 50% | yes |
| 2.1 a-b Transfer to mixing tank | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0082 | 8.2% | yes |
| 2.2 a-b Mixing tank | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0007 | 0.7% | yes |
| 3 a-b Transfer to mixing barrel | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0137 | 14% | yes |
| 4 a-b Mixing barrel | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0137 | 14% | yes |
| 5 a-b Transfer to injection pump | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0137 | 14% | yes |
| 6 a-b Application | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.001785 | 1.8% | yes |
| 7 a-b Daily cleaning | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0137 | 14% | yes |
| 8 a-b Final cleaning | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0142 | 14% | Yes |
| 9 a-b*Maintenance/repair | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | - | - | - |

* No calculations for scenario 9 (maintenance/repair) have been performed as the exposure is covered by scenario 7 and 8.

Combined scenarios

| Scenarios combined | Systemic NOAEL mg/kg bw/d | AOEL mg/kg bw/d | Estimated uptake mg/kg bw/d | Estimated uptake/ AEL (%) | Acceptable (yes/no) |
| --- | --- | --- | --- | --- | --- |
| Scenario [1] | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.05 | 50% | yes |
| Scenarios [2.1-7] | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.064 | 64% | yes |
| Scenario [8] | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | 0.0142 | 14% | Yes |
| Scenario [9]* | 9.6 mg B/kg bw/day | 0.1 mg B/kg bw/day | - | - | - |

* No calculations for scenario 9 (maintenance/repair) have been performed as the exposure is covered by scenario 7 and 8.

Secondary exposure

Since no bystander are allowed during treatment and the applied product is contained under the building foundation below the cellar floor in combination with the very low vapour pressure for the active substances, no secondary exposure as a result of use of the active substance in the biocidal product is expected.

Conclusion

Risk assessment has been performed for the different steps (production, mixing, transfer, application, cleaning and maintenance/repair) involved in the Jerbor treatment. The tool ECETOC TRA, which is considered as valid as a lower tier tool for exposure assessment under the REACH legislation, has been used for the exposure calculations and risk assessments.

The risk quotient has been calculated for Scenario 1 (production of the product Jerbor), Scenarios 2.1-7 (combined; mixing/loading, application of the Eurobor slurry and daily cleaning) and Scenario 8 (final cleaning), respectively and gave 50%, 64% and 14% of the AOEL.  Maintenance/repair is mainly performed during the daily and final cleaning therefore these scenarios are covering for scenario 9.

The percentage of AOEL were calculated by using total systemic long-term values from dermal and inhalation exposure recalculated to mg/kg/day. Thus, it can be concluded that Jerbor can be used in the different scenarios (production, mixing/loading, application, cleaning and maintenance/repair) without any unacceptable risk for the operators, provided that the prescribed PPE is worn and that the work is performed in accordance with Manual Euroborbehandling (Bandh, 2022).

2.2.6.3.3 Local effects

According to the Guidance on the Biocidal Products Regulation Volume III Human Health - Assessment & Evaluation (Parts B+C) Version 4.0 December 2017 section 4.3.2, a risk characterisation for local effects is triggered when the product is classified for local effects.

Jerbor is classified as Eye Irritant category 2, H319. The Jerbor/water solution and the slurry have the concentration of 12% resp. 9.6% Jerbor. As Jerbor is a mix of boric acid (53.7%) and disodium tetraborate pentahydrate (46.3%) and only disodium tetraborate pentahydrate is classified as an eye irritant neither the Jerbor/water solution nor the slurry will contain a concentration of disodium tetraborate that will trigger classification as eye irritant H319. Therefore, a local risk characterisation for eye irritation has only been performed, according to the Guidance, for potential exposure to Jerbor dust.

Furthermore, there have been reports of irritation of pharynx by workers inhaling dust of borax, however as no classification for respiratory irritation is warranted no local risk characterisation has been performed.

The sodium bentonite used in the slurry is not expected to have any serious adverse health effects, except that it, as a powder, may cause serious eye irritation, skin irritation and respiratory irritation (notified classification; C&L inventory, ECHA). Considering this, a local risk characterisation has been performed for the potential exposure to dust of sodium bentonite.

Local qualitative risk characterisation

Respiratory irritation

Jerbor in its granular form is not classified as respiratory irritant. Sodium bentonite, as a powder, may cause respiratory irritation.

In scenario 4 sodium bentonite (powder) is mixed with the Jerbor/water-solution. In this scenario protective goggles, respiratory mask with particle filter P3, protective clothing and protective gloves of nitrile rubber are mandatory to wear.

There is no other scenario when respiratory irritation could be expected.

Dermal irritation

Jerbor in its granular form is not classified as dermal irritant. Sodium bentonite, as a powder, may cause skin irritation.

In scenario 4 sodium bentonite (powder) is mixed with the Jerbor/water-solution. In this scenario protective goggles, respiratory mask with particle filter P3, protective clothing and protective gloves of nitrile rubber are mandatory to wear.

There is no other scenario when dermal irritation could be expected.

Eye irritation

Jerbor, dry formulation, was placed in the hazard category low based on the classification as eye irritating H319 (causes serious eye irritation). The biocidal product is only used by professionals wearing protective googles and exposure to dry granules (concentrated product) with the potential of dusting is only possible in scenario 1 and 2.1. The work is performed according to instructions for use.

The sodium bentonite, as a powder, may cause serious eye irritation.

In scenario 4 bentonite is mixed with the Jerbor/water-solution. In this scenario protective goggles, respiratory mask with particle filter P3 (protective clothing and protective gloves of nitrile rubber are mandatory to wear.

Local risk assessment –Jerbor, granules of boric acid (53.7%) and disodium tetraborate pentahydrate (46.3%)

| Hazard category | Effects in term of C&L | PT | Who is exposed? | Tasks, uses, processes | Potential exposure route | frequency and duration of potential exposure | Relevant RMM & PPE | Conclusion on risk | Uncertainties attached to conclusion may increase (↑) or decrease (↓) risk or both (↑↓) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | Eye irrit. Cat 2, H319 | 8 | Professionals | Scenario 1 Production of Jerbor (mixing disodium tetraborate and boric acid granulas) | Accidental contact: Eye (dust from granules) | 2h/ day, 20 working days/year | The facility has LEV (Local Exhaust Ventilation) Open packages under a ventilated hood. PPE: Protection goggles, Respiratory mask with particle filter P3, Protective clothing, Protective gloves of nitrile rubber | Acceptable: +Reversible effect + Protection googles +LEV +Professionals +Packaging is opened under a ventilated hood +Instructions for use | Adherence to instructions for use may vary (↑↓) Frequency of use may be higher than recommended (↑) |
| Low | Eye irrit. Cat 2, H319 | 8 | Professionals | Scenario 2.1 Loading of mixing tank | Accidental contact: Eye (dust from granules) | 10 min/day, 100 working days/year | PPE: Protection goggles, Respiratory mask with particle filter P3, Protective clothing, Protective gloves of nitrile rubber | Acceptable: +Reversible effect + Protection googles +Professionals + Instructions for use | Adherence to instructions for use may vary (↑↓) Frequency of use may be higher than recommended (↑) |

Local risk assessment – mixing sodium bentonite powder into Jerbor/water-solution

| Hazard category | Effects in term of C&L | PT | Who is exposed? | Tasks, uses, processes | Potential exposure route | frequency and duration of potential exposure | Relevant RMM & PPE | Conclusion on risk | Uncertainties attached to conclusion may increase (↑) or decrease (↓) risk or both (↑↓) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | Notified as eye irritant (H319), skin irritant (H315) and respiratory irritant (H335) | 8 | Professionals | Scenario 4: Mixing barrel - Jerbor water solution mixed with sodium bentonite | Accidental contact: Eye (powder) Dermal (powder) Inhalation (powder) | 50 min/ day, 100 working days/year | PPE: Protection goggles, Respiratory mask with particle filter P3, Protective clothing, Protective gloves of nitrile rubber | Acceptable: +Reversible effect +Wearing of PPE +Professionals + Instructions for use | Adherence to instructions for use may vary (↑↓) Frequency of use may be higher than recommended (↑) Ventilation may vary (↑↓) |

Conclusion on local effects

The qualitative local risk assessment of Jerbor (see Table above) shows that the risk of eye irritation, from dust of granules (scenario 1 & 2.1) when used by professionals is acceptable due to reversible effect and professionals wearing protective goggles and following the instructions for use. In addition, for scenario 1 LEV is used to minimise dusting.

Also, when mixing sodium bentonite into the Jerbor/water solution (scenario 4), the risk of exposure to bentonite dust that could irritate the eyes, skin and respiratory tract could be considered acceptable, as this is only performed by professionals wearing appropriate PPE (protective goggles, respiratory mask with particle filter P3, protective clothing and protective gloves of nitrile rubber) and following the instructions for use.

2.2.6.3.4 Risk for non-professional users

Not relevant since the product is only used by professionals.

2.2.6.3.5 Risk for the general public

Not relevant since only professionals are allowed in the working areas.

2.2.6.3.6 Risk for consumers via residues in food

Not relevant since the biocidal product should not be used in a way that results in dietary exposure.

2.2.6.3.7 Risk characterisation from combined exposure to several active substances or substances of concern within a biocidal product

Not relevant since the biocidal product only contains the two active substances (boric acid and disodium tetraborate pentahydrate) and the risk is calculated for boron and the bentonite is considered inert.

2.2.6.3.8 Summary risk assessment for human health

The risk is acceptable during production, mixing/loading, application, cleaning and maintenance/repair if the following risk mitigating measures are applied:

Production (scenario 1):

The packages with the active substances should be opened under a ventilated hood (LEV).

Use of respiratory protective equipment (RPE) with particle filter P3 is mandatory (filter type (code letter, colour) to be specified by the authorisation holder within the product information).

Wear a protective coverall [type 6, EN 13034].

Wear protective chemical resistant gloves during production (glove material to be specified by the authorisation holder within the product information).

The use of eye protection during production of Jerbor is mandatory.

Mixing/loading (scenario 2-5):

Use of respiratory protective equipment (RPE) with particle filter P3 is mandatory (filter type (code letter, colour) to be specified by the authorisation holder within the product information).

Wear a protective coverall [type 6, EN 13034].

Wear protective chemical resistant gloves during mixing/loading of the product (glove material to be specified by the authorisation holder within the product information).

The use of eye protection during mixing/loading is mandatory.

Application (scenario 6):

Application must be done via a closed system.

Wear a protective coverall [type 6, EN 13034].

Wear protective chemical resistant gloves during application of the product (glove material to be specified by the authorisation holder within the product information).

Cleaning (scenario 7-8):

Wear a protective coverall [type 6, EN 13034].

Wear protective chemical resistant gloves during cleaning of the equipmentequipment (glove material to be specified by the authorisation holder within the product information).

The use of eye protection during cleaning of the equipment is mandatory.

Maintenance/repair (scenario 9):

Wear a protective coverall [type 6, EN 13034].

Wear protective chemical resistant gloves during cleaning of the equipment (glove material to be specified by the authorisation holder within the product information).

The use of eye protection during cleaning of the equipment is mandatory.

2.2.7 Risk assessment for animal health

Not relevant since the treatment with Jerbor will not take place in the vicinity of animals.

2.2.8 Risk assessment for the environment

All naturally occurring forms of boron are oxidised and are present as the boric acid species at the concentrations and acidities typically found in the environment. For practical reasons boric acid and other boron species present in different environmental media are usually expressed on the basis of boron (B).

Boron is a naturally occurring element that is essential to a variety of organisms. Boron concentrations in freshwater systems rarely exceed 1.0 mg B/L and are usually <0.1 mg B/L. A recent review of European monitoring data report ranges of <0.001 to 7.5 mg B/L, the mean 95-percentile concentration over 15 European countries ranges from 0.017 mg B/L in the United Kingdom to 0.632 mg B/L in Germany. Oceans have a natural level of boron of about 5 mg/L.

Boron concentrations in soil are highly variable with ranges between 4 to 200 mg/kg. Boron occurs mostly as an unavailable earth mineral tourmaline and its usual levels are 45-124 mg/kg dwt, based on observations in the United States.

Boron levels in European groundwater range for Italy and Spain from 0.5 to 1.5 mg/L (mean boron concentrations); up to approximately 0.6 mg boron/L in the Netherlands and the United Kingdom, and levels in approximately 90% of samples in Denmark, France, and Germany were found to be below 0.3, 0.3, and 0.1 mg boron/L respectively (all data in the paragraph above are from the final CAR (2008) for boric acid).

Since boron species are inorganic substances, they are not degraded in the environment. Concerning data on their distribution in the environment as well as their effects on biota, the applicant refers to the final CARs (2008) and assessment reports for the relevant boron species.

No new studies regarding the effects on non-target species of the active substances have been submitted with this application. However, data from a number of monitoring (field) studies and updated PEC-modelling regarding the fate and behaviour in the environment of the boric acid have been submitted. They are summarised in section 2.2.8.1.8. Moreover, a long-term monitoring study (Bandh, 2020) has been performed, and is submitted with the Jerbor product application.

Disclaimer: Jerbor with borax pentahydrate is chemically identical with Jerbor with borax decahydrate regarding:

water solution i.e. dissolution in the preparation phase, and

in mixture with the additive, sodium bentonite

The differentiation in water of formation for pentahydrate with 5 H2O compared to 10 H2O for decahydrate do not impact the chemistry of the tetraborate ion other than water content and dilution factor. Regardless, the active substance in application phase is characterized as concentration of boron (B) or boric acid (BAE – Boric Acid Equivalent) depending on the method of analysis and it does not affect the function or efficacy. The active substance concentrations are presented in the report tables in both boron (B) and boric acid (BAE) equivalents.

Jerbor is a fungicide used in remedial/curative in-situ treatment of supporting foundations (wooden piles and grillages) located in dense clay underneath buildings. The product contains the active substances boric acid (orthoboric acid) at 53.7% w/w, and Disodium tetraborate pentahydrate at 46.3% w/w. Jerbor is mixed with water and sodium bentonite to get a treatment solution/slurry which in daily speech is called Eurobor. The treatment solution contains 70.4% water and 20% bentonite. The concentration of the active substances in the final solution of the product is 9.6% BAE (Boric Acid Equivalent) w/w. The estimated dose is between 5 -12 kg BAE/m3 but the applied dose is determined during injection. Normally the applied dose is consistent with the estimated dose for a complete treatment but will differ somewhat within different parts of the foundation.

The treatment solution is applied by injection and is pumped through closed systems into small cavities surrounding the supporting foundations whereupon borates penetrate the wood. The surplus of borates constitutes a depot which maintains the protection of the wood for many years. Highly swelling sodium bentonite regulates the release of borates from the depot and seals the treatment zone (Bandh and Theorin, 2011a). Application is made once. Use of Jerbor is strictly limited to professionals trained according to Manual Euroborbehandling (manual of Eurobor technique, Bandh 2022).

Description of the treatment area and prerequisites for treatment

Jerbor controls decay by wood-rotting fungi in supporting foundations. Only buildings with proper geotechnical conditions may be subject to treatment. Treatment is only performed on foundations surrounded by dense clay both vertically and horizontally. The clay is of marine origin and the thickness of the layer of clay below the building must be at least 2 m but is usually more than 10 m. Naturally deposited clays have in general low permeability and the hydraulic conductivity for such clay is very low. The hydraulic conductivity (also called coefficient of permeability) of a soil or clay is a measure of soil ability to transmit water when submitted to a hydraulic gradient. Also, re-filled clays can be expected to show relatively low permeability but the disturbed structure of such clays generally makes them more permeable than the naturally deposited ones. It can be mentioned that clays are often used as geological barriers under landfills.

Treatment is only done after an investigation has been performed showing that treatment can be done without risks for dispersion/transport of the product from the treatment area to the surrounding environment for example through transport in storm water drainage/systems. There are a number of prerequisites that must be fulfilled in order for the treatment to be effective. The most important among these is, as mentioned above, that the treatment area is submerged in dense clay soils. If there are reasons to suspect that the clay is not naturally deposited, an assessment of the risks of dispersion/transport must be made in order to be able to determine whether a treatment can be made or not. If the soil type and/or the layer of filling material around the shafts made for the foundation are too permeable then treatment will not be performed. Also, the groundwater below the building should be a separate aquifer in order to reduce the risk for dispersion/transport. If the point of water discharge (outlet/drainage system shaft) in connection to the foundation is considered to be too low in relation to the treatment zone (at least 0.5 m above the lower (deeper) end of the foundation wall), sealing measures and other precautions are made before the treatment (Bandh, 2022).

2.2.8.1 Effects assessment on the environment

Background concentration of boron in marine clay and groundwater

background concentration of boron in marine clay from different areas of the world varies considerably and values between 150 to 7400 mg BAE/kg have been reported (review in Theorin, 2011a). Results from recently performed analyses on marine clay from Gothenburg and Uppsala in Sweden showed boron concentrations between 300 and 770 mg BAE/kg (Theorin, 2011a).

The water turnover in a treatment area with marine clay can normally be expected to be low since the clay has low permeability and therefore also low infiltration. Rainwater can also be expected to be diverted in shallow porous soil to nearest drainage structure as cable shafts or surface water/storm water systems. Since the buildings in question are situated in central parts of urban areas they are often surrounded by hard surfaces where the major part of the rainwater is transported to sewers (Vickman and Bandh, 2011).

Due to human activities like leakage of contaminated wastewater and the location of these urban regions in areas under the marine shoreline, the occurrence of boron in groundwaters is considerably higher than in other areas. In order to establish the background concentration of boron in this kind of environment, samples of urban groundwater have been collected at a number of locations in four of the Nordic countries (Vickman and Nilsson, 2011). In Sweden, samples were collected in Gothenburg, Stockholm and Uppsala and the results are displayed in the table below.

Background concentrations of boron in urban groundwater in Sweden

|  | Gothenburg | Stockholm | Uppsala |
| --- | --- | --- | --- |
| Number of samples | 208 | 19 | 2 |
| Mean concentration (mg BAE/l) | 4.9 | 26.7 | 3.1 |
| Standard deviation | 5.8 | 35.1 | 0.5 |
| Min concentration (mg BAE/l) | <0.1 | 1.2 | 2.7 |
| Max concentration (mg BAE/l) | 36 | 130 | 3.4 |

Based on these measurements, the background level of boron for urban groundwater of 5 mg BAE/L was applied (based on urban background levels in groundwater Vickman & Nilsson, 2011) in the PEC modelling calculations.

PNECs

In the final CARs for boric acid and disodium tetraborate the Reference Member State (RMS) conducts the risk assessment based on added concentrations. Following the argumentation in the final CARs for all boron species, the PNECadd is used in the risk characterisation for Jerbor (please refer to section 2.2.8.3 for the risk characterisation).

The PNECadd should be regarded as the concentration that may be added to the background without inducing any adverse effects on the ecosystem. It is assumed that the background concentration itself does not induce a toxic effect and that the background concentration is sufficient to meet the natural demands of the organisms and that there is no risk for deficiency.

Terrestrial compartment

The PNECadd,terrestrial is 0.4 mg B/kg dwt soil, which is equivalent to 0.35 mg B/kg wwt soil (Assessment Report Boric acid, 2009, and Assessment Report Disodium tetraborate, 2009). The PNECadd,terristrial is estimated based on data from the most sensitive species which is a plant (Hordeum vulgare) (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009).

Aquatic compartment

The PNECadd,aquatic for freshwater systems is 0.18 mg B/L which is based on the lowest NOEC of 1.8 mg B/L and an assessment factor of 10. The PNECadd,sediment is adjusted to 0.24 mg B/kg wwt using equilibrium partitioning.

The PNECadd, STP is 1.8 mg B/L which is based on a NOEC of 17.5 mg B/L from a microbial inhibition test with microorganisms in activated sludge and an assessment factor of 10 (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009).

Ground water

The limit value for boron in drinking water is 1 mg B/L according to Directive 98/83/EC Annex I, part B (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009).

Atmosphere

No toxicity data for air are available (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009). Therefore, a PNECair has not been calculated.

Primary poisoning

The toxicity values for birds and mammals are presented in the table below.

| Acute and reproductive toxicity values for birds and mammals exposed to borates. |  |  |  |
| --- | --- | --- | --- |
| Active substance | Type of endpoint | Endpoint | Reference |
| Boric acid | Acute toxicity to mammals (rat) | Oral LD50 = 3450 mg/kg bw Oral LD50 > 2500 mg/kg bw | Assessment Report Boric acid, 2009 |
|  | Acute toxicity to birds (Colinus virginianus) | LD50 > 527 mg B/kg bw | Assessment Report Boric acid, 2009 |
|  | Reproductive toxicity to mammals (lowest relevant developmental) | NOAEL/LOAEL = 9.6 mg boron/kg bw/day | Assessment Report Boric acid, 2009 |
|  | Reproductive toxicity to birds | no data available | Assessment Report Boric acid, 2009 |
| Disodium tetraborate pentahydrate | Acute toxicity to mammals (rat) | Oral LD50 > 2000 mg/kg bw | Assessment Report Disodium tetraborate, 2009 |
|  | Acute toxicity to birds (Colinus virginianus) | LD50 > 527 mg B/kg bw | Assessment Report Disodium tetraborate, 2009 |
|  | Reproductive toxicity to mammals (lowest relevant developmental) | NOAEL/LOAEL = 9.6 mg boron/kg bw/day | Assessment Report Disodium tetraborate, 2009 |
|  | Reproductive toxicity to birds | no data available | Assessment Report Disodium tetraborate, 2009 |

Secondary poisoning

Boron is not bioconcentrated or bioaccumulated along the food chain in neither aquatic nor terrestrial ecosystems (Assessment Report Boric acid, 2009 and Assessment Report Disodium tetraborate, 2009).

Summary of PNECs of the active substances

The PNECadd values considered relevant to use in the risk characterisation are presented in the table below.

| Summary table of PNEC values for the active substances |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Active substance | PNECadd, terrestrial | PNECadd, aquatic | PNECadd, sediment | PNECadd, STP | PNECgw (drinking water limit) |
| Boric acid and Disodium tetraborate pentahydrate | 0.4 mg B/kg dw (2.29 mg BAE/kg dw)* | 0.18 mg B/L (1.03 mg BAE/L)* | 0.24 mg B/kg wwt (1.37 mg BAE/kg ww)* | 1.8 mg B/L (10.3 mg BAE/L)* | 1 mg B/L (5.72 mg BAE/L)* |

*The values in brackets are recalculated to boric acid equivalents (BAE) using the factor of 5.72.

2.2.8.1.1 Information relating to the ecotoxicity of the biocidal product which is sufficient to enable a decision to be made concerning the classification of the product

No environmental classification of the active substances is needed (please refer to Annex VI of Regulation (EC) No 1272/2008), and no other constituents in the product have an influence on the environmental classification and labelling of the product. The conclusion of the environmental classification of the product is presented below.

Environmental classification and labelling of the product

Classification: none Labelling: none

2.2.8.1.2 Further Ecotoxicological studies

No further ecotoxicological studies are required.

2.2.8.1.3 Effects on any other specific, non-target organisms (flora and fauna) believed to be at risk (ADS)

No additional data are required. The data in the CARs for boric acid and disodium tetraborate pentahydrate are sufficient.

2.2.8.1.4 Supervised trials to assess risks to non-target organisms under field conditions

Exposure of the product to non-target organisms is unlikely due to the following reasons:

The treatment solution is applied by injection and is pumped through closed systems into the ground.

If an accident occurs and there is a spill, exposure to non-target organisms will be stopped by:

sweeping /shovelling up large spills alternatively using an industrial vacuum cleaner if the spill is in the form of granules.

pumping off product or picking it up with suitable absorbent material if the spill is in the form of a water-based slurry.

Trials to assess risks to non-target organisms under field conditions are therefore not considered to be required. The data in the CARs for boric acid and disodium tetraborate pentahydrate are sufficient.

2.2.8.1.5 Studies on acceptance by ingestion of the biocidal product by any non-target organisms thought to be at risk

Exposure of the product to non-target organisms is unlikely due to the reasons mentioned in 2.2.8.1.4.

Studies on acceptance by ingestion of the product by non-target organisms are therefore not considered to be required. The data in the CARs for boric acid and disodium tetraborate pentahydrate are sufficient.

2.2.8.1.6 Secondary ecological effect e.g. when a large proportion of a specific habitat type is treated (ADS)

No additional data are required. The data in the CARs for boric acid and disodium tetraborate pentahydrate are sufficient.

2.2.8.1.7 Foreseeable routes of entry into the environment on the basis of the use envisaged

The following environmental compartments have been identified as relevant for the exposure assessment (Vickman, 2011):

Soil (clay)

Groundwater

Boron in the groundwater could also be transported to surface water and STP via drainage structures. However, exposure of these compartments is not very likely since treatment is not allowed near surface water if there is a risk for leakage. Moreover, mending of any broken parts of the sewage system is mandatory before treatment according to the instructions for use (Bandh, 2022).

Air is not considered to be a relevant environmental compartment since the release of boron to the air will be minimal because of its low vapour pressure according to the CARs for boric acid (Assessment Report Boric acid, 2009) and disodium tetraborate pentahydrate (Assessment Report Disodium tetraborate, 2009).

2.2.8.1.8 Further studies on fate and behaviour in the environment

- 2.2.8.1.8.1 Monitoring of variation in groundwater levels in dry wall foundations (Bandh, 2012a)

The purpose of the study by Bandh (2012a) was to show the magnitude of variation in groundwater levels in dry wall foundations. Fifteen buildings were sampled at random occasions not related to rainfall. The results showed that a rise of the groundwater level (relative to a mean value) of > 20 cm did not occur during any of the observations made which were 228 in total.

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | The variation in groundwater level is small in dry wall foundations. |

- 2.2.8.1.8.2 Monitoring studies of the environmental distribution and exposure of ground water and soil related to treatment with Jerbor (Vickman and Danielsson, 2011, supplemented by Bandh, 2012b and 2020)

A monitoring study of the environmental distribution and exposure related to treatment with Eurobor in urban Gothenburg were performed by Vickman and Danielsson (2011) and supplemented by Bandh (2012b) and (2020). Three buildings (on Kobbarnas väg 2 to 8), which were built during 1935 to 1937, were part of the study. The foundation of the buildings consists of concrete walls and a foundation bedding (plate) resting on embedded wooden piles. In order to monitor the environmental exposure during and after treatment with Eurobor, groundwater tubes were installed at different locations inside the buildings by drilling through the cellar or installed in test pits. These tubes were both used for Eurobor application and groundwater sampling in the treatment zone to ensure a sufficiently high concentration of boron. Since the depth of the clay within the properties was over 40 meters (according to the City Planning Office of the City of Gothenburg's geographic map), the greatest risk of undesirable spread of the borates would therefore be a horizontal transfer in the groundwater above the clay layer via shafts where the sewers leave the buildings. In the treated buildings, the sewage communications go out into the Kobbarnas väg where they connect to a larger pipeline that runs along the street on to the municipal treatment plant. To monitor a possible leakage of borates via this route, six control tubes for groundwater sampling were installed prior to treatment 6-7 m outside the western and eastern sides of the buildings. The groundwater tubes west of the buildings were installed in the shafts for sewers in the Kobbarnas väg.

Water samples were taken from the tubes prior-, during- and after the treatment. The concentration of boron in the groundwater measured prior to treatment varied between <1 and 2 mg BAE/L outside the buildings, and between <1 and 8.3 mg BAE/L (and a mean value of 3 mg BAE/L) in the prospective treatment zone under the buildings. The treatment period lasted approximately a month during which injections of Eurobor were done on 15 occasions. Totally 11400 litres of Eurobor were injected (approx. 1000 kg Jerbor) equally split between the three properties in 2011. During the four-weeks treatment period, on 2 out of 9 sampling occasions the groundwater samples showed somewhat elevated levels of boron, compared to the levels measured before treatment. Although slightly elevated, the levels (of max. 4.2 and 13 mg BAE/L) were however not different from what has been found as being representative for urban groundwater in Gothenburg which ranges between <1.0 and 36 mg BAE/L with a mean of 4.9 mg BAE/L (Vickman & Nilsson, 2011). The authors of the report did not believe that the slightly elevated levels were related to the treatment since the groundwater concentrations in the same tube at seven other sampling occasions during the treatment were <1.0 mg BAE/L.

Sampling in the groundwater tubes outside the buildings four and twelve weeks after treatment did not show any elevated levels of boron in the groundwater (all samples <1.0 mg BAE/L). Additional measurements have been made (after finalisation of the report by Vickman and Danielsson, 2011) on two occasions: January 2012 and September 2012 equivalent to 8 and 16 months respectively, after the treatment. The results showed that the concentration in the groundwater at the locations outside the buildings was still below the background concentration on site < 3 mg BAE/L (Bandh, 2012b).

An additional point treatment of BAE was performed for the Kobbarnas väg 4 in the result of the 5-year warranty control inspection in 2016 (Bandh, 2020). Three years later (and 8 years after the original treatment in 2011) a long-term follow-up of boron levels in the groundwater from the control tubes outside the building was done in July 2019 (Bandh, 2020). The samples taken from 3 of the 6 original tubes showed boron variation from <1 to 2.3 mg BAE/L (mean <1.6 mg BAE/L) which is within the limit of the 5 mg BAE/L background level of boron (based on urban background levels in groundwater Vickman & Nilsson, 2011).

Sampling in the four groundwater tubes located inside the building, but not in direct contact with the treatment zones, showed that the groundwater in one of the tubes, very close to the treatment zone, was somewhat elevated four days after injection (58 mg BAE/L). The other three tubes did not show any markedly elevated groundwater levels during or after treatment. Still, 8 and 16 months after treatment the levels sampled in these tubes were not elevated compared to boron levels measured prior to treatment.

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | The treatment with Jerbor does not result in any significant environmental exposure. The Jerbor injected under the foundation of the buildings remains within the treatment area and the transport/dispersal from the area is limited |

- 2.2.8.1.8.3 Literature review on dispersal of boron to the surrounding groundwater (Vickman and Nilsson, 2011)

A literature review was performed by Vickman and Nilsson (2011) with the aim of clarifying whether Eurobor treatments cause dispersal of boron to the surrounding groundwater. The report summarizes seven field studies; four studies performed during 2002-2009 and three studies conducted in 2011.

The result from the four studies conducted between 2002-2009 showed no transport of borates to the environment outside the treatment zone. There were high levels of borates in direct connection to the treatment area, but the levels were significantly lower in the groundwater a short distance from the treatment zone.

One of the studies (Humlegårdsgatan 17, Åbo, Finland) reports elevated levels of boron 1-4 years after the treatment in 2005 in one of the groundwater sampling tubes installed outside the building in the drain shaft just behind a bentonite seal created before the treatment to reduce the spread of the boron (<0.57 mg BAE/L before the treatment vs. the maximum value of 13.2 mg BAE/L 2 years after the treatment). This isolated case emphases the importance of a proper sealing of low-lying openings of drain shafts.

Elevated boron levels (19-44 mg B/L or 103-252 mg BAE/L) in the groundwater samples outside the building in another study (Sten Sturegatan 10, Göteborg, 2011) was subsequently interpreted that the groundwater tubes were installed in connection to the treatment zone (i.e., the foundation shaft for the house).

The result from three studies performed in 2011 showed high levels of boron in the treatment tubes and close to the treatment tubes. There were also elevated levels in the observation tubes a short distance from the treatment tubes. This was explained by the fact that all tubes (treatment and observation tubes) were located within the treatment zone. The detected levels were significantly lower in the observation tubes in comparison with the treatment tubes.

The study also set out to establish the background concentration of boron in urban environments by sampling urban groundwater at several locations in four of the Nordic countries. Sampling in areas relevant for treatment with Jerbor (but far from treated buildings) in Gothenburg, Uppsala, Copenhagen, Stockholm, Oslo, Åbo and Helsinki) showed mean concentrations of 3.0 – 26.7 mg BAE/L (0.5 – 4.6 mg B/L).

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | The treatment with Jerbor does not result in any significant environmental exposure. The Jerbor injected under the foundation of the buildings remains within the treatment area and the transport/dispersal from the area is limited. |

- 2.2.8.1.8.4 Monitoring studies of groundwater and clay related to Jerbor treatment (Theorin, 2011b)

The aim of this study by Theorin (2011b) was to investigate what concentrations of boron remain in samples taken from foundations which have been treated with Jerbor in the past. Groundwater and clay were sampled underneath a number of buildings and analysed for boric acid. The results showed that 19 - 89% of the applied concentration still remained in the treatment zone several years after treatment. Groundwater in direct connection to the Eurobor slurry in the treatment tubes contained significant amounts of borates. However, groundwater and clay at the outer edges of the treatment zone only contained concentrations in the same range as the background concentration of borates in clay from the same area (i.e., Gothenburg and Uppsala in areas relevant for treatment with Jerbor but far from treated buildings).

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | Clay located at the outer edges of an area which has been treated with Jerbor shows boron concentration in the same range as the background concentration of boron in clay from the same area. |

- 2.2.8.1.8.5 Monitoring studies of sediment and clay related to Jerbor treatment (Theorin, 2011c)

The purpose of this study, performed by Theorin (2011c), was to supplement the data on the concentration of boric acid and borax in sediment and clay, and to show what the typical background concentration is in clay of marine origin sampled in areas in which treatment with Jerbor is commonly made. Sampling of the clay was done underneath existing buildings in central parts of Gothenburg and Uppsala in spring 2011. The results of the study coincide with other published data of boron concentrations in clay of marine origin.

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | The background concentration of boron in clay, sampled in areas where treatment with Jerbor is commonly made, is in the same range as background concentrations of boron in clay from other areas of the world. |

- 2.2.8.1.8.6 Literature review on borates in marine clays (Theorin, 2011a)

The study is a literature review which compiles the data on borates in clay that is available from published studies. The study describes the formation and occurrence of marine clays, the amount of boron in clay of marine origin, and the intrinsic properties of borates such as the rate of diffusion and adsorption of boron in clay.

| Conclusion used in Risk Assessment |  |
| --- | --- |
| Conclusion | The background concentration of boron in marine clays from different areas of the world varies considerably and values between 25 and 1300 mg B/kg have been reported. |

2.2.8.1.9 Leaching behaviour

No additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.1.10 Testing for distribution and dissipation in soil

No additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.1.11 Testing for distribution and dissipation in water and sediment

No additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.1.12 Testing for distribution and dissipation in air

No additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.1.13 If the biocidal product is to be sprayed near to surface waters then an overspray study may be required to assess risks to aquatic organisms or plants under field conditions

The product is not sprayed. Thus, no additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.1.14 If the biocidal product is to be sprayed outside or if potential for large scale formation of dust is given then data on overspray behaviour may be required to assess risks to bees and non-target arthropods under field conditions (ADS)

No additional data is required. The data in the CARs for boric acid and disodium tetraborate pentahydrate is sufficient.

2.2.8.2 Exposure assessment

The routes of environmental exposure as foreseen by the applicant are illustrated in the figure below.

Routes of environmental exposure. From top to bottom:

Injektering = Eurobor injection;

Behandlingszon = Treatment zone;

Grundvatten = Groundwater;

Jord = Soil;

Ledningsschakt/Dränering = Discharge/Drainage structure (shafts);

Ytvatten = Surface water;

Spillvattenledning = Wastewater pipes;

Reningsverk = Wastewater/sewage treatment plant.

The picture is taken from Vickman (2011b).

The environmental compartments that are considered to be relevant for exposure estimations are thus: the soil/clay, groundwater, surface water and a sewage treatment plant (STP).

The most important route for transport of boron from the treatment zone to other areas and compartments is via the groundwater (Vickman, 2011). If the inflow of water is larger than what can be diverted in the clay downwards and sideways (horizontally), the level of the groundwater will rise in the filling layer/soil and eventually overflow at the lowest point which is usually represented by a drainage structure (drainage shaft) for outgoing discharge and these have in most cases been constructed above (i.e. at a higher level than) the wooden foundations in order for the groundwater to cover the wood for it to be protected. In the drainage system it can be expected that the transport is faster and that a faster dilution of the boron species occurs. The level of the groundwater is however, over a longer time perspective, usually below the threshold level since the use area of Jerbor is against wood-decaying fungi which requires oxygen, i.e. if the wooden foundation was submerged in water, the oxygen access would be very limited and the fungi would not grow.

Boron species can also penetrate the surrounding clay by diffusion and by the very limited water transport within the clay. These processes can result in an increased concentration of boron in the clay, both in the porewater and adsorbed to the clay particles. If the boron is transported in the groundwater via drainage structures as described above, also adsorption to the soil of fill can occur.

The environment in which Jerbor is used is very specific and different from environments usually exposed to wood preservatives (Vickman and Bandh, 2011). Jerbor is applied by injection and is pumped through closed systems into small cavities surrounding foundation wood. Only foundations surrounded by dense clay are treated (Bandh, 2022). The thickness of the clay below the building is at least 2 m but usually > 10 m.

Three different approaches have been used in order to estimate the boron concentration added by the treatment of Jerbor in different compartments:

- mathematical modelling,

- analytical calculations and

- direct measurements (see 2.2.8.1.8).

Mathematical modelling and analytical calculations

The groundwater flow has been estimated with MODFLOW-FLEX since the models (e.g. PEARL and PELMO) suggested in the technical guidance document on the Biocidal Products Regulation (ECHA, 2017) and in the revised ESD for wood preservatives (OECD, 2013) are not suitable to predict the transport of inorganic substances below the groundwater table. The reasons for using Modflow are explained in Joodaki & Vickman (2020).

MODFLOW is a modelling program developed by the U.S. Geological Survey. It has been used with the graphical user interface Visual MODFLOW FLEX (Schlumberger Water Services, 2006), and the belonging module MT3D (Zheng and Wang, 1999) (please refer to Joodaki & Vickman, 2020 for more information). The modelling procedure is described in a report by Joodaki & Vickman (2020), Vickman (2011), (and supplemented by Bermin, 2012) together with descriptions and explanations of the model structure, equations used, selection in input parameters, model cases and dimensions, terms and conditions, calibration, sensitivity analysis, etc. It can be mentioned that when selecting input data for the different parameters generally worst-case values were selected in order to make the calculations conservative enough.

The modelling has also been supplemented with calculations performed in a tool developed by the Swedish Protection Agency, the Swedish Geotechnical Institute, Kemakta konsult AB and the Institute of Environmental Medicine at the Karolinska Institute to derive guideline values for contaminated land (Naturvårdsverket, 2009a) (please refer to Bermin 2012 for more information). The content of boron in clay has been calculated based on concentrations in water and the Langmuir sorption isotherm for clay soils (please refer to Joodaki & Vickman, 2020 for more information).

Initially the dispersion of borates in the treatment zone at boundary layer between the construction foundation and the ground water table was modelled using a time- and space-resolved model in MODFLOW software package (see Vickman 2011). Vickman assessed 6 different scenarios (1B, 2B, 3B, 1K, 2K and 3K) where the B-scenarios handled the dispersion dynamics of the treatment zone of a building with foundation wall of concrete and the K-scenarios handled the treatment zone of a building with stone foundation.

However, pursuant to the application of renewal of the biocidal product application in 2020, the MODFLOW-version of 2011 (Visual MODFLOW Classic) were not supported by the software supplier anymore and the more updated versions of the MODFLOW software, re-placing Visual MODFLOW Classic, does not use exactly the same parameterization set as the 2011 version.

It was thus decided to re-build the scenario modelling using a version of MODFLOW (MODFLOW FLEX) offering even better performance on the time- and space-resolved simulations by the M3TD module.

When re-assessing the scenarios with the updated software, it was revealed that three (3) scenarios sufficiently describe the dispersion of borates in the treatment zone as a function of time, for both the B- and K-scenarios. For instance, the vertical dispersion and transport velocity of borates in scenarios 2K, 3K and 3B of Vickman 2011, is fully covered by the simulation of scenario 2B which is considered the worst-case simulation since the groundwater table is assessed to be above the drainage point in Scenario 2B.

The scenarios and the model used in the exposure assessment are presented below.

2.2.8.2.1 General information

| Assessed PT | PT 8 |
| --- | --- |
| Assessed scenarios | Building with foundation wall of concrete (initial treatment concentration is 89 500 mg BAE/L, treatment zone is 1 cm thick): Scenario 1B - Dispersion of boron with shallow groundwater which is transported upwards through the filling material around the building to the drainage shaft and eventually ends up in surface water, sediment and STP. - The groundwater level is above the discharge point to the drainage system Scenario 2B - Vertical and horizontal dispersion of boron to the deeper underlying layers of clay. - The groundwater level is above the discharge point to the drainage system - This is the worst case (the highest boron treatment concentration) – it covers boron dispersion in clay for the stone foundation scenario (K) The scenarios (1B-2B) with a foundation wall of concrete are considered to be the standard case. Building with stone foundation (initial treatment concentration – 19 200 mg BAE/L, treatment zone is 14 cm thick): Scenario 1K - Dispersion of boron to shallow groundwater which is transported upwards through the filling material around the building and crevices in the wall foundation to the drainage shaft and eventually ends up in surface water, sediment and STP. - Groundwater level is above the discharge point to the drainage system |
| ESD(s) used | Revised ESD for wood preservatives (OECD, 2013) Use class: 4A wood in contact with the ground |
| Distribution in the environment | Calculated based on TGD 2003 and on measured data |
| Groundwater simulation | MODFLOW with the graphical user interface Visual MODFLOW FLEX and the belonging module MT3D |
| Confidential Annexes | NO |
| Life cycle steps assessed | All scenarios: Production: No Formulation: No Use: Yes Service life: Yes |

The case with a foundation wall of concrete is considered to be the standard case and a schematic picture of a typical building treated by Eurobor can be seen in the figures below and this concept is used in the modelling.

Sectional view - the standard case for a building (HUS) that is submerged in naturally deposited clay (Le) with clay filling material closest to the walls of the building (F) and a thin gritty layer directly under the building into which the Eurobor is injected (red colour). The building is built on foundation walls of concrete placed on wooden piles (the piles are not marked in the schematic picture). The distances shown on the picture represent the thickness (height) of each geological unit.

F = filling material;

Le = naturally deposited clay;

Markyta = ground surface;

Avloppsschakt = discharge/drainage system (shaft) which is the lowest level for outflow of groundwater; Friktionsmaterial = frictional material

The picture is taken from Joodaki & Vickman (2020).

Top view - dimensions of a normal/standard building that is treated with Eurobor. The shaded grey areas are hard surfaces (asphalt) and the area marked in a dark brown colour shows the foundation of the building equal to the treatment zone.

From top to bottom: till närmaste dränstråk = to closest distance to a drainage area, i.e. possibility for water infiltration; lägsta utgående ledningsschakt = lowest point for discharge (drainage system shaft)

The picture is taken from Vickman (2011).

Calculations have been made on a building with a foundation under the outer walls and in two lanes along the house. A few meters (6 m in the northern and western directions and 10 m in the southern and eastern directions) from the building there is the nearest draining lane such as a drainage pipe or pipe shaft. West and north of the building, the ground is asphalted, while the rest of the ground mostly consists of grass or gravel surfaces. The ground surface slopes from south to north and in the northern part of the building the lowest drainage shaft goes out towards the street.

Among the input parameters and data can be mentioned (see details in 2.2.8.2.2):

- The geometry for a standard building (see figure below)

- Hydraulic conductivity

- Porosity

- Groundwater formation

- Coefficient of diffusion

- Isotherm for adsorption

- Added amounts of boron

- Background concentration of boron in groundwater

And the output data are:

- Groundwater flow (also input data for further calculations)

- Concentrations in groundwater (also input data for further calculations)

- Diffusion in clay

- Concentrations in clay

- Amounts transported to surface water, sediment and STP

The physical and chemical properties of boric acid are presented in the table below. Since boron is a naturally occurring element, many of the parameters are not relevant for use in the exposure assessment.

| Physical and chemical properties of boric acid |  |  |  |
| --- | --- | --- | --- |
| Input | Value | Unit | Remarks |
| Melting point | No melting point can be defined | °C | Assessment Report Boric acid, 2009 |
| Boiling point | No boiling point can be defined | °C | Assessment Report Boric acid, 2009 |
| Vapour pressure | Not applicable | Pa | Assessment Report Boric acid, 2009 |
| Water solubility | 36.24 ± 1.03 g/L at 10 °C (pH of the test solution 3.91 - 3.95) | mg/l | Assessment Report Boric acid, 2009 |
| Log Octanol/water partition coefficient | -1.09 at 22°C and pH 7.5 in potassium/sodium phosphate buffer at a concentration of 0.0097 M boron | Log 10 | Assessment Report Boric acid, 2009 |
| Organic carbon/water partition coefficient (Koc) | Average KF 2.6 L/kg | l/kg | Assessment Report Boric acid, 2009 |
| Henry’s Law Constant | Not applicable | Pa/m3/mol | Assessment Report Boric acid, 2009 |
| Biodegradability | No |  | Assessment Report Boric acid, 2009 |
| DT50 for biodegradation in surface water | Not applicable | d or hr (at 12ºC) | Assessment Report Boric acid, 2009 |
| DT50 for hydrolysis in surface water | Hydrolytically stable | d or hr (at 12ºC /pH) | Assessment Report Boric acid, 2009 |
| DT50 for photolysis in surface water | Photolytically stable | d or hr | Assessment Report Boric acid, 2009 |
| DT50 for degradation in soil | Not applicable | d or hr (at 12ºC) | Assessment Report Boric acid, 2009 |
| DT50 for degradation in air | Not applicable | d or hr | Assessment Report Boric acid, 2009 |

2.2.8.2.2 Emission estimation

The modelling procedure together with descriptions of the model, equations used, selection of input parameters, scenarios, terms and conditions, calibration, sensitivity analysis etc are described by Joodaki & Vickman (2020) and supplemented by Bermin (2012). In the table below, the input parameters for calculating the fate and distribution in the environment of borates are presented. When choosing input data for the different parameters, generally worst-case values were selected in order to make the calculations sufficiently conservative.

| Input parameters for calculating the fate and distribution in the environment of borates |  |  |  |
| --- | --- | --- | --- |
| Input | Value | Unit | Remarks |
| Application rate (IC, initial concentration) | 89 500 mg BAE/L (1B, 2B) 19 200 mg BAE/L (1K) | mg BAE/L | Before treatment, the estimated dose is set to between 5-12 kg BAE/m3. However, the final dose (i.e the applied dose) is determined during treatment. Normally the applied dose is consistent with the estimated dose for a complete treatment but will differ some within different parts of the foundation. |
| Background concentration of borates in groundwater | 5 mg BAE/L | mg BAE/L | Based on urban background levels in groundwater (Vickman & Nilsson, 2011). |
| RC (recharge concentration) | 5 mg BAE/L | mg BAE/L | Concentration in the precipitation added to the model |
| Groundwater formation | 200 mm/year | mm/year | Based on literature data for groundwater formation in different types of Swedish soils (Rodhe et al., 2006). |
| Dry density of soil | 1600 kg/m3 | kg/m3 | Based on literature data on the grain density of most soils (USDA, 2008; Larsson, 2008). |
| Molecular diffusion for boron | 5 × 10-10 m2/s | m2/s | No value of molecular diffusion of boron has been found in literature. The value of 5 × 10-10 m2/s which is used in the modelling corresponds to the diffusion of water in clay (Couch et al., 1968). This value overestimates the diffusion. |
| Horizontal hydraulic conductivity (Kxy) | 1 × 10-10 - 1 × 10-4 m/s | m/s | Based on results from sludge tests (Vickman & Valtersson, 2011), literature data (Fetter, 2001; Kresic, 2007) and an experience-based assessment. The interval refers to the lowest and highest hydraulic conductivity value in different layers of the soil. |
| Vertical hydraulic conductivity (Kz) | 1 × 10-10 - 5 × 10-4 m/s | m/s | Based on results from sludge tests (Vickman & Valtersson, 2011), literature data (Fetter, 2001; Kresic, 2007) and an experience-based assessment. The interval refers to the lowest and highest hydraulic conductivity value in different layers of the soil. |
| Adsorption maximum (Smax) | 500 mg BAE/kg | mg BAE/kg | The adsorption is slightly more than a factor of 2 lower than reported (Couch et al., 1968). Thus, the adsorption is underestimated, which gives an overestimation of the dispersion. |
| Langmuir constant (KL) | 0.153 | L/mg BAE | The Langmuir constant of 0.153 applies at pH 9.3, which is approximately the pH that will be achieved during a treatment, and to soils with a low content of clay (Communar et al., 2004). The constant is expected to be greater (up to 0.9) in soils where the content of clay is higher. |
| Total porosity (n) | 0.25-0.5 | share of total volume | Based on literature data (Naturvårdsverket, 2009a) and an experience-based assessment. The interval refers to the lowest and highest porosity value in different layers of the soil. |
| Effective porosity (ne) | 0.05-0.2 | share of total volume | Based on literature data (Naturvårdsverket, 2009a) and an experience-based assessment. The interval refers to the lowest and highest porosity value in different layers of the soil. |

The basic structure of the model, with properties and geometries, is the same for all scenarios, and in all current calculation cases (1K, 1B and 2B) the groundwater surface is above the drainage level in the outgoing sewage shaft. However, a few scenario-specific adjustments have been made:

K-scenario with stone foundation

- Please note that in the 1K-scenario the stone wall contains cavities between the stones which will result in a faster dispersion of borates from the treatment zone vertically upwards compared to the horizontal dispersion in the filling material. The stone wall was assumed to have a hydraulic conductivity of 1×10-3 m/s and an effective porosity of 15% (ne = 0.15).

- The initial treatment zone concentration of scenario 1K was set to 19 200 mg BAE/L in a 14 cm thick treatment zone just beneath the stone wall. The 14 cm thick layer consists of permeable material where cavities are filled with Eurobor. The assumptions are based on field experiences from applying/injecting the Eurobor mixture in amounts ca 3 times the applied dose compared to injections under concrete foundations.

- Transport of borates to the drainage discharge point was primarily calculated based on a dynamic dispersivity of 1.5 m (this is the distance between the treatment zone and the drainage point used in the calculations of borates transport and the resulting borate concentrations).

B-scenarios with foundation wall of concrete

- Scenarios 1B and 2B, the concrete foundation, has been assumed to have zero permeability to borates (no cracks nor cavities).

- The initial treatment zone concentrations of borates were set to 89 500 mg BAE/L in a 1 cm thick treatment zone.  These assumptions are also based on expert knowledge on the field conditions prevailing when injecting Eurobor, normally there is ca a 1 cm cavity/void between the concrete foundation and the underpinning clay which is completely filled with the Eurobor paste.

- Transport of borates to the drainage discharge point was primarily calculated for 1B based on a dynamic dispersivity of 1.5 m (this is the distance between the treatment zone and the drainage point used in the calculations of borates transport and the resulting borate concentrations). However, in Scenario 1B a dynamic dispersivity of 10 m was used thus calculations of borates dispersion through the filling material reflect a very conservative estimate.

- Lastly, in the simulation model for calculating the downward vertical transport of borates into the clay layers, the vertical dynamic dispersity was set to 0.2 m while increasing model resolution in the upper decimetre of clay under the building body by increasing the number of simulation cells and thus making each cell thinner.

- Scenario 2B is considered to be the worst case (due to high water pressure in combination with the highest initial concentration), therefore scenario 1K does not include calculations of dispersion in clay.

- Corresponding background concentrations of borates in clay were calculated from the assumed background concentration in the groundwater (C = 5 mg BAE/L) and the Langmuirs adsorption isotherm for borates (see the report by Joodaki & Vickman, 2020 for references):

Cads=Kl×Sads×C1+Kl×C

Where Cads – adsorbed concentration (mg/kg)

Kl – adsorption isotherm (L/mg)

Sads — adsorption isotherm constant

C – concentration in solution (groundwater) (mg/L)

It has been shown that adsorption to clay in the conditions as relevant to Eurobor treatment as possible, typical values of the adsorption isotherm was chosen to Kl = 0.153 L/mg and the maximum adsorption capacity to Sads = 500 mg BAE/kg (Comnunar & Keren 2004). The background concentration in clay has thus been calculated to be 217 mg BAE/kg.

2.2.8.2.3 Fate and distribution in exposed environmental compartments

Once borates are released to the environment, the predominant species is undissociated boric acid (Assessment Report Boric acid, 2009). The most important route for transport of boron from the treatment zone to other areas and compartments is via the groundwater (Vickman, 2011 as well as Joodaki and Vickman 2020). Boron can also penetrate into the surrounding clay by diffusion and by the very limited water transport within the clay. These processes can result in an increased concentration of boron in the clay, both in the porewater and adsorbed to the clay particles.

Boron in the groundwater could also be transported to surface water, seawater and STP via drainage structures. However, exposure of these compartments is not very likely since treatment is not allowed near surface water if there is a risk for leakage. Moreover, mending of any broken parts of the sewage system is mandatory before treatment according to the instructions for use (Bandh, 2022).

Air is not considered to be a relevant environmental compartment since the release of boron to the air will be minimal because of its low vapour pressure according to the CARs for boric acid (Assessment Report Boric acid, 2009) and disodium tetraborates (Assessment Report Disodium tetraborate, 2009). Thus, the environmental compartments that are considered to be relevant for exposure estimations are soil and groundwater. In the table below, the relevant receiving compartments based on the exposure pathways are presented.

|  | Fresh-water | Freshwater sediment | Sea-water | Seawater sediment | STP | Air | Soil | Ground- water |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| All six scenarios | NO | NO | NO | NO | NO | NOT RELEVANT | YES | YES |

2.2.8.2.4 Calculated PEC values

In the final CARs for boric acid and disodium tetraborates, the RMS conducted the risk assessment based on added concentrations. Following the argumentation in the final CARs for all boron species, the PECadd is used in the risk characterisation for Jerbor (please refer to section 2.2.8.3 for more information). The PECadd should be regarded as the concentration that are added to the background concentration.

- 2.2.8.2.4.1 PEC groundwater

The results of the groundwater modelling are presented in the table below. The PEC represents the concentration added to the groundwater as a result of treatment with Jerbor. The groundwater modelling estimates the mean concentration of boron within 50 cm from the treatment zone vertically and horizontally (the same approach as in the method described in OECD, 2013). Please refer to Annex 3.3 for screen shots of the input and output values of the simulations.

| Summary table on simulated PECgw values |  |
| --- | --- |
| Scenario | PECadd, gw |
|  | [mg B/L] |
| Scenario 1B (foundation wall of concrete) – boron vertical and horizontal dispersal with groundwater in the filling material to the point of discharge/drainage (0- 30 years). The ground water table is assumed to be above the point of drainage. This scenario is expected to result in a lower dispersion of borates to the drainage point than the corresponding scenario of the treatment zone under the stone foundation (1K) due to the fact that dispersion and transport in Scenario 1B occurs only through the filling material around the building and not via the permeable stone foundation. Since the treatment scenario of the concrete foundation wall is by far the most common treatment case it was considered important to assess that case. | 0 |
| Scenario 2B - boron horizontal dispersal with groundwater in clay 0-50 cm from the treatment zone (10 days, the dispersal front has reached <6 cm) | 15.7 mg B/L (89.8 mg BAE/L) |
| Scenario 2B – boron horizontal dispersal with groundwater in clay 6-50 cm from the treatment zone (after 30 years). After 30 years boron horizontal dispersal front has reached 6 cm from the treatment zone. This scenario covers boron horizontal dispersal in 1K | 0 |
| Scenario 2B - boron vertical dispersal (downward) with groundwater in clay 0-50 cm from the treatment zone (10 days, the dispersal front has reached <3 cm). The scenario assumes a groundwater table above the drainage point in combination with the highest initial concentration and hence the calculated dispersion and transport of borates vertically down to clay layers is considered the worst-case scenario and covers vertical dispersal in 1K. | 73.5 mg B/L (420.5 mg BAE/L) |
| Scenario 2B – boron vertical dispersal (downward) with groundwater in clay 12-50 cm from the treatment zone (after 30 years). After 30 years boron dispersal front with concentrations above the PNECadd, gw has reached 12 cm from the treatment zone. | 0 |
| Scenario 1K – boron vertical and horizontal dispersal with groundwater in the filling material and the stone foundation crevices to the point of discharge/drainage (0- 30 years) | 0 |

Scenario 1B

The simulation for scenario 1B did not show any increase of boron in shallow, outflowing water at the point of discharge/drainage. Even modelling performed for a longer time span (30 years) did not show any increase in the concentrations of boron at the point of discharge/drainage.

Scenario 2B

Transport of boron occurs vertically and horizontally in the clay pore water. This transport results in a very steep concentration gradient with the highest concentrations closest to the depot of Eurobor and then decreasing concentrations with the increasing distance. The mean groundwater concentration in the clay pore water within a 50 cm distance from the Eurobor depot in vertical direction reached maximum 420.5 mg BAE/L (73.5 mg B/L) after 10 days after treatment (Annex 3.3.3, Figure 3.3.3-1) and after 1 year significantly decreased (Annex 3.3.3, Figure 3.3.3-2 and Joodaki & Vickman, 2020, §6.2, Fig. 8). The distribution of boron in pore water in horizontal direction within a 50 cm layer has resulted in a maximum value of mean groundwater concentration of 89.8 mg BAE/L (15.7 mg B/L) after 10 days (Joodaki & Vickman, 2020, §6.2, Fig. 11).

According to simulations performed for a distance of 6-50 cm horizontally and 12-50 cm vertically from the treatment zone, the PECadd, gw was 0 mg B/L over a 30-year time span due to the low permeability of the soil/clay.

In conclusion, the worst-case groundwater concentration in the clay (PECgw-clay) right beneath the treatment zone is 420.5 mg BAE/L (73.5 mg B/L). This concentration represents the concentration added to the groundwater as a result of treatment with Eurobor.

| Value used in the Risk Assessment – PECadd, gw |  |
| --- | --- |
| Value | 73.5 mg B/L |
| Justification for the selected value | Worst-case value |

An underlying assumption in the simulation of scenario 2B is that the groundwater table is above the level of the discharge/drainage system. In general, buildings treated with Jerbor exhibit low levels of groundwater which is part of the reason for them needing treatment with Jerbor. If the wooden foundation frequently was submerged in groundwater, the fungi would not grow since it requires oxygen and consequently no boron treatment would be needed. For buildings needing treatment, the groundwater table is typically at the bottom of the foundation wall.

Hence, the dispersal of boron vertically has been calculated for a scenario with groundwater levels that are higher than would be expected. With the groundwater levels that can be expected under normal circumstances, the dispersal will be lower due to the lower water pressure from above. So in conclusion, even though increased levels of boron vertically have been indicated by the model, the probability for such a scenario is so low that it cannot be considered to be representative for the intended use.

Scenario 1K

The modelling for scenario 1K resulted in no added concentration of boron in shallow, outflowing water at the point of discharge/drainage even after 30 years.

Monitoring studies of groundwater related to Jerbor treatment

The results of the modelling are supported by data from the monitoring studies where concentrations of boron in the groundwater were measured in the surroundings of buildings which had been treated with Jerbor (see 2.2.8.1.8).

Consequently, PECadd, gw is negligible if measured outside the treatment zone according to the results from the monitoring studies. The likelihood, that groundwater in adjacent areas will be exposed to groundwater from the treatment area, is low according to the data provided (Vickman and Danielsson, 2011; Vickman and Nilsson 2011).

Furthermore, the groundwater that may be affected by treatment of Jerbor is urban ground water located below buildings or roads. However, this groundwater will not be used as drinking water. According to the instructions for use, treatments are never done close to groundwaters used as drinking water (Bandh, 2022).

- 2.2.8.2.4.2 PEC soil

The results of the calculations of boron concentration in soil are presented in the table below. The PEC represents the concentration added to the soil as a result of treatment with Jerbor. The calculations of boron concentration in soil estimate the mean concentration of boron within 50 cm from the treatment zone vertically and horizontally (the same approach as in the method described in OECD, 2013).

|  | PECadd, soil |
| --- | --- |
|  | [mg/kg dw] |
| Horizontal dispersal (30 years) | 39.7 mg B/kg dw (227 mg BAE/kg dw) |
| Vertical dispersal (30 years) | 6.1 mg B/kg dw (249 mg BAE/kg dw) |

The short-term PEC calculations (365 days) showed an added average concentration of 41.4 mg B/kg (237 mg BAE/kg) dw horizontally in the soil within 50 cm from the treatment zone. The corresponding short-term PEC (365 days) for vertical dispersal was 43.6 mg B/kg (249.5 mg BAE/kg) dw within 50 cm from the treatment zone.

The calculations indicated comparably high concentrations of boron close to the treatment zone, but the concentrations quickly decreased in relation to the distance from the treatment zone due to the low permeability of the soil/clay. The calculations also showed that the transport of boron occurred within a very limited distance (6 cm horizontally and 12 cm vertically) from the treatment zone after 30 years.

| Value used in the Risk Assessment – PECadd, soil |  |
| --- | --- |
| Value | 43.6 mg B/kg dw (249.5 mg BAE/kg dw) |
| Justification for the selected value | Worst-case value |

Monitoring studies of soil (clay) related to Jerbor treatment

The results of the calculations are complemented by data from two monitoring studies (Vickman and Nilsson 2011, and Theorin 2011b), where concentrations of boron in soil (clay) were measured 5 years after treatment with Jerbor in different parts of Gothenburg (see 2.2.8.1.8).

In one of the studies, samples were taken next to the treatment zone (horizontally; 1.3 m below ground level). In the other study, samples were taken below the treatment zone (vertically). The clay was in direct contact with the treatment zone in both cases. The results from the horizontal sampling of the clay showed concentrations of boron of 61 and 82 mg B/kg dw (350-470 mg BAE/kg) 5 years later after the treatment (Vickman and Nilsson, 2011 - Eklandagatan 14 i Göteborg) whereas the vertical sampling showed concentrations of 52-85 mg B/kg dw (294-484 mg BAE/kg) (Theorin, 2011b - Kv. Malörten, Göteborg).

There are no measurements of background concentrations of boron from the two study sites before treatment with Jerbor. The results from the studies are therefore compared with background concentrations of boron from other parts of Gothenburg and Uppsala (see 2.2.8.1). Analysis of concentrations of borates in clay sampled in Gothenburg and Uppsala in areas relevant for treatment with Jerbor (but far from the treated buildings) showed concentrations between 53 and 135 mg B/kg dw (Theorin, 2011c), which is somewhat lower than the concentrations reported for marine clays in general. The background concentration of boron in marine clays from different areas of the world varies considerably and values between 25 and 1300 mg B/kg have been reported (review in Theorin, 2011a).

Consequently, the results from both monitoring studies show that the boron concentration in clay in direct contact with the treatment zone is within the same range as the background concentration of boron in marine clay. This implies that the PECadd, soil in clay, both vertically and horizontally in relation to the treatment zone, is thus negligible. This also means that the background concentration of boron in clay in urban areas is much higher than the PNECadd, terrestrial.

- 2.2.8.2.4.3 PEC surface water/sediment/STP

Due to the limited dispersal distance of boron from the treatment zone (Joodaki & Vickman, 2020), surface water, sediments and sewage treatment plants will not be exposed to any substantial amounts of boron. Calculations of PECsw, PECsed or PECSTP are therefore not considered relevant.

| Data waiving |  |
| --- | --- |
| Information requirement | PEC surface water/sediment/STP |
| Justification | Surface water, sediments and sewage treatment plants will not be exposed to any substantial amounts of boron due to the limited dispersal distance of boron from the treatment zone. |

2.2.8.2.5 Primary and secondary poisoning

Primary poisoning

A direct uptake of the product by mammals or birds is unlikely due to the following reasons:

- The treatment solution is applied by injection and is pumped through closed systems into the ground.

- If an accident occurs and there is a spill, exposure to non-target organisms will be stopped by:

  - sweeping /shovelling up large spills alternatively using an industrial vacuum cleaner if the spill is in the form of granules.

  - pumping off product or picking it up with suitable absorbent material if the spill is in the form of a water-based slurry.

As no exposure to birds and mammals are foreseen with the intended use of Jerbor, no PECprimary poisoning has been calculated. No further assessment of primary exposure of birds or mammals is therefore considered necessary.

Secondary poisoning

Boron does not bioconcentrate or bioaccumulate along the food chain in neither aquatic nor terrestrial ecosystems according to the CARs for boric acid (Assessment Report Boric acid, 2009) and disodium tetraborates (Assessment Report Disodium tetraborate, 2009). The low accumulation potential is supported by the low BCF of < 2000 L/kg wwt for fish.

No further assessment of secondary exposure via the food chain is therefore considered necessary.

| Data waiving |  |
| --- | --- |
| Information requirement | Primary and secondary poisoning |
| Justification | No exposure to birds and mammals is foreseen with the intended use of Jerbor. Moreover, boron does not bioconcentrate or bioaccumulate along the food chain in neither aquatic nor terrestrial ecosystems. |

2.2.8.3 Risk characterisation

In the final CARs for boric acid (Assessment Report Boric acid, 2009) and disodium tetraborates (Assessment Report Disodium tetraborate, 2009) the RMS states that the risk assessment should be conducted based on the added risk approach. The use of the added risk approach implies that only the anthropogenic amount of a substance, i.e. the amount added to the natural background concentration, is considered to be relevant for the effects assessment of that substance. In the added risk approach both the ‘Predicted Environmental Concentration’(PEC) and the ‘Predicted No Effect Concentration’ (PNEC) are expressed as added by man, resulting in an ‘added Predicted Environmental Concentration’ (PECadd) and ‘added Predicted No Effect Concentration’ (PNECadd), respectively.

The following arguments for using the added risk approach are brought forward by the RMS:

- There is inadequate knowledge of the natural boron concentrations in water, sediment and soil.

- There is insufficient knowledge to incorporate bioavailability.

- No studies are available that indicate acclimatization and adaptation effects to the sensitivity of species to Boron with the exception of deficiency occurring in some fish and plant studies.

- There is no information that boron is essential to all biota.

Following the argumentation in the final CARs for all boron species, the added PECs estimated by the applicant and discussed above should then be related to the PNECadd presented in the CARs. Although this approach may be relevant in areas where the background concentrations can be expected to be “natural background concentration” it may not be equally relevant to use in the present case. In the urban areas discussed in this report, the groundwater and the soil/clay are already influenced by anthropogenic activities.

In any case, it is clear that the worst-case PECadd for groundwater (420.5 mg BAE/L or 73.5 mg B/L) is much higher than the PNECadd suggested for groundwater in the CARs (5.7 mg BAE/L or 1 mg B/L). The worst-case PECadd for the soil/clay (249.5 mg BAE/kg dw or 43.6 mg B/kg dw) is also higher than the PNECadd for terrestrial organisms established in the CARs (2.29 mg BAE/kg dw or 0.4 mg B/kg dw).

The relevance of these comparisons of PECs and PNECs for the current risk assessment can and should however be discussed. There are several arguments that can be brought forward for not applying the PEC/PNEC approach in the risk assessment for Jerbor. The PNEC established in the CARs for terrestrial organisms may not be totally relevant for the organisms present in the soil/clay of the treatment area. Also, the background concentrations in the clay are much higher than the PNECadd for terrestrial organisms. Even though the PECadd for groundwater is higher than what is normally found in urban areas it will not be used for drinking water. Further, the likelihood that the groundwater in adjacent areas will be exposed by groundwater from the treatment area is low according to the data provided.

Since the intended use of Jerbor/Eurobor as wood preservative product is quite different compared to other wood preservative used for wood in the ground, the risk characterization should therefore be focused on the risk for transport/dispersion of boron from the treatment area to other areas. The Swedish Chemicals Agency therefore concludes that even though high boron levels can be expected for groundwater and clay close to the depot of Eurobor injected, the data submitted by the applicant indicate that the transport/dispersion of boron from the treatment zone is sufficiently limited to such an extent that the risks can be regarded as acceptable.

In the inclusion directives for boric acid and for disodium tetraborate there is a specific provision that products shall not be authorised for the in-situ treatment of wood outdoors unless data are submitted to demonstrate that the product will meet the requirements of Article 5 and Annex VI of the BPR (2012), if necessary, by the application of appropriate risk mitigation measures.

The Swedish Chemicals Agency considers that the data submitted demonstrates that the requirements of Article 5 and Annex VI of the BPR (2012) will be fulfilled if the conditions stated in the decision are applied.

2.2.8.3.1 Atmosphere

No risk characterisation of the atmosphere has been performed since the release of boron to the air will be minimal because of its low vapour pressure (Assessment Report Boric acid, 2009; Assessment Report Disodium tetraborate, 2009).

Conclusion: No risk characterisation of the atmosphere has been performed since air is not considered to be a relevant environmental compartment due to the low vapour pressure of boron.

2.2.8.3.2 Sewage treatment plant (STP)

No risk characterisation of sewage treatment plants has been performed since no considerable amounts of boron are expected to reach STPs due to the limited dispersal distance of boron from the treatment zone (Joodaki & Vickman, 2020). Moreover, investigation and mending of any broken parts of the sewage system is mandatory before treatment (Bandh, 2022).

Conclusion: No risk characterisation of sewage treatment plants has been performed since no considerable amount of boron is expected to reach STPs.

2.2.8.3.3 Aquatic compartment

No risk characterisation of the aquatic compartment has been performed since no considerable amounts of boron are expected to reach surface waters or sediments due to the limited dispersal distance of boron from the treatment zone (Joodaki & Vickman, 2020). Moreover, treatment is not allowed near surface waters if there is a risk for leakage (Bandh, 2022).

Conclusion: No risk characterisation of surface waters or sediments has been performed since no considerable amount of boron are expected to reach surface waters or sediments.

2.2.8.3.4 Terrestrial compartment

See information in the first section in 2.2.8.3

2.2.8.3.5 Groundwater

See information in the first section in 2.2.8.3

2.2.8.3.6 Primary and secondary poisoning

Primary poisoning

No risk characterisation of the primary poisoning of birds or mammals has been performed since exposure to birds and mammals is considered to be negligible as a result of the intended use of the product.

Secondary poisoning

Boron does not bioconcentrate or bioaccumulate along the food chain in neither aquatic nor terrestrial ecosystems according to the CARs for boric acid (Assessment Report Boric acid, 2009) and disodium tetraborates (Assessment Report Disodium tetraborate, 2009). The low accumulation potential is supported by the low BCF of < 2000 L/kg wwt for fish. No further assessment of the potential of secondary poisoning is therefore considered necessary.

2.2.8.3.7 Mixture toxicity

As Jerbor consists of two active substances, the environmental risk characterization should be based on the combined risk.

Screening Step 1: Identification of the concerned environmental compartments

The only environmental compartment that will be subject to exposure of the active substances is soil due to the intended use of the product i.e. treatment of supporting foundations located in dense clay more than 2 meters deep underneath buildings. The treatment solution is applied by pumping it through closed systems into the ground which means that there is virtually no risk of exposure of the surrounding environment.

It is only the treatment zone that will be subject to exposure. This area is limited to a few centimetres and dispersal from the treatment zone is unlikely due to the low permeability of the clay.

Screening Step 2: Identification of relevant substances

The active substances in Jerbor are both borates. When borates are released to the environment, the predominant species is un-dissociated boric acid. There are no other substances of concern in the product. Upon treatment, Jerbor is mixed with water and sodium bentonite to get a treatment solution which is called Eurobor. The treatment solution contains 70.4% water and 20% bentonite. Sodium bentonite is not considered as a substance of concern for the environment.

Screening Step 3: Synergistic interactions

There is no indication that boric acid and disodium tetraborate pentahydrate together produce an effect greater than the sum of their individual effects. No synergistic effects for the product or its constituents are therefore expected.

Conclusion

Conducting an assessment of the combined risk is not relevant in the case of Jerbor for the following reasons:

The number of relevant substances is one namely un-dissociated boric acid.

A significant environmental exposure is not likely due to the intended use of the product.

The product or its constituents do not have synergistic effects.

2.2.8.3.8 Aggregated exposure (combined for relevant emission sources)

At the time when this application was submitted, no agreed guidance on aggregated exposure was available. It is also unclear if an assessment of aggregated exposure should be conducted in the evaluation of an active substance or in the product authorisation process. An aggregated exposure assessment for different emission sources has therefore not been carried out.

Moreover, the product Jerbor will not contribute to increased levels of boric acid and disodium tetraborate pentahydrate in the environment due to the intended use of the product and the limited dispersal distance of borates from the treatment zone.

Figure 2.2.8.3.8-1: Decision tree on the need for estimation of aggregated exposure

Overall conclusion on the risk assessment for the environment of the product

The risk characterisation indicates that: - The intended use of the biocidal product Jerbor by professionals does not represent unacceptable risks to the environment if the instructions for use are carefully considered to prevent dispersal of boron to environmental compartments. Therefore, product authorisation of Jerbor can be granted, if appropriate mitigation measures are specified on the labels and associated literature is considered.

2.2.9 Measures to protect man, animals and the environment

Measures to protect man, animals and the environment are specified in the assessment. They are stipulated to be included in label directions.

2.2.10 Assessment of a combination of biocidal products

Jerbor is not used with other biocidal products.

2.2.11 Comparative assessment

No comparative assessment has been made as it has not yet been assessed for whether the active substance meets the criteria for exception under Article 5(2) and is a candidate for substitution in accordance with Article 10 of BPR.

- AnnexeS

- 3.1 List of studies for the biocidal product

| Author (s) | Year | File name, Title. Source (where differnt from company), Company, Report no. GLP (where relevant)/ (un)published | Data protection claimed (Y/N) | Owner |
| --- | --- | --- | --- | --- |
| Analys och konsultlaboratoriet i Borås AB. | 2019a | Provrapport, Grundvatten, Kobbarnas väg, Göteborg, no 2019_6052. Borås: AK lab. Not GLP / Unpublished | Y |  |
| Analys och konsultlaboratoriet i Borås AB. | 2019b | Rapport, borsyra i trä, Kobbarnas väg, Göteborg, no 2019_6301. Borås: AK lab AB. Not GLP / Unpublished | Y |  |
| Bandh, K. | 2022 | Manual Euroborbehandling. Rapport WSP. Not GLP / Unpublished | Y | Jerbol System AB |
| Bandh, K. | 2020 | PM, Kobbarnas väg 2-6, Långtidsuppföljning av Euroborbehandling Report no. K2208111 Not GLP / Unpublished | Y | Jerbol System AB |
| Bandh, K.-M. | 2012a | Sammanställning grundvattenvariationer kallmur. PM 2012-10-11, WSP. Not GLP / Unpublished | Y |  |
| Bandh, K.-M. | 2012b | PM, Kompletterande provtagning i kontrollrör, Kobbarnas väg. WSP Göteborg. Not GLP / Unpublished | Y |  |
| Bandh, K.-M. | 2010a | Djupedalsgatan 4 - Uppföljning Eurobor och utvärdering av dosering. Report, WSP. Not GLP / Unpublished | Y |  |
| Bandh, K.-M. | 2010b | Grundläggningsved i Sverige, behov av underhållsåtgärder. Report, WSP. Not GLP / Unpublished | Y |  |
| Bandh, K.-M. | 2010c | Kvarteret 13, Gamla Tullen, Göteborg. Exempel på en Jerborbehandling och dess långsiktiga funktion i fastigheterna 13:10 och 13:3. Report, WSP. Not GLP / Unpublished | Y |  |
| Bandh K.-M., Fimmerstad, K. and Theorin, M. | 2011 | Effectiveness of Water Based Bentonite-Borate Slurry for In-situ Remedial Protection of Historic Wooden Piles. The Eurobor Protocol. Part 2. IRG/WP 11-30574. IRG-WP. Not GLP / Unpublished | Y |  |
| Bandh, K. and Theorin, M. | 2011a | Eurobor svällande egenskaper och behandlingszonen Not GLP / Unpublished | Y | Jerbol System AB |
| Bandh K. and Theorin, M. | 2011b | Protective Levels of Borates in Wooden Foundation Piles 5 – 18 Years After In-situ Remedial Treatment in Areas with Wet Clay Ground Conditions. The Jerbor and Eurobor Protocol, Part 1. IRG/WP 11-30571. IRG 2011. Not GLP / Unpublished | Y |  |
| Bech-Andersen, J. | 2005 | Hussvampe og Husbukke. ISBN 87-89560-34-5. |  |  |
| Bermin, J. | 2012 | Eurobormetoden, Spridningsberäkning i kallmur. Rapport WSP. Not GLP / Unpublished | Y |  |
| Blanchette, R. A. | 1984 | Manganese Accumulation in Wood Decayed by White Rot Fungi. Ecology and Epidemology, 725-730. |  |  |
| Communar, G., Keren, R. and Li, F. | 2004 | Deriving Boron Adsorption Iso-therms from Soil Column Displacement Experiments. Soil Science Society of America Journal 68(2). |  |  |
| Couch, E. L. and Grim, R. E. | 1968 | Boron fixation by Illites. Clays & Clay Minerals 16: 249 - 256. |  |  |
| Daniel, G. and Bergman, Ö. | 1997 | White Rot and Manganese deposition in TnBTO-AAC precervative treated pine stakes from field tests. Holz als Roh- und Werkstoff, 197-201. |  |  |
| ECHA | 2017 | Guidance on the Biocidal Products Regulation Volume IV Environment - Assessment and Evaluation (Parts B + C), Version 2.0. |  |  |
| Fetter, C.W. | 2001 | Applied Hydrogeology. 4th Ed. Prentice Hall, Upper Saddle River. |  |  |
| Flemming, R and Koch, A. P. | 1992 | Valid scientific names of wood decaying fungi in construction timber and their vernacular names in England, Germany, France, Sweden, Norway and Denmark. The International Research Group on Wood Preservation. Document No: IRG/WP/1546-92 |  |  |
| Freeman, M. H., McIntyre, C. R. and Jackson, D. | 2009 | A Critical and Comprehensive Review of Boron in Wood Preservation. Report. Unpublished |  |  |
| Freitag, C. and Morrell, J. J. | 2005 | Development of threshold values for boron and fluoride in non-soil contact applications. Forest Prod. J. 55:97-101. |  |  |
| Grenberg, U. and Engström, E. | 2022 | Validation of method B-01 for the quantitative determination of B and Na in boric acid/borax mixture by ICP-SFMS ALS Scandinavia Report no. VB01 | Y |  |
| Henningsson, B., Friis-Hansen, H., Käärik, A. and Edlund, M.-L. | 1988 | Remedial ground-line treatment of CCA poles in service. A progress report after 28 months’ testing. Document No: IRG/WP/3481. |  |  |
| IVL | 2012 | ”Effektiva åtgärder mot damm på byggarbetsplatser”, Christinasson B, Östlund G, Alvarez E, Antonsson A-B, IVL Rapport B 2057, pp 1-99. |  |  |
| Jerbor | 2004 | Avd 3.2 Diagnos av röta/ tänedbrytande svamp. Rapport Jerbol AB. Not GLP / Unpublished | Y |  |
| Joodaki, S. and Vickman, A. | 2020 | Haltberäkning – Användning av Jerbor enligt Eurobormetoden. Rapport WSP. Not GLP / Unpublished | Y |  |
| Klaassen, R. K. W. M. | 2008 | Bacterial decay in wooden foundation piles—Patterns and causes: A study of historical pile foundations in the Netherlands. Int. Biodeterior. Biodegradation. 61:45-60. |  |  |
| Kochkodan, V., Hilal, N. and Bin, N. | 2015 | The Chemistry of Boron in Water. Boron Separation Processes (ss. 35-63). Elsevier |  |  |
| Kresic, N. | 2007 | Hydrogeology and Groundwater Modelling. 2nd Ed. CRC Press, Boca Raton. |  |  |
| Larsson, R. | 2008 | Jords egenskaper, 5:e utgåvan, SGI (Statens Geotekniska Institut). |  |  |
| Lehringer, C. | 2011 | Permeability improved of Norway spruce wood with the white rot fungus Physisporinus vitreus. Doctoral thesis. 1-23. |  |  |
| Naturvårdsverket | 2009 | Riktvärden för förorenad mark, modellbeskrivning och vägledning, Rapport 5976. |  |  |
| Naturvårdsverket | 2008 | ”Hälsoriskbedömning vid utredning av förorenade områden”, Liljelind I & Barregård L, rapport 5859, september 2008, ISBN 978-91-620-5859-3 |  |  |
| OECD | 2013 | Series on Emission Scenario Documents, Number 2. Revised Emission Scenario Document for Wood Preservatives. |  |  |
| RISE | 2020 | Evaluation of the concentration of “Jerbor” needed for decay resistance of wood. Report. 1-10. Not GLP / Unpublished | Y |  |
| Rodhe, A., Lindström, G., Rosberg, J. and Pers, C. | 2006 | Grundvattenbildning i svenska typjordar, Geotryckeriet, Institutionen för geovetenskaper, Uppsala Universitet. |  |  |
| Schmidt, O. | 2007 | Indoor wood-decay basidiomycetes: damage, causal fungi, physiology, identification and characterization, prevention and control. Mycol Progress. 6:261-279. |  |  |
| Schlumberger Water Services | 2006 | Visual MODFLOW Professional. User’s Manual. |  |  |
| Schwarze, F. W. M. R. | 2007 | Wood decay under the microscope. British Mycological Society. Fungal biology reviews. 1-38. |  |  |
| Schwarze, F. W. M. R., Landmesser, H., Zgraggen, B. and Heeb, M. | 2006 | Permeability changes in heartwood of Picea abies and Abies alba induced by incubation with Physisporinus vitreus. Holzforschung. 60:450–454. |  |  |
| Theorin, M. | 2022 | Jerbor Penta – Physical and Chemical Properties RISE AB, Report no. 140185 Not GLP / Unpublished | Y | Jerbol System AB |
| Theorin, M. | 2021 | PM – Physical and chemical properties of the biocidal product (wood preservative) Jerbor Penta WSP Jerbol, Report no. K2208111 Not GLP / Unpublished | Y | Jerbol System AB |
| Theorin, M. | 2011a | Trägrundläggningar, lera och borföreningar i mark. Rapport 2010:4, Rev. 2011:1. Not GLP / Unpublished | Y |  |
| Theorin, M. | 2011b | Kvantitativ bestämning av borater i prover från Euroborbehandlingar. Rapport 2011:2. Not GLP / Unpublished | Y |  |
| Theorin, M. | 2011c | Kvantitativ bestämning av borater i prover från leror med marint ursprung. Rapport 2011:5. Not GLP / Unpublished | Y |  |
| Theorin, M. | 2011d | Bestämning av fysikaliska egenskaper for bentonitförtjockad Jerbor (Eurobor) Report no. 2011:6 Not GLP / Unpublished | Y | Jerbol System AB |
| Theorin, M., Fimmerstad, K. and Bandh K-M. | 2011 | Effectiveness of Water Based Bentonite-Borate Slurry for In-situ Remedial Protection of Historic Wooden Piles: The Eurobor Protocol, Part 2 (IRG/WP 11-30574). Not GLP / Unpublished | Y |  |
| USDA | 2008 | USDA Natural Resources Conservation Service, 2008. Soil Quality Indicators. |  |  |
| Vickman, A. | 2011 | Emissionsscenario - Användning av Jerbor enligt Eurobormetoden. Rapport WSP. Not GLP / Unpublished | Y |  |
| Vickman, A. and Bandh, K.M. | 2011 | Beskrivning av behandlingsmiljön – Användning av Jerbor enligt Eurobormetoden. Rapport WSP. Not GLP / Unpublished | Y |  |
| Vickman, A. and Danielsson, L. | 2011 | Rapport: Spridningsundersökning vid injektering av Eurobor för bevarande av träpålar. Kobbarnas väg 2-8, Göteborgs stad. Report no. K2208040 Not GLP / Unpublished | Y | Jerbol System AB |
| Vickman, A. and Nilsson, A. | 2011 | Undersökningar av tidigare utförda Euroborbehandlingar. Rapport WSP. Not GLP / Unpublished | Y |  |
| Vickman, A. and Valtersson E. | 2011 | Test av genomsläppligheten i fyllnadsmaterial Eurobormetoden. WSP Environmental. Not GLP / Unpublished | Y |  |
| Zheng, C. and Wang, P. P. | 1999 | MT3DMS: A modular three-dimensional multispecies transport model for simulation of advection, dispersion, and chemical reactions of contaminants in groundwater systems; documentation and user guide. US Army Corps of Engineers. Contract Report SERDP-99-1. |  |  |

- 3.2  Output tables from exposure assessment tools

3.2.1 Input files for the human health risk assessment

3.2.2 Input data for environmental risk assessment model scenario 2B

Figure 3.2.1-1. Adsorption constant (Kl) in l/mg and maximum absorption (S) in kg/kg.

Figure 3.2.1-2. Density parameter

Figure 3.2.1-3. Diffusion coefficient.

Figure 3.2.1-4. Dispersion coefficient (for 0.2 m transport distance).

3.2.3 Output of model scenario 2B

Figure 3.3.3-1. A vertical profile of the spreading of borates (mg BAE/L) in groundwater for scenario 2B after 10 days. The gray/green color represents the concrete wall. The highest concentration is marked with red, and the lowest concentration is marked with blue color. The treatment area is just under the concrete wall which is a 0.01 m thick cell. The next 10 cells below have the thickness of 0.0048 m.

Figure 3.3.3-2. A vertical profile of the spreading of borates (mg BAE/L) in groundwater for scenario 2B after 100 days. The gray/green color represents the concrete wall. The concentration variation in the treatment zone after 100 days is shown by the color scale. The treatment area is just under the concrete wall which is a 0.01 m thick cell. The next 10 cells below have the thickness of 0.0048 m.

  - Confidential annex

NB: This information is confidential and should not be disclosed to third parties.

Qualitative and quantitative information on the composition of the biocidal product

| Common name | IUPAC name | Function | CAS number | EC number | Content % w/w |
| --- | --- | --- | --- | --- | --- |
| Boric acid (min. purity 99.0%) | Boric acid | Active substance | 10043-35-3 | 233-139-2 | 53.7 (53.2) |
| Disodium tetraborate pentahydrate (min. purity 99.0%) | Disodium tetraborate pentahydrate | Active substance | 12179-04-3 | 215-540-4 | 46.3 (45.8) |

- 3.4 Other

Additional information from the applicant and not edited by refMS

Introduction

Jerbor* was first developed during the late 1970s by the late professor Allan Jerbo and has been used for curative treatment of foundation wood, piles and grillages (rafts), against decay by wood rooting fungi since 1983. In the early 1990s the Eurobor technique was developed from the Jerbor technique. Sodium bentonite was mixed with Jerbor solution to reduce the risk for leakage and increase efficacy over time.

Soft soils dominated by clay are often situated when rivers meet or have met the ocean and this coincide with places where humans often have built their cities (Holm, C. Olsson, G. 1993). In areas with mighty clay depths buildings have traditionally been built with wooden piles as supporting foundation. Timber foundation with piles and grillages has been used for centuries with great success and often for buildings of historic important value (Klaassen, René K. W. M. 2008). In northern Europe these foundations are known in

Copenhagen, Oslo, Stockholm, Helsinki, Gothenburg any many other cities. In Sweden alone there is approximately 60 cities with wooden foundation in soft clay (Lundin, S-E 2000). Early man knew that wood beneath water had long durability (Brynjer, T. 2001). Piles was therefore installed in deep ditches beneath ground water table and covered with clay (Bergren, K. et al. 1990). Wooden grillages, stone or concrete foundations were then placed on top of the piles. The wooden foundation takes the load from the building either by transfer of the force to the clay with cohesion forces or by standing on the underlying rock i.e. point bearing piles.

Urbanization with subway development, tunnelling, installation of sewage pipes, impervious surfaces (reduces infiltration of rain water) contribute to a

lowered groundwater table which causes timber foundations to get exposed to air with increasing risk for accelerated decay by rot fungi (Bjerking, S-E. 1989; Bergren, K., Fabricius, O. et al. 1990). Decay by bacteria is always present but effects on wood durability are often minor. If oxygen in measurable concentrations is present fungal decay will dominate and destroy wood in relative short periods (Gelbrich, Huisman, Kretschmar, Klaassen, Keijer, Lamerdorf 2005). Fungal decay of supporting constructions such as a timber foundation will eventually cause odd settlements with a consequence of severe damages to the building.

If the decay is discovered in time when the wood still has sufficient strength a preservation technique can be used, such as the Eurobor technique. Other alternatives to preserve foundation wood is only hydrological manipulation which is often difficult to do and rarely with good result over time (Brynjer, T. 2001; Bandh, Karl-Magnus 2010). (Elam, Björdal, 2020)

However there are great advantages to address the problem early and avoid reinforcement with underpinning. Underpinning often results in risk for damages to the building itself and the surroundings (Brynjer, T. 2001). Reinforcement of a foundation with large amounts of concrete and steel results in big costs and considerable use of natural resources as well as

release of carbon dioxide (Bjerking, S-E. 1989; Bandh, Karl-Magnus 2010).

Decay by wood root fungi on wooden foundations can be curative treated with Jerbor and the Eurobor technique (Brynjer, T. 2001; Bandh, K-M. 2011). Toxic retention levels of borates in the foundation piles and grillages have been proved after 5 years or more (Bandh, K. et al. 2011) Since 1940:s 1 kg boric acid/m3 wood has been recognized as toxic retention level against decay by basidiomycetes (Drysdale, J. A. 1984; Lloyd, J. D. et al. 1998; Peylo, A. et al. 2001). European standard for boron retention, EN113, in non

termite areas, are set to 0.89 for Coriolus versicolor, 0.69 for Gloeophyllum trabeum, 0.38 for Coniophora puteana and 0.35 Poria placenta as kg BAE/m3 wood (Lloyd, J. D., Schoeman, M. W. et al. 1998). More recent studies (Freitag, C. et al. 2005) showed toxicity of 0.4-0.44 kg BAE/m3 of wood against decay by fungi, for more details se section 2.2.5.6.

*) Jerbor is a registered trademark at Jerbol System AB.

3.4.1.1 Overall use pattern

Remedial in-situ treatment of foundation wood against decay by wood rooting fungi. Use is restricted to trained professional staff according to the manual of Eurobor technique (Bandh, K-M. 2022).

Overall use pattern

The following main activities is mandatory:

1.    Foundation investigation

2.    Preparations before treatment

3.    Treatment with measurements

4.    Follow-up of result

For more specific requirements and details, see 2.2.1.4 below and the manual of Eurobor technique (Bandh, K-M. 2022).

Overall requirements

An investigation of the surroundings, foundation and the foundation wood must always be performed before a curative treatment can be made. The state of the foundation wood quality must be documented in order to evaluate the need and possibility for curative treatment with biocide. Ground conditions required for treatment are dominated by dens clay (of marine origin) with low water turnover in the treatment zone (Bandh, K-M. 2022).

Fungal decay zone and treatment zone

Wooden foundations were constructed deep down in soft clay to ensure coverage of water and thereby increase the durability. Natural deposited and undisturbed dense clay surrounds these deep foundations at bottom and along sides, well over level of upper

foundation wood. Refilling around the foundation was often made with the soft clay from the trench to seal the foundation and also to avoid unnecessary transport of heavy spoil (Bergren, K., Fabricius, O. et al. 1990). The water surrounding these foundations is therefore often to be regarded as isolated from other groundwater. In most cases the overflow level from a foundation is the level of outgoing sewage pipe. The isolated nature of this water makes it anoxic over time which can be seen both in the clay with formation of FeS(s) deposits and in decayed wood with precipitation of Mn-deposits formed during degradation in lack of oxygen (Blanchette 1984, Schmidt et al. 1996)

Bacterial decay in anaerobic environment is normally a very slow process often over hundreds of years but when just measurable levels of oxygen is present fungi decay will be the problem (Gelbrich et al 2005). Damages in buildings due to lack of strength in wooden foundation can occur just in a few years after an incipient fungal attack (Brynjer, T. 2001). In most cases a lowered level of water table leads to air intrusion with increased levels of oxygen which enables fungal decay to start. There are many reasons for water level to drop but most common is urbanization with disposal of rainwater through storm drains and installation of sewage pipes and tunnelling with unwanted draining effects (Bergren, K., Fabricius, O. et al. 1990, Elam and Björdal 2020). Fungi decay in wooden foundations is aerobe and therefore always located in upper parts of the wooden foundation (top of piles and/or grillage). Fungal degradation is common in this environment despite the often very high moisture content in the wood, e.g., the white rot fungi Physisporinus vitreus is able to attack wood at moisture content up to 320%. Often there is a thin permeable layer that increases the risk of decay where the upper part of the supporting wooden structure meets the bottom of the dry wall or concrete fundament. This permeable layer is the treatment zone (Lundin, S-E 2000), (Schmidt et al, 1997).

Treatment of wood

A treatment according to the Eurobor technique aims to fill the treatment zone with the Jerbor in a water bentonite mixture to create an external depot of borates in the decay area. The wet environment enables excellent diffusion of active substances into the wood, with high water content, and also it enables bentonite swelling (2,7 times original volume) (Theorin, M. 2011b) which seals the area from water circulation and thereby keeps active substances in the zone over time (Schoeman, M. W. et al. 1998; Brynjer, T. 2001; Askania 2005; Bandh, K-M. et al. 2011).

Planning and risk analysis is always made before a treatment to evaluate any needs of sealing measures.

Measurements (function and environmental controls) is always carried out and documented before, during and after treatment according to the manual of Eurobor technique (Bandh, K-M. 2022). A treatment is always committed with a commercial warranty of 5 years that guarantees levels of active substances to control fungi in wood.

Release estimates

No typical OECD use pattern scenario is available for curative treatment, with an external depot of the biocidal product, of upper part of foundation wood such as piles and grillages. A modified scenario according to use class 4 has been developed for the use of Jerbor with the Eurobor technique to preserve wooden foundations (Joodaki & Vickman, 2020).

In the risk assessment possible risk for soil and ground water exposure have been identified. Control measurements outside the treatment zone have not shown any increased boron concentrations in soil or ground water. Simulated data show a possible exposure of soil and groundwater below the treatment zone but the exposure is limited

to the area closest to the treatment zone (6-12 cm in 30 years).

In addition, the material in the treatment zone is always of marine origin with substantial natural background concentration of borates. Other organisms than microbes are not present close to the treatments zone at these depths.

3.4.1.2 Method of application including description of system used

F.20, injection and F.60 diffusion

Remedial D.50, use class E.40, trained professional A.30, in-situ injection, indoor on-site mixing, water temp 50-70 °C, slightly modified according evaluated intended uses for active substances. (Assessment Report Boric acid, 2009, and Assessment Report Disodium tetraborate, 2009)

Concentrations of substances and function of sodium bentonite

Jerbor (9,6% w/w) is diluted in hot water (70,4% w/w) and the solution is mixed with sodium bentonite (20% w/w) before injection. Two of the most important purposes of the addition of sodium bentonite is to increase viscosity and thereby significantly reduce risk of accidental leakage to surroundings (the slurry has pseudoplastic properties and will get increased viscosity with time) (Theorin, M. 2011b) and secondly the bentonite will swell in volume (up to 2,7 times), decrease circulation even more and seal the treatment zone. This function will secure a long-time wooden protection (Bandh, K-M.  et al. 2011).

Bentonite hydration and swelling starts on mixture and continuous after injection and the premise for swelling is water and available voids. Swelling is therefore not possible in a dry environment. The treatment zone (see 2.2.1.3 above), where the biocide product is injected, is located below the building inside the surrounding trench of the wooden foundation. This zone is a limited space, with floor and walls of the original trench borders forming a geological barrier that enclose the biocide inside. Often there is a permeable layer, with minor voids, within this area that will be sealed trough bentonite swelling (Bandh, K-M. 2022).

Detailed use pattern

The use and application of Jerbor is strictly limited to wooden foundations, piles and grillage, in ground with dense soft soil and high clay content (of marine origin) and low water turnover according to the manual of Eurobor technique (Bandh, K-M. 2022).

- Foundation investigation

A foundation investigation aims to control foundation design, relevant conditions in surroundings (for any necessary measures) and set the state of foundation wood quality. The purpose with the inspection is to give appropriate recommendations of measures.

An investigation mainly includes:

- Study of existing drawings of building, geological mapping and geotechnical information

- Visual inspection of the property and surroundings

- Installation of groundwater tubes

- Measurements of groundwater levels and levels of upper part of foundation wood to determine if there is lack of water coverage. Important is also to study water table gradient and the overflow level from the foundation.

- Sampling of water for laboratory analysis

- Making inspection pits for wood sampling and inspection of surrounding soil, voids, construction design and water inflow etc.

- A full laboratory analysis of wood samples including several physical and microbiological properties in order to set the state of wood quality

- Control of sewage pipes beneath the building

- Subsidence levelling

Recommendation of a treatment according to the Eurobor technique is only submitted if conditions are suitable and risk analysis is executed regarding environmental impact.

- Preparations before treatment

Preparations before a treatment are based on report and documentation from the foundation inspection. The purpose with the preparations is to get necessary documents and conditions for a successful implementation of the treatment.

- If necessary, additional investigation

- Repair any sewage pipes beneath the building with leakage and pump out groundwater contaminated with wastewater

- Approved working documents

- Approved quality-, environment- also health and safety plans

- Necessary control document

- Information to third parties

- Agreement with subcontractor for drilling

- Schedule is prepared

- Inspection of site

- Surveying of installations in ground

- Mark out injection holes with the identity of each hole on site according to drilling drawing

- Installation of ground water tubes for environmental control

- Water sampling for analysis of background levels of boric acid before treatment

- If necessary, performance of sealing measures

- First meeting to inform client, staff and subcontractor

- Treatment with measurements

Treatment begins when preparations are done and it is important that the work is performed according to prepared plans, working documents and the manual of Eurobor technique. Only trained professional users are allowed to perform a treatment and responsible management at site always have good knowledge of results from the

foundation investigation and all preparations.

- Drilling of injection holes are made with concrete drilling equipment. The diameter of the holes is 50 mm and normally the distance between holes are 0,8 – 1,2 m. Normally drilling is made from the basement trough the dry wall or the concrete fundament to the upper part of the wooden foundation. Drilling can be done from outside of building if necessary. The accuracy in placement of holes is important because injection has to be made with precision. The drilling is documented in protocols and all holes have to be drilled before other work can start.

- Before treatment measurements are performed in the injection holes regarding water level, electric conductivity and pH value. All values are documented in protocols.

- Mixing of granular active substances to granular Jerbor is always made at special production and storage site. A fix mixing station has been set up with a weighing scale and local exhaust ventilation and a filling device adapted to the UN-proved containers used. Personal protection according to safety data sheet is mandatory to use in all handling with active substances and Jerbor

- The Jerbor concentrate is transported and stored in labelled UN-proved containers. The storage room is always locked when it is not supervised. The safety data sheet and the health and safety plan are posted at site.

- Set up of mixing station is done indoors and a spill tray is placed on the floor. Any floor drain will be temporary sealed and equipment for spill handling is always at site. The equipment for mixing and injection is:

  - Mixing tank for Jerbor and water (stainless steel, conical bottom, with circulation pump and removable lid), 250 l

  - Mounted plaster and mortar mixer (50 l vessel with tilt function) which is connected with hose and valves to the mixing tank.

  - Remote controlled plaster and mortar pump

  - Hoses, valves and manometer

  - Injection packer

- Mixing of Jerbor to the treatment solution with water and bentonite (colloquially called Eurobor slurry) is made in the following steps:

  - 151 l of hot water (50-70°C) is filled in mixing tank and circulation pump is started

  - 20,6 kg of Jerbor is portioned in mixing tank under carefully stirring

  - 18 l of the Jerbor water solution is filled (through a connected hose with a ball valve) in to the mixing vessel and 4,8 kg of sodium bentonite is portioned during mixing with the plaster and mortar mixer.

  - Mixing is done until all bentonite is in the mixing vessel and then mixing will stop for a minute and the mixing starts again until the slurry is smooth and the viscosity is correct.

  - The 20 l of slurry is poured by tilting the mixing vessel into the plaster and mortar pump, ready for injection.

- A team for a treatment shall at least consist of 3 trained professional users and communication is continuous with Walkie-talkie between mixing station and injection place.

- Any spill containing Jerbor is always collected and disposed in the injection holes.

- Injection to treatment zone and control of spread is made in the following steps:

  - Two copies of the injection plan must be at site, one at the mixing station and one at the injection place.

  - An injection packer (with expanding rubber) is installed into the injection hole and green light is given to the mixing station to start injection.

  - Injection is always done with low pressure (normally less than 1 bar and never above 3 bar) and a 20 L batch is injected at slow pace. A continuous control of the horizontal spread in the treatment zone is made through nearby injection holes. If necessary, another 20 L bath is started after order from injection place and injection can be stopped at any time from both ends. When the Eurobor slurry is clearly detected in the nearest hole(s) the injection is stopped and the hose with injection packer is moved to the next hole for injection. This routine, with maximum of 20 L slurry injected at one time enables excellent monitoring and documentation of the treatment. Also, each part of the foundation is only treated with the amount required since the injection stops when obvious spread is detected in the next hole(s) The advantage of working with small batches at the time is that risk of incorrect injection is reduced if a mistake would occur. Planned amount (estimated dose) of slurry shall be seen as a target but the spread control determines if more or less will be injected (within certain limits). In some cases, it can be justified return to a previous injected hole after a day or two (when viscosity and swelling has increased) to make a minor additional filling. The control of the spread in nearby holes is made through visual observations, measurement of conductivity or water level changes. All observations, measurements and the injected amount are documented in protocols.

- Environmental controls consist mainly of:

  - The continuous spread control during the injection, described above.

  - Controls in ground water tubes outside the treatment zone through conductivity measurements and water sampling for boric acid analysis. This is made during and after injection.

  - Follow-up action and warranty inspections are also a part of the environmental controls.

- The restoring and finishing of the works include sealing the injection holes with concrete and cleaning the site.

- All protocols and lab results are attached to final report of the works that also includes instructions to the client.

Function of treatment:

Applied Eurobor slurry (in treatment zone) constitutes a depot of active substances which penetrates into foundation wood by diffusion (due to high water content in wood) to reach toxic retention levels against decay by fungi. Available active substances left in the injected slurry in the treatment zone act as outer depot available for future need in-situ. Over time the concentration of active substances strives for equilibrium between depot and wood. Five years after treatment analysis and estimation show that approximately 
10 – 25 % of the active substances penetrate and adsorb in foundation wood while the rest is available in the outer depot (Bandh, K-M 2011).

Bentonite swelling will reduce (the already low water turnover) the risk of leakage from treatment zone. Upon swelling borates is released (Theorin, M. 2011b), which lead to reduced cohesive forces between bentonite particles in the swelling front. This will give

increased swelling and with time sealing of the void. Application is made by injection through bore holes to voids in the upper part of wooden foundation, creating external depots of borates in direct contact with the foundation wood. The amounts of voids is

highest near the upper part of the foundation (Lundin, S-E 2000). The purpose is to reach toxic retention levels against fungal decay in the wood and seal the foundation from air and water circulation. The fungi demand for oxygen will thereby also be minimized as a

consequence of the treatment. Decay will be prevented by both biocide effect and decrease of oxygen levels close to the wood (Schubert, M. et al. 2009).

- Follow-up of results

First and most important control of result is the spread control during injection. Another control of results is the environmental control of water sampling in water tubes outside the building during and a few weeks after injection. A leakage outside the building is a failure to the environment and if severe also to function as wood protection.

A treatment is always committed with a commercial warranty of 5 years that guarantees levels of active substances to control fungi in wood located in the treatment zone. Client receives terms, conditions and instructions to take no action concerning the foundation without contacting the responsible for carrying out the treatment (furthermore, any measures at this depth under the building are very unusual).

Follow-up controls after treatment:

- Most of the buildings with wooden foundation are continuously monitored with subsidence levelling. This measurement aims to detect any vertical movements in the building before cracks appear. Typical for decay related movements are differential subsidence within the building. Indications of how the treatment has succeeded can be obtained by analyzing new data with settlement history.

- Wood sampling and analyzing of active substances is the safest way to evaluate the function of a treatment. In some cases, wood sampling is done as a function control in the beginning of warranty time. A routine sampling of wood is always made after five years before warranty expires. Sampling can be done by coring or by digging test pits.

- Other ways to try controlling the function is to sample water or Eurobor slurry in the treatment zone. This sampling is often difficult to evaluate and only used in some cases as complement.

3.4.1.3 Application rate and the final concentration of the biocidal product and active substance in the system in which the preparation is to be used

Required concentration of active substances in preservative treated wood

Retention of toxic concentration levels (of BAE according to EN 113 standard protocol) is set as lower concentration limit for effective treatment with the system used. Borates have been used as preservatives since the early 20th century and are well documented (McCullen, J M 1953; Cockcroft, R. et al. 1978; Lloyd, J D 1998).

Toxicity of borates for decay fungi degradation of wood have been evaluated during decades and the established limit of 1 kg BAE/m3 of wood is still accepted (Drysdale, J. A. 1984; Lloyd, J. D., Schoeman, M. W. et al. 1998; Peylo, A. and Bechgaard, C. G. 2001). European standard, EN113, for boron retention, in non-termite areas, are set to 0.89 for Coriolus versicolor, 0.69 for Gloeophyllum trabeum, 0.38 for Coniophora puteana and 0.35 Poria placenta as kg BAE/m3 wood (Lloyd, J. D., Schoeman, M. W. et al. 1998). More recent studies (Freitag, C. and Morrell, J. J. 2005) showed toxicity of 0.4-0.44 kg BAE/m3 of wood against decay by fungi, for more details se section 2.2.5.6.

Target concentration in treatable wood over 5 years

The target for a treatment with Jerbor according to the Eurobor technique is to maintain a concentration above 1 kg BAE/m3 for at least 5 years in treatable wood. Treatable wood is roughly the sapwood located in the ½ - 1 m of upper part of wooden foundation, (see also 2.2.1.3 above, Fungi decay zone and treatment zone).

Foundation wood is of timber from soft wood species such as Pinus sylvestris and Picea abies. All foundation wood have a high-water content which enables radial diffusion of the water-soluble borates (Brynjer, T. 2001; Bandh, K-M., Fimmerstad, K. et al. 2011).

Often the applied depot of the Eurobor slurry is in contact with the end grain which results in a quick and longitudinal spread in the wood. Intact heartwood has normally low permeability for the biocide compared to intact sapwood. Commenced decay increases the permeability (and the water content) and thereby also increases the spread of borates in wood (Lehringer, C. 2011). Approximately ½ - 1 m of the upmost part of the piles and grillage are available for biocide treatment. The dense clay beneath (and at the sides of) the upper part of the wooden foundation means that water turnover in the pile is negligible at depths over 1 m. Fungi decay requires oxygen and foundations with a lack of stable water coverage always risk decay in upper parts. Part of foundation intended for biocide treatment is therefore consistent with upper part of foundation where fungi decay appears.

It is particularly important that the target concentration (maintained for at least 5 years) in treatable wood is sufficient (above 1 kg BAE/m3) in the boundary between decayed and healthy wood.

A successful treatment according to the Eurobor technique normally gives a protection well beyond 5 years (Bandh, K. and Theorin, M. 2011).

Concentration of active substances in Eurobor slurry

Concentration of active substances in final solution of Eurobor slurry is 9,0 % BAE w/w (about 10,6 % BAE w/v) with a mixing temp of 50-70°C, (compared to 6 – 12 % w/v BAE in evaluation of active substances listed in annex II) (European Union, Standing Committee on Biocidal Products 2009b; a).

The target dose for treatable wood

The target dose for treatable wood is set to 2 kg BAE/m3 (Assessment Report Boric acid, 2009, and Assessment Report Disodium tetraborate, 2009)

Estimated dose of active substance

The foundation inspection is the basis for the estimation of the dose for application. When estimating the dose for application there are several factors to consider, first is to estimate the volume of treatable of wood. Applied dose have to be at least twice the target dose of 2 kg BAE/m3 because the external depot will strive towards equilibrium with foundation wood. However 4 kg BAE/m3 is just enough for an ideal state without any losses and full equilibrium between depot and wood. Considering adsorption to concrete, clay surfaces, some dilution in present water of treatment zone and uneven distribution the needed dose usually is estimated to 6 – 9 kg BAE/m3.

Another estimation that has to be in consideration is the amount of voids in the treatment zone. This is important to get an accurate horizontal spread between injection holes and to get a sealing effect of the bentonite swelling. This estimation of voids requires experience and is used as final adjustment factor for the estimated of needed dose (6 – 9 kg BAE/m3). If amount of voids is high the adjustment results in an increase of needed dose and in some cases when amount of void is low the adjustment can be a minor reduction.

Estimated dose is thereby often between 5 -12 kg BAE/m3.

Needed dose = target dose for treatable wood (2 kg BAE/m3) × 2(for equilibrium between depot and wood) × 1,5 to 2,25 (for adsorption and dilution etc.) Þ 6 – 9 kg BAE/m3 treatable wood.

Estimated dose = needed dose adjusted for amount of voids.

Applied dose

Applied dose is finally determined during injection; see 2.2.1.4 above (Injection to treatment zone and control of spread).

Normally applied dose is consistent with the estimated dose for a complete treatment but will differ some within different parts of the foundation.

Experiences have showed that estimated and applied amounts correlate well, the obtained concentrations in analyzed wood samples taken years after application show results of toxic retention levels 1 – 5 kg BAE/m3 wood (Bandh, K. and Theorin, M. 2011).

3.4.1.4 Number and timing of applications, and information relating to geographical variations

Number of applications

Application is made once regarding the fungicide treatment of the entire foundation. Determination of foundation wood BAE concentration is made 5 years post application to evaluate applied amount and obtained effectiveness of the biocide treatment.

Evaluations of treatments with Jerbor: show that dosage and effect correlate and that application only have to be made one time (Bandh, K-M 2011). This is due to the enclosed environment with dense clays of marine origin, low water turnover and sealing effect of bentonite.

If a 5 year control of wood indicates low retention of borates in some part of the foundation the first step is to investigate possible causes. Next step is an extended sampling program to determine boundaries between sufficient and insufficient retention levels of borates from treatment.

A partial and additional treatment can then made, in addition to original, with a minor amount of Eurobor slurry. A renewed control with wood sampling is made 3-5 years after addition. The Eurobor technique was developed during the 1990s and commercial used since 2002. Until today there have just been a few additional minor treatments made and the technique have been used in about 130 buildings before 2015. The technique has been continuously developed on increased experience, which suggests that additional treatments will be exceptional in future.

Timing

Period of time for treatment of an entire foundation depend on size and properties. After drilling of injection holes it usually takes 2-6 weeks to perform the injection of a building (included all measurements and controls). Prior injection measurements and documentation are made for each hole. This requires some hours. Each batch of 20 dm3 slurry requires 5 – 10 minutes for mixing and injection. The warm (50-70°C) water solution with Jerbor is mixed in batches of 163 dm3 and the time required for filling, mixing and dissolve Jerbor is about 20 minutes. During most of this period no injection is made why the time often is used for additional measurements and controls of spread.

Geographical/geological variations

Treatment is only made in foundations located in soft clay of marine origin. Areas that have been below saline sea after the last ice age can be studied at geological maps (Maxe, L. et al. 2007; Theorin, M. 2011c).

Risk assessment is always required

A risk assessment is always required before a treatment in order to determine if environments of concern (such as water supply areas etc.) are located nearby (Bandh, K-M. 2022).

3.4.1.5 Analytical methods for monitoring

Compartments identified as relevant for monitoring

Jerbor is restricted for use for in-situ application of a sodium bentonite slurry of borates into deep wooden foundations in dense clay soil. The foundations are always situated in a trench with very restricted groundwater circulation and low risk of emission of the active substances, as boric acid, in outflowing groundwater. Monitoring is made by sampling of groundwater in groundwater tubes. Injection holes within the treatment zone is sampled and measured regarding spread of Jerbor slurry for verification that the complete treatment zone is covered with the active substance. Foundation wood is sampled prior treatment for determination of material quality regarding load bearing capacity, level of deterioration and availability of the foundation wood for curative treatment with Jerbor.

Monitoring methods and time scale

Groundwater tubes are sampled prior, during and post (30 days) use of Jerbor in a deep wooden foundation. Sampling of foundation wood is made prior and post treatment (5 years) with Jerbor.

Monitoring in the environment

The active substances in Jerbor are not classified as dangerous to the environment and do not meet the criteria for ecotoxicologically relevant components (see section 2.6.11 in the BPR Guidance). Use conditions for Jerbor is considered niche with a specific labled use and use description. Boric acid and the corresponding borate anion are the identified chemical species that can be monitored by sampling and chemical analysis. The only relevant identified pathway of monitoring is groundwater. Monitoring is made by installation of groundwater tubes in the adjacent area of the treatment zone. Samples of groundwater are collected in a time series before, during and after application is made with the in-situ method for Jerbor. Analysis of the water samples are made qualitative and quantitative of total boric acid equivalent as mg/dm3 by the accredited (Swedac Accreditation no 1790 for testing ISO/IEC 17025) chemical analysis laboratory AK Lab AB located in Borås, Sweden. The method is based on Ion chromatography with conductivity detection (column IonPac ICE-AS1, no 031181-07) with a limit of detection for boric acid <1,0 mg/dm3 (1 ppm). Also, determination of temperature (method SS-EN ISO 10523:2012) and level of pH (SS-EN ISO 10523:2012 @ 25°C) is made. Analysis accuracy have been evaluated by the method “Evaluation of measurement data - Guide to the expression of uncertainty in measurement", JCGM 100:2008 Corrected version 2010” with a factor of cover of 2 and level of confidence of 95 % (AK Lab AB, 2019).

Monitoring during application

During application of the Jerbor mixture monitoring of the treatment zone spread of Jerbor is made by in-situ measuring of electrical conductivity. When the borates spread out in the treatment zone the content of ions, from borates and sodium, increase dramatically. Injection is made through bore holes into the foundation every 1-1.2 meters. Measurements are made in adjacent bore holes in order to detect and confirm spread of the active substances and as complete spread as possible throughout the hole treatment zone. Documentation is made of every injection hole with data on electrical conductivity, volume of Jerbor solution applied and when possible if ocular identification of the spread has been observed. The application monitoring procedure is well described in the manual of Eurobor (ver 2.0, Bandh 2022).

Monitoring in of wood in treatment zone, post-application

After 5-10 years post application with Jerbor samples of wood is taken from the foundation wood by coring. Depending on the conditions for each building foundation a number of samples (approximately n = 3-5) are collected for determination of the content of moisture (% H2O w/w) and boric acid (mg boric acid/dm3 wood). With an indicative colorimetric method Curcumin staining method, the distribution of borates in the wood is also determined.

Quantitative chemical analysis of borates is made by the accredited (Swedac Accreditation no 1790 for testing ISO/IEC 17025) chemical analysis laboratory Analys- & Konsultlaboratoriet AB located in Borås, Sweden. The method used for preparation of the wooden samples is CDS Pyroprobe and analysis of boric acid is based on Ion chromatography with conductivity detection (column IonPac ICE-AS1, no 031181-07). Analysis accuracy have been evaluated by the method “Evaluation of measurement data - Guide to the expression of uncertainty in measurement", JCGM 100:2008 Corrected version 2010” with a factor of cover of 2 and level of confidence of 95 % (AK Lab AB, 2019).

Highest shoreline in an area formed during a marine period after last glacial period.

The limit value according to Directive 98/83/EC